3GPP TSG-WG SA2 Meeting #171	S2-2508758

Wuhan, CN, 13th Oct – 17th Oct, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	KI#1: Consideration on Accuracy evaluation of estimation

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract:

# 1. Introduction/Discussion

In last meeting we discussed on accuracy and we had som disagreemtn on how to consider thje accuracy of proposed solutions. This document intend to provide a formal mathematical procedures which are describe din detailed in the annex. The results and proposals are resumed in the following:

Conclusion #1: the accuray of EC at different granularity calculated for Rel-19 , for example for the UE granularity is determined by equation (A) and (B). For other granulalrity the accuracy can be determined mathematically in equivalent manner

δEC,accuracy=2gNBi∈gNBUETECgNb,i×a100×DVx,gNBiDVgNBi2+UPFi∈UPFUETECUPFi×b100×DVx,UPFiDVUPFi2=   1100   2a2 ×gNBi∈gNBUETECgNb,i×DVx,gNBiDVgNBi2+b2 ×UPFi∈UPFUETECUPFi×DVx,UPFiDVUPFi2	(A)

δEC, relative accuracy=δEC,accuracyECx100%	(B)

Conclusion #2: The relative accuracy of the composite parameter ECx is generally better (lower percentage) than the relative accuracy of its individual components (a% or b%),

.Conclusion #3: The accuracy of EC at gNb and UPF can be determined using the formula defined in TS 28.554 and it can be an input to SA2 if it is required to associated an uncertainity values to the exposed estimation by SA2.

Conclusion #4: for Rel-19 formula the EC for a UE with constant data volume change due to the change of the total DV at the node, therefore the overal dat atraffic influences the estimated EC for a UE. The same behaviour occurs also for the other granularity.

Conclusion #5: The Sol#10 solution has the following charateristics:

- The EC per UE only depend by the DV consumed by the UE and not by the overall DV in the node,

- The accuracy of the formula depends by 2 factors:

  - EC per node measurement provided by OAM which has an uncertainity of ~5% or 3% depending by the load per TS 28.554.

  - the formula uses the differences (EC_Ta - EC_Tb) to determine the slope, therefore the overal accuracy of the result depends by the selection of Ta and Tb for the determination of the slope.

  - The best accuracy is obteined of the Ta and Tb are the farest

  - The accuracy for UE granulalrity is given by the following formulas. For other granulalrity the accuracy can be determined mathematically in equivalent manner

- The absolute accurcay of equation 4) is given by

- σEC,Ta=2gNBi∈gNBUETaσT,gNb,i2+UPFi∈UPFUETσT,UPF,j2	(D)

- where

- σT,gNb,i=a100×2DVgNB,UE,Ta,iEgNB,Ta,i2+EgNB,Tb,i2 DVgNB,Ta,i-DVgNB,Tb,i	(E)

- σT,UPF,j=a100×2DVUPF,UE,Ta,jEUPF,Ta,j2+EUPF,Tb,j2 DVUPF,Ta,j-DVUPF,Tb,j	(F)

- δEC,RelativeAccuracy%=σEC,ECUE,Ta*100	(G)

Conclusion #6: The relative accuracy of the composite parameter EC at slice is generally better (lower percentage) than the relative accuracy of its individual components (a% or b%) since it is a weighted average of many individual measurements. 
The accuracy for UE granulalrity is given by the following formulas.

δEC,accuracy=2gNBisliceTi2+UPFi∈sliceSi2+AMF∈sliceVj2+SMF∈sliceZl2

(I)

δEC, relative accuracy=δEC,accuracyECx100%	(H)

Where

-	Ti=ECgNb,i∙a100∙DVSlice ,gNBiDVgNBi	(J)

-	Si=ECUPF,Estimatedj∙b100∙DVSlice,,UPFjDVUPFj	(K)

-	Vj=ECAMF,Estimated∙b100∙Mean Number Registered UESlice  ,AMFMean Number Registered UE,AMF,	(L)

-	Zl=ECSMF,Estimated∙∙b100∙Mean Number PDU Sessionslice , SMFMean Number PDU Session UE,AMF,	(M)

# * * * * First change * * * *all new text

### 7.1.1	Agreed Principles for KI#1 System Architecture to Support Sensing

The agreed principle for the KI#1 are the following::

-	No new formula and mechanism for evaluation of EC at different granulalrity are considered in normative phase:

-	The EC at slice level provided by OAM is considered as additional input for the KI#2 and KI#3.

-	Renewable energy aspect is taken into account for collecting, calculating and exposing renewable energy information by EIF, as proposed by equation in Solution#8 and exposure in Solution #9 described in clause 6.8 and clause 6.9, respectively.

NOTE 2:	The renewable energy information evaluation at different granularity has dependency on Renawable enrgy at node level provided by OAM to EIF, so whether to be included in normative stage needs aligment with SA5.

-	The accuracy of the Energy Consumption at node level provided by OAM is defined by TS 28.554 as follow:

a) for Phycal node the accuracy is

-	 ±3 % from 25 % to 100 % of maximum load of the equipment (load range 1)

-	 ±5 % between 5 % and 25 % of maximum load of equipment (load range 2)

b)	For Virtual Network Function the EC is an estimation proportional to the average consumed resource provided by ETSI MANO of the EC of the hardware hosting the NF as defined by equations in clause X.Y ogf TS 28.554 which can not be lower than the accuracy of the measurement EC for physical node.

# * * * * End of changes * * * *

# Annex A- Accuracy estimation for the proposed methodology

## A.1 Introduction

The scop eof this annex is to introduced detailed mathematical definitioin of accuracy.

In order to evaluate the accuracy of the estimation formula the following principles are defined:

-	 the relative accuracy of EC at gNb is qual to ±a % of the EC measured at gNB. We assume that the accuracy is the same for the EC of all gNB.

-	 the relative accuracy of EC at UPF is qual to ±b % of the EC measured at gNB. We assume that the accuracy is the same for the EC of all gNB.  The absolute uncertain is σgNB,i=ECgNB,i×a100 , σUPF,i=ECUPF,i×b100

- 	the errors are of different EC values are uncorrelated

## A.2 Accuracy of R19 EC estimation at smaller granularity

If we assume that:

-	The DV at granularity x measured at gNB and the DV measured at gNB are precise, i.e. deterministic (no error)

-	The DV at granularity x measured at UPF and the DV measured at UPF are precise, i.e. deterministic (no error)

The R19 estimation at UE, PDU session and SDF granularity are defined in generic way as follow:

Ex=gNBi∈gNBUETEx, gNBi+ UPFi∈UPFUETEx, UPFi=gNBi∈gNBUETECgNb,iDVx,gNBiDVgNBi+UPFi∈UPFUETECUPFiDVx,UPFiDVUPFi  (1)

Where Ex represent the EC per granularity x, (e.g. per UE), DVx, gNbi and DVx, UPFi the Data Volume for the granularity x (e.g. per UE) at gNBi   and UPFi respectively.

Now let consider the above assumptions, the relative accuracy of EC is given by equation (2) and the accurcay in percentage is given by equation (3)

δEC,accuracy=2gNBi∈gNBUETECgNb,i×a100×DVx,gNBiDVgNBi2+UPFi∈UPFUETECUPFi×b100×DVx,UPFiDVUPFi2=   1100   2a2 ×gNBi∈gNBUETECgNb,i×DVx,gNBiDVgNBi2+b2 ×UPFi∈UPFUETECUPFi×DVx,UPFiDVUPFi2	(2)

δEC, relative accuracy=δEC,accuracyECx100%	(3)

The relative accuracy of the composite parameter ECx is generally better (lower percentage) than the relative accuracy of its individual components (a% or b%), since

- Averaging Effect: ECx is a weighted average of many individual measurements (ECgnb,i and ECupf,j). When independent measurements are summed, random errors tend to cancel each other out. A measurement that is too high is likely to be balanced by another that is too low.

- Root Sum of Squares (RSS): The formula for accurcay uses the RSS of the individual term uncertainties. This is fundamentally different from simply averaging the percentages. The RSS method grows more slowly than a linear sum. For example, if you have N identical terms, each with a relative uncertainty of a%, the relative uncertainty of their simple sum would be a% / sqrt(N). While ECx is a weighted sum, the principle is similar: combining many terms reduces the overall relative error.

- Dominant Terms: The formula shows that terms with larger weighted values DVx,gNBiDVgNBi or DVx,UPFiDVUPFi contribute more to the final uncertainty. However, even the contribution from a large term is diluted when summed with other terms to form the total ECx in the denominator.

Conclusion #1: the accuray of EC at different granularity calculated for Rel-19 , for example for the UE granularity is determined by equation (2) and (3). For other granulalrity the accuracy can be determined mathematically in equivalent manner.

Conclusion #2: The relative accuracy of the composite parameter ECx is generally better (lower percentage) than the relative accuracy of its individual components (a% or b%),

The accuracy of EC measured and gNB needs to be further determined considering where a node is composed by one or more physical hardware component where the Energy consumption is measured separatly or is meausure for the single component. If the UPF is implented as a vitual function, the EC is an estimation based on the reasourced used. Therefore the accurcay a% for gNB may be different from the baseline of 3% or 5% depending by the load, and for the UPF is defintively different since it depend also by the accuracy of the reasource measured by MANO and the fimrul aused to perfomr the estimation. The determination of  the value a% and b% can be mathematically determined using the festimation methods defined by SA5 in TS 28.554 , but for the considerations on SA2 proposed formula we can consider a value a% and b% which is provided or determined. The mathematical approach is not impacted.

Conclusion #3: The accuracy of EC at gNb and UPF can be determined using the formula defined in TS 28.554 and it can be an input to SA2 if it is required to associated an uncertainity values to the exposed estimation by SA2.

### A.2.1 Example of accuracy of R19 estimation at smaller granularity

Let consider the example with the following characteristics

- UE data volume:

-  DVUE,gNb,Ta= DVUE,gNb,Tb =200 Mbit

- gNB values

-	DVgNB,Ta=2000 Mbit, ECgnb,Ta=1150 kJ

-	DVgNB,Tb=3600 Mbit, ECgnb,Tb=1830 kJ

-  gNb Maximum capacity 4 Gb

-  gNB relative accuracy ±a % = ±5 %

- UPF values

-  DVupf,Ta=3500 Mbit, ECupf,Ta=1593.75 kJ

-  DVupf,Tb=5000 Mbit, ECupf,Tb=2062.5 kJ

-  UPF relative accuracy ±c % = ±5 % of EC at UPF

Therefore applying the equations (2) and (3) the EC and the accuracy at UE granularity at at Ta and Tb are when the UE at Ta and Tb has the same data volume consumption. The EC of the node increase due to the increase of traffic from other users.

-at Ta EC= 206,07 kJ absolute accuracy 6.49 kJ, relative accuracy= 3.15%

-at Tb EC= 184,16 kJ absolute accuracy 5.11 kJ, relative accuracy= 2.77%

Conclusion #4:  For the rel-19 formulas the EC for a UE with constant data volume change due to the change of the total DV at the node, therefore the overal dat atraffic influences the estimated EC for a UE. The same behaviour occurs also for the other granularity.

## A.2.2 Accuracy of solution #10 estimation at smaller granularity

The solution is based on the formula

ECUE,Ta=gNBi∈gNBUETaEUE, gNB,Tai+ UPFi∈UPFUETaEUE, UPFi,Ta	(4)

Where for exmaple for UE granularity

EUPF, UE, Ta=EUPF,Ta-EUPF,TbDVUPF,Ta-DVUPF,TbDVUPF,UE,Ta=EUPF,idle+EUPF,Ta,dynamic-EUPF,idle-EUPF,Tb,dynamicDVUPF,Ta-DVUPF,TbDVUPF,UE,Ta=	(5)

EUPF, UE, Ta=EUPF,Ta,dynamic-EUPF,Tb,dynamicDVUPF,Ta-DVUPF,TbDVUPF,UE,Ta

EgNB,UE,Ta=EgNB,Ta-EgNB,TbDVgNB,Ta-DVgNB,TbDVgNB,UE,Ta	(6)

The absolute accurcay of equation 4) is given by

σEC,Ta=2gNBi∈gNBUETaσT,gNb,i2+UPFi∈UPFUETσT,UPF,j2	(7)

where

σT,gNb,i=a100×2DVgNB,UE,Ta,iEgNB,Ta,i2+EgNB,Tb,i2 DVgNB,Ta,i-DVgNB,Tb,i	(8)

σT,UPF,j=a100×2DVUPF,UE,Ta,jEUPF,Ta,j2+EUPF,Tb,j2 DVUPF,Ta,j-DVUPF,Tb,j	(9)

δEC,RelativeAccuracy%=σEC,ECUE,Ta*100	(10)

### A.2.1 Example of accuracy of solution #10 estimation at smaller granularity

Let consider the following numerical example

- UE data volume:

-  DVUE,gNb,Ta= DVUE,gNb,Tb =200 Mbit

For the gNB:

- ECgnb,Ta=1150 kJ

- ECgnb,Tb=1830 kJ

- DVgnb,Ta=2000 Mbit

- DVgnb,Tb=3600 Mbit

- The ECgNB at DV=0 Mbit is 300 kJ, the EC at DV=4000 Mbit is 2000 kJ

For the UPF:

- ECupf,Ta=1593.75 kJ

- ECupf,Tb=2062.5 kJ

- DVupf,Ta=3500 Mbit

- DVupf,Tb=5000 Mbit

- The ECUPF at DV=0 Mbit is 500 kJ, the EC at DV=8000 Mbit is 3000 kJ

Let’s assume as example that that the relative accuracy for measument of EC at gNB and UPF is a%=b%=5%

Therefore the results based on equations (4), (7) and (10) are,

ECUE,Ta=EgNB,Ta-EgNB,TbDVgNB,Ta-DVgNB,TbDVgNB,UE,Ta+EUPF,Ta-EUPF,TbDVUPF,Ta-DVUPF,TbDVUPF,UE,Ta = 85 kJ+62.5 kJ=147.5 kJ 	(11)

With absolute accuracy at Ta

σEC,Ta=3.31 kJ				(12)

The relative accuracy at Ta is

δEC,RelativeAccuracy%=2.24%			(13)

The equations (4), (6) and (7) has the characteristic that is the DV per UE in Ta and Tb does not change and it is evaluated at Ta or at Tb, the results are the same. This means that if the DV for a UE doed not change between 2 measurement interval, while the DV and EV per node change, the EC per UE remains the same.

Now consider only the UPF for sake of simplicity The formula in solution #10  uses the evaluation based on determing the slope, so in the example with 5% uncertainity the EC range at Ta and Tb are:

- Ecupf,Ta= 1593 +/- 5%=1593 +/- 79.69 kJ		(15)

- Ecupf,Tb= 2062 +/- 5%=2062 +/- 103.125 kJ	(16)

In this case the maximum slope (steepest) is obtained using the lowest EC at Ta and highest EC at Tb corresponding to  0.4344 kJ/Mbit, the minimum slope (shallowest) is obtained using the highest EC at _Ta and lowest EC at Tb corresponding to 0.1906 kJ/Mbit

If the positions of the points Ta and Tb change (while keeping their relative uncertainties fixed at 5%), the minimum and maximum slopes will change, which directly affects the accuracy of EC(x) evaluated at DV=200 which depend by the minimum and maximum slope. The minum and maximum slopes are not arbitrary; they are mathematically derived from the extreme possible values of EC at Ta and EC at Tb:

Max Slope: (EC_Tb + ΔEC_Tb) - (EC_Ta - ΔEC_Ta) / (DV_Tb - DV_Ta)		(17)

Min Slope: (EC_Tb - ΔEC_Tb) - (EC_Ta + ΔEC_Ta) / (DV_Tb - DV_Ta)		(18)

Where ΔEC_Ta and ΔEC_Tb are the absolute uncertainties (e.g., 5% of their respective values).

The key variable here is the numerator: (EC_Tb ± ΔEC_Tb) - (EC_Ta ± ΔEC_Ta).

If you move Ta and Tb such that the difference (EC_Tb - EC_Ta) becomes smaller, the same absolute uncertainties ΔEC_Ta and ΔEC_Tb will represent a larger relative error in that difference.

Conversely, if the difference (EC_Tb - EC_Ta) becomes larger, the same absolute uncertainties will represent a smaller relative error.

The figure 1 shows the change of slope when different Ta and Tb are selected. The figure exagered the behaviour for representation. It also shows that the accuracy of measured of EC is relative, i.e. in percentage, therefore the absolute error increases with the increasing of the EC value.

Now let consider the numerical example, but the Ta for both gNB and UPF corresponds to the load with only 1 UE with DV of 200 Mbits, then the EC for the gNB is 385 kJ and for the UPF is EC 562.5 and for Tb we consider the EC at maximum load. The EC per UE is 147.5, but the relative and absoluty accuracy are 1.25 kJ and 0.85%

Figure 1: representation different slope with different Ta and Tb

Now if the numerical example if the Ta is selected to corresponds to the EC for load=0% and Tb fo rteh load of 100%, the EC per UE results EC = 147,5 kj as in equation (11) while the accuracy becomes

Conclusion #5: The Sol#10 solution has the following charateristics:

- The EC per UE only depend by the DV consumed by the UE and not by the overall DV in the node.

- The accuracy of the formula depends by 2 factors:

  - EC per node measurement provided by OAM which has an uncertainity of ~5% or 3% depending by the load per TS 28.554.

  - the formula uses the differences (EC_Ta - EC_Tb) to determine the slope, therefore the overal accuracy of the result depends by the selection of Ta and Tb for the determination of the slope.

  - The best accuracy is obteined of the Ta and Tb are the farest

  - The accuracy for UE granulalrity is given by the formulas (7), (8), (9) and (10). For other granulalrity the accuracy can be determined mathematically in equivalent manner

Figure 2: maximum and minimujm slope area of the provided example

## A.3 Accuracy slice based solution #2

The energy consumption for slice is defined in TS 28.554 as

(17)

Where TS 28.554 defines that:

-	For all gNBs in the network slice, clause 5.1.1.19.3 (PNF Energy consumption) of TS 28.552 [6] applies. This measurement is obtained according to the method defined in ETSI ES 202 336-12 [10] – clauses 4.4.3.1, 4.4.3.4, Annex A;

-	In case a 5GC NF is composed of Virtualized Network Functions (VNF) and/or Physical Network Functions (PNF), clause 6.7.3.1 of this document defines the NF Energy Consumption (EC);

-	In case a NF is dedicated to a network slice, the energy consumption of the NF is entirely attributable to the network slice;

Furthermore in case a NF is shared between multiple network slices, the participation of the NF to the energy consumption of the network slice has to be estimated, as it can't be measured as follow:

1)	In case of a gNB shared between multiple network slices, the energy consumption attributable to each network slice is estimated as a proportion of the total gNB energy consumption, where the proportion is calculated as the data volume of the network slice relatively to the total data volume carried by the gNB,

2)	In case of a UPF shared between multiple slices, the energy consumption attributable to each network slice is estimated as a proportion of the total estimated UPF energy consumption, where the proportion is calculated as the data volume of the network slice relatively to the overall data volume of the UPF during the same time period.

3) In case of a AMF shared between multiple network slices, the energy consumption attributable to each network slice is estimated as a proportion of the total estimated AMF energy consumption, where the proportion is calculated as the mean number of registered subscribers of the network slice relatively to the overall mean number of registered subscribers of the AMF during the same time period (see TS 28.552 [6] clause 5.2.1.1 for the definition of the mean number of registered subscribers),

4)	In case of a SMF shared between multiple network slices, the energy consumption attributable to each network slice is estimated as a proportion of the total estimated SMF energy consumption, where the proportion is calculated as the mean number of PDU sessions of the network slice relatively to the overall mean number of PDU sessions of the SMF during the same time period (see TS 28.552 [6] clause 5.3.1.1 for the definition of the mean number of PDU sessions),

The above definitions can be transformed in the following equation

ECns, slice =gNBECgNb, MeasuredDVSlice , gNb, DVgNb +  UPFECUPF,EstimatedDVSlice , UPFDVUPF+AMFECAMF,EstimatedMean Number Registered UESlice  ,AMFMean Number Registered UE,AMF,+SMFECSMF,EstimatedMean Number PDU Sessionslice , SMFMean Number PDU Session UE,SMF,

(18)

It shall be noted that SA5 into the slice includes also the AMF and the SMF. When a node is dedicated to only a slice, for example a gNB the DVslice,gNB is equal to the DVgNb , so the ration becomes equal to 1.

It is important to be noted that the terms ECgNB, Meausured corresponds to the ECPNF in solution Y for which the accuracy is defined in clause 6.y.1.1, while for UPF, AMF and SMF the accuracy is defined in clause 6.y.1.2.

Now if we define that:

-	ECgNb,i accuracy of ±a %  ( with a=5 load range 5%-25% , a=3 for load range 25%-100% )

-	assume that of EC for NF has same accuracy ECUPF,j ,ECSMF,j , ECAMF,j  accuracy of ±c %

Then if we assume that

-	The DV at slice granularity measured at gNB and UPF are precise, i.e. deterministic (no error)

-	The Mean Number Registered UESlice ,AMF and Mean Number PDU SessionSlice ,SMF  measured at slice are precise, i.e. deterministic (no error)

The relative standard deviation for the evaluation of the EC for the slice is given by the following equation

δEC,accuracy=2gNBisliceTi2+UPFi∈sliceSi2+AMF∈sliceVj2+SMF∈sliceZl2

(19)

δEC, relative accuracy=δEC,accuracyECx100%	(20)

Where

-	Ti=ECgNb,i∙a100∙DVSlice ,gNBiDVgNBi	(21)

-	Si=ECUPF,Estimatedj∙b100∙DVSlice,,UPFjDVUPFj	(22)

-	Vj=ECAMF,Estimated∙b100∙Mean Number Registered UESlice  ,AMFMean Number Registered UE,AMF,	(23)

-	Zl=ECSMF,Estimated∙∙b100∙Mean Number PDU Sessionslice , SMFMean Number PDU Session UE,AMF,	(24)

The ECUPF,Estimatedj, ECAMF,Estimated and ECSMF,Estimated are estimated as Virtual Network function, so the respective EC is estimated as defined in clause 6.y.1.2 based on estimated percentage of EC of the physical node hosting the Virtual Function .

The accurcay has the similar to those defined for other granulalrity. However it shall be noted that The relative accuracy of the composite parameter ECx is generally better (lower percentage) than the relative accuracy of its individual components (a% or b%), since

- Averaging Effect: ECx is a weighted average of many individual measurements (ECgnb,i and ECupf,j). When independent measurements are summed, random errors tend to cancel each other out. A measurement that is too high is likely to be balanced by another that is too low.

- Root Sum of Squares (RSS): The formula for accurcay uses the RSS of the individual term uncertainties. This is fundamentally different from simply averaging the percentages. The RSS method grows more slowly than a linear sum. For example, if you have N identical terms, each with a relative uncertainty of a%, the relative uncertainty of their simple sum would be a% / sqrt(N). While ECx is a weighted sum, the principle is similar: combining many terms reduces the overall relative error.

- Dominant Terms: The formula shows that terms with larger weighted values DVx,gNBiDVgNBi or DVx,UPFiDVUPFi contribute more to the final uncertainty. However, even the contribution from a large term is diluted when summed with other terms to form the total ECx in the denominator.

Since the number of nodes in slices is much higher that the node for a single UE or PDU session, the accuracy at sllice is in percentage lower than those of single node.

Conclusion #6: The relative accuracy of the composite parameter EC at slice is generally better (lower percentage) than the relative accuracy of its individual components (a% or b%) since it is a weighted average of many individual measurements.