SA WG2 Meeting #171	S2-2508897

13 – 17 October, 2025, Wuhan, China

Title:	Update of solution #23: Selective data collection

Source:	Mitsubishi Electric

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This contribution proposes an update of solution #23 to introduce selective data collection.

# 1.		Introduction

This contribution proposes an update of solution #23 to introduce selective data collection.

# 2.	Discussion

Collecting and transferring through the network data related to sensing will likely generate an important payload. Exploiting redundancy and selecting in a smart way data in multi-sensor systems can significantly reduce the communication system load and allow more efficient operations. Redundancy means that multiple sensors provide overlapping information; smart data selection means that less relevant data are avoided to be transmitted.

This smart data selection may consist of, for example, selecting a specific angle of view among several redundant ones, or a most representative angle of interest among complementary ones, or a sufficient subset of measurements in the context of cooperative data fusion. Only relevant data may then be effectively transferred through the network, thus leading to more efficient sensing data collection and reporting procedures.

In a first approach, the smart data selection can be based on static characteristics of the sensor. Nevertheless, this approach does not allow to take into account occlusion phenomena. Occlusion refers to area portions that are masked by objects in the scene that may be temporarily visible only from specific angles. For example, a first sensor entity may provide a general view of a scene, but some parts can be masked by an object. A second sensing entity may be selected to complement the scene with the portion that is masked in the first view. Since both sensing targets and masking objects are susceptible to move, occlusion is dynamic by nature.

Hence, we propose to introduce the principle of dynamic data excerpt and smart data selection. This implies that interface for data collection over the Sensing Entity (SE) / Sensing Function (SF) includes the following features:

1) 	the SE may provide a data excerpt. It can be for example data corresponding to its complete view but with low resolution,

2)	the possibility to collect only a portion of the data available at the SE, typically with full resolution.

Operations 1) and 2) are typically executed in parallel, with different periodicities: SF may request operation 1) at a low pace, thus being able to decide which is the relevant data portion with full resolution in 2). In an environment where moving objects can dynamically occlude some sensors or some angles of view, collecting data excerpts allows to minimize the amount of data collected for the task.

Operation 1) can also be trigger-based: The SE could be requested to provide a data excerpt only when certain conditions are met, for example an object detection.

As a consequence, a sensing request from a consumer may require several requests/responses between SE and SF. Indeed, the SF shall do a mapping between request ids coming from the consumer, and request ids that are used between SF and SE.

It is then proposed the following:

a) Clarify that request IDs are different over the (sensing consumer, SE) interface and over the (SF, SE) interface.

b) (SF, SE) interface supports the collection of a selection of data

c) (SF, SE) interface supports the collection of a dynamic data excerpt

# 3.	Text Proposal

It is proposed to agree the following text to be part of TR 23.700-14.

FIRST CHANGE

## Solution #23: Distributed Sensing Data Processing

This solution addresses KIs #4 and #5.

### 6.23.0	High-level solution principles

The following high-level solution principles apply to this solution:

-	A solution supporting multiple sensing data consumer entities (e.g. Sensing Processing Function).

-	The solution supports multiple sensing processing and consumers of the 5GS by leveraging 5GS functions and services (i.e. processing, consumption of sensing data is done in the 5GS).

-	The solution introduces dynamic data excerpt and selective data collection

### 6.23.1	Description

It is anticipated that while sensing is ongoing, one or multiple sensing entities will generate sensing data that requires processing to obtain a final sensing result. This processing may require multiple and independent processing steps (e.g. a user, sensing subscriber, has to include proprietary fusion algorithms or external AIML models for sensing processing). To ensure that processing is fast, reliable, flexible and scalable, it is proposed that multiple Sensing Processing Functions (SePF) are supported. This might require supporting the chaining of multiple SePFs to get a final Sensing Result (or multiple Sensing Results). Furthermore, this proposal allows the selection of a single SePF or multiple SePFs. In case multiple SePFs are used, this solution provides a procedure which allows to deploy a Sensing Processing chain, which is scalable and flexible.

Hence, the motivation for this solution is computational flexibility for sensing processing. It is indeed anticipated that complex processing techniques may be employed on the sensing measurements, compounded by the fact that the measurements may be collected in multiple Sensing Entities. Splitting the processing by means of concatenating and applying different computational techniques on potentially different parts of the measurement datasets can bring benefits in the accuracy of the sensing results.

However, the proposed split of the processing need not be mandated for all scenarios and use cases and it can be optionally applied on the cases requiring processing of high complexity and modularity. The term processing is used as a general term which includes different kinds of compute and inference processes such as fusion, AIML and rule-based processing. It is also possible that a SePF may be specialized only in one type of computing algorithms, e.g. AI/ML. How the processing is implemented is out of scope of this solution.

The proposed procedures enable, for instance, an MNO to offer 5G services that meet customer requirements and provide flexibility for processing in the 5G network.

NOTE:	The split of sensing processing can also be left up to SePF implementation.

It is also anticipated that collecting and transferring through the network data related to sensing will likely generate an important payload. Exploiting redundancy and selecting in a smart way data in multi-sensor systems can significantly reduce the communication system load and allow more efficient operations. Redundancy means that multiple sensor entities provide overlapping information; smart data selection means that less relevant data are avoided to be transmitted. This smart data selection may consist of, for example, selecting a specific angle of view among several redundant ones, or a most representative angle of interest among complementary ones, or a sufficient subset of measurements in the context of cooperative data fusion. For example, a first sensor entity may provide a general view of a scene, but some parts can be masked by an object (occlusion). A second sensing entity may be selected to only complement the scene with the portion that is masked in the first view. Since both sensing targets and masking objects are susceptible to move, occlusion is dynamic by nature.

Hence, it is proposed that a sensing entity may have the possibility to provide a dynamic data excerpt, e.g. data corresponding to a complete view but at low resolution. In addition, the description of parameters in the sensing request from a sensing function to a sensing entity includes the possibility to request only a portion of the data a sensing function could provide.

#### 6.23.1.1	Definitions

The following definitions are applied in this solution:

Sensing Service Consumer (SSC): The entity that may consume the Sensing Result. The Sensing Service Consumer may also request the Sensing Result.

Sensing Service Request (SSR): The service operation used by the SSC to request sensing service.

### 6.23.2	Procedures

#### 6.23.2.1	Procedure for Sensing Data Processing in the Core Network

Figure 6.23.2.1-1: Procedure for Sensing Data Processing in the Core Network

The following is a description of the procedure for processing in the 5GC only:

0.	The procedures of registering network functions and services, evaluating the Sensing Service Request (SSR) by the SeMF and discovering network functions and services required to fulfill the SSR are performed here. The SSR itself is not shown in Figure 6.23.2.1-1. The SSR was triggered before by the Sensing Service Consumer (SSC).

1.	[SensingEntityConfigurationReq]: The SeMF sends a configuration request to a gNB. As an example, the message signature may include an identifier and description of sensing parameters. The identifier is required to identify a SSR during the whole procedure of a sensing session. The description of parameters may include more detailed information, e.g. parameters which are specific to the gNB which needs to be configured for sensing.

Editor's note:	Whether any intermediate NF is involved in the interactions between the gNB and other 5GC NF (e.g. SeMF, SePF, NEF) shown in Figure 6.23.2.1 1 is FFS.

Editor's note:	Interactions between the gNB and the 5GC requires coordination with RAN WGs.

2.	[SensingEntityConfigurationReq]: The gNB responds with a message indicating if the configuration was successful or not.

NOTE 1:	After this message the Sensing Entity may start the measurements.

3.	[SensingProcessingReq]: The SeMF sends a processing configuration request to the SePF. The request includes the SSR identifier and additional parameters which may include other SePF(s) involved in sensing processing.

Editor's note:	How the SeMF selects the SePF(s) involved in sensing processing and notifies the selection is FFS.

4.	[SensingDataReq, Optional]: The SePF sends a data request message to the gNB. This request includes a request identifier and an indication for a dynamic data excerpt. It may include other parameters, e.g. whether the data request expects a periodic response (e.g. every 30ms) or a one-off response is expected.

4a.	[SensingDataRsp, Optional]: The gNB responds with data corresponding to its dynamic data excerpt.

4b.	After receiving dynamic data excerpt data from the gNB, the SePF selects data of interest

5a.	[SensingDataReq]: The SePF sends a data request message to the gNB. This request includes a request identifier and may include other parameters, e.g. whether the data request expects a periodic response (e.g. every 30ms) or a one-off response is expected. Parameters may include data portions of interest as selected at step 4b.

5.	[SensingDataRsp]: The gNB responds with sensing data. Depending on the configuration this event can occur periodically or only once. After SePF #1 has received data from the gNB, it processes the data.

6.	[Optional] After processing the data, the SePF #1 needs to know before step 7 and step 8 are triggered which the next processing entity is. To figure that out, the following options are possible:

-	a static pre-configuration at SePF with a list of the next processing entities. In addition, which entity to choose from in this list might be depending e.g. on the sensing result.

-	the SePF may trigger a service discovery on the service bus to identify the next processing entity. Parameters to discover the next entity via a service discovery request are provided by the parameters in step 3.

-	the parameter-set in the request from step 3 may include a list of next possible entities. The SePF has to follow that list.

NOTE 2: 	As an example, the top of the list may be the next target processing entity to visit. After identifying the next entity, the SePF may remove the top from the list and with that the next entry in the list becomes the new top entry. Removing the entity form the top of the list modifies the parameter-set. This changed parameter-set has then to be provided by the actual SePF to the next entity as described in step 8.

Editor's note:	The details on SePF discovery/selection of other SePF in the network including the required parameters is FFS.

7.	[SensingProcessingReq, Optional]: SePF #1 sends a SensingProcessingReq message to the SePF #2 to notify that new data for processing will arrive. This step is triggered when not all required SePFs were notified by the SeMF in step 3.

8.	[SensingResult, Optional]: After processing the data, the result is sent via a sensing result message SePF #2. It is required to add all required parameters described in step 3, 4, 6 to this message as well to ensure that the SePF #2 knows from which entity to request new or additional data (if required), and to which entity to send the result (see optional step 7). This allows that multiple SePFs can be chained in a controlled way. The sensing result message might be an unsolicited message since SePF #2 was notified before by a SensingProcessingReq message that it has to expect data. SePF #2 confirms the result calculation and may provide it to SePF #1, depending on implementation.

9.	The sensing results are delivered to the SSC. Either SePF may provide them, depending on implementation.

Editor's note:	Details on result delivery are FFS.

10.a SePF #1 maps the request identifier(s) to the initial SSR

10.	[SensingProcessingRsp]: After the sensing result is delivered to the SSC, SePF#1 will send a response message back to the SeMF (or to the requester of the SensingProcessingReq message as in the optional step 7) to signal the success or failure of the request. The response includes a response code as well to indicate the success/failure of the operation.

NOTE 3:	Steps 4a and 4b may be executed in parallel with steps 5a to 8, with independent periodicities.

### 6.23.3	Impacts on services, entities and interfaces

SePF:

-	Supports the operation of multiple SePF instances.

SeMF:

-	Supports the management of multiple SePFs, within the 5GS.

End of Changes