3GPP TSG-WG SA2 Meeting #171	S2-2509015

Wuhan, CN, 13th Oct – 17th Oct, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	How to resolve the compatibility issue for PEI and emergency PDU session from RAN3

Document for:	Discussion

Agenda Item:	4.1

Work Item / Release:	NR_UE_pow_sav_enh-Core, NR_LPWUS-Core / Rel-17

Abstract: Propose to address the compatibility issues introduced by previously agreed SA2 CR on PEIPS.

# 1. Introduction

The LS (S2-2508189/R3-255906) from RAN3 points out that the CR (S2-2502428) on PEIPS adopted by SA2 will introduce compatibility issues, when different releases of UE/NG-RAN/AMF are deployed. In this adopted proposal, the restriction that PEIPS cannot be used when emergency PDU sessions ongoing was removed from Rel-19. This proposal discusses how to address the compatibility issues.

The compatibility issues and scenarios identified are provided for reference in the following figures:

Figure 1-1: Legacy UE (before Rel-19) with new network (Rel-19 or after)

Figure 1-2: New UE (Rel-19 or after) with legacy network (before Rel-19)

# 2. Discussion

To address the compatibility issue, one potential solution RAN3 has proposed is that SA2 reverts the previously agreed CR. However, the CR previously agreed was based on the technical consensus reached through numerous rounds of meeting discussions. This consensus confirmed two key points:

-	using PEIPS for paging when emergency PDU session ongoing will not introduce significant delay; and

-	that PSAP callback for emergency service is delivered as normal call.

Thus, the earlier conclusion should not be arbitrarily reversed. Furthermore, removing the restriction of emergency PDU session for PEI from Rel-17 onwards (rather than Rel-19) by SA2 also cannot fully resolve the compatibility issue for the UE already launched in the market.

The compatibility issues are very common in such scenarios, and they can be resolved using conventional and effective method-specifically through capability negotiation. The capability negotiation solution only requires defining two indicators, one for the UE reporting its capability, and the other for the network to report its capability. This approach is lightweight, as it only requires modification to the Rel-19 CN specification, with impacts limited to just the UE and AMF.

Below is the whole picture of the solution for the UE in idle state and RRC inactive.

Figure 2-1: UE is in IDLE

Figure 2-2: UE is in RRC Inactive

# 3. Conclusion and proposal(s)

The capability negotiation solution is proposed to solve the compatibility issue above, and CRs are provided for this.