3GPP TSG-WG SA2 Meeting #171	S2-2509040

Wuhan, CN, 13th Oct – 17th Oct, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon, China Mobile

Title:	KI#2: New Sol: Optimization of SIP timers for Voice over GEO over NB-IoT

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract: The contribution defines dedicated values for GEO access. Besides, the procedures for UEs and IMS elements to enable SIP timers base on RAT type is proposed.

# 1	Discussion

TS 24.229 clause 7.7 specifies a set of timers applied to SIP messages delivery. For example, the estimated RTT of a SIP message (indicated by T1) at the UE is set to 2s by default and this timer is used to initialise the timeout of an INVITE (Timer B) and non-invite transactions (Timer F).

Table 7.7.1: SIP timers

| SIP Timer | Value to be applied between IM CN subsystem elements | Value to be applied at the UE | Value to be applied at the P-CSCF toward a UE | Meaning |
| --- | --- | --- | --- | --- |
| T1 | 500ms default (see NOTE) | 2s default | 2s default | RTT estimate |
| T2 | 4s (see NOTE) | 16s | 16s | The maximum retransmit interval for non-INVITE requests and INVITE responses |
| T4 | 5s (see NOTE) | 17s | 17s | Maximum duration a message will remain in the network |
| Timer A | initially T1 | initially T1 | initially T1 | INVITE request retransmit interval, for UDP only |
| Timer B | 64*T1 | 64*T1 | 64*T1 | INVITE transaction timeout timer |
| Timer C | > 3min | > 3 min | > 3 min | proxy INVITE transaction timeout |
| Timer D | > 32s for UDP | >128s | >128s | Wait time for response retransmits |
|  | 0s for TCP/SCTP | 0s for TCP/SCTP | 0s for TCP/SCTP |  |
| Timer E | initially T1 | initially T1 | initially T1 | non-INVITE request retransmit interval, UDP only |
| Timer F | 64*T1 | 64*T1 | 64*T1 | non-INVITE transaction timeout timer |
| Timer G | initially T1 | initially T1 | initially T1 | INVITE response retransmit interval |
| Timer H | 64*T1 | 64*T1 | 64*T1 | Wait time for ACK receipt. |
| Timer I | T4 for UDP | T4 for UDP | T4 for UDP | Wait time for ACK retransmits |
|  | 0s for TCP/SCTP | 0s for TCP/SCTP | 0s for TCP/SCTP |  |
| Timer J | 64*T1 for UDP | 64*T1 for UDP | 64*T1 for UDP | Wait time for non-INVITE request retransmits |
|  | 0s for TCP/SCTP | 0s for TCP/SCTP | 0s for TCP/SCTP |  |
| Timer K | T4 for UDP | T4 for UDP | T4 for UDP | Wait time for response retransmits |
|  | 0s for TCP/SCTP | 0s for TCP/SCTP | 0s for TCP/SCTP |  |
| Timer L | 64*T1 | 64*T1 | 64*T1 | Wait time for accepted INVITE request retransmits |
| Timer M | 64*T1 | 64*T1 | 64*T1 | Wait time for retransmission of 2xx to INVITE or additional 2xx from other branches of a forked INVITE |
| Timer N | 64*T1 | 64*T1 | 64*T1 | Wait time for receipt of a NOTIFY request upon sending SUBSCRIBE |
| NOTE: As a network option, SIP T1 Timer's value can be extended, along with the necessary modifications of T2 and T4 Timers' values, to take into account the specificities of the supported services when the MRFC and the controlling AS are under the control of the same operator and the controlling AS knows, based on local configuration, that the MRFC implements a longer value of SIP T1 Timer. |  |  |  |  |

In typical terrestrial scenarios (e.g., LTE, NR), these default values are sufficient given the low latency and stable connectivity of terrestrial networks. However, considering NB-IoT accesses via GEO satellite, the RTT could be much longer than the default T1 due to the long propagation delay and low transmission rate.

Therefore, the SIP timer default values for a SIP transaction involving a GEO leg should be extended for two reasons:

1) The SIP transactions take longer time, so the SIP timer should be extended to avoid undesired session timeouts.

2) The SIP timer of retransmission interval (T1) should adapt to the RTT in GEO scenario to avoid frequent and unnecessary signalling retransmissions.

It is proposed that SIP timers are configured in consideration of the RAT type used by the UE. Ideally the least stringent SIP timers should be used during a session.

# 2	Proposal

It is proposed to include the following changes in TR 23.700-19 V1.0.0.

* * * First Change * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 |
| x |  | X |  |  |

* * * Next Change (all new)* * *

## 6.X	Solution #X: Optimization of SIP timers for Voice over GEO over NB-IoT

### 6.X.1	High level principles

This solution resolves Key Issue #2 IMS enhancement for GEO NB-IoT NTN access, the following principles are included in this solution:

-	The SIP timer for NB-IoT (GEO) access are extended to take into account the longer Round Trip Time.

-	Any UE that is connected via GEO satellite as well as the IMS elements serving that UE apply the set of SIP timers dedicated for GEO access.

-	If during an IMS voice call only one of the UEs is connected via GEO satellite, the UE not connected via GEO and the IMS elements serving that UE may also apply the set of SIP timers to the session-related SIP messages. The MO and MT UEs communicate the RAT type in use to each other. This information is carried to the peer network via a new header.

NOTE:	Session-related SIP messages refer to the SIP messages transmitted between UEs during a IMS session, such as INVITE, 183 Session Progress, UPDATE, etc.

### 6.X.2	Description

When a UE registering on IMS indicates in the P-Access-Network-Info (PANI) header field in the SIP Registration Request that the RAT type is 3GPP-NB-IoT(GEO), the UE and the IMS serving the UE shall configure optimised SIP timers for GEO access. A third-party registration procedure is also executed to inform the AS that the UE is accessing via GEO.

The use of optimised SIP timers minimises the risk of a session timer timeout which may result from the larger propagation delay and the possibility of messages being segmented when using 3GPP-NB-IoT(GEO) access.

It is suggested to modify the following SIP timers compared to the default values defined in TS 24.229 [16].

Table 6.X.2-1: SIP Timer Optimization Requirements

| Timer | Default Value | Definition | Suggested Value | Optimization Information |
| --- | --- | --- | --- | --- |
| T1 | 0.5s | Round-trip message delay | 12s | Based on the formula of TR 23.700-19 A.1.2. The time required to send a SIP message: 8 × 1.3 KB (maximum round-trip message size) / 1 kbps (satellite link rate) + 0.6 (RTT delay) s = 11 s, recommended to adjust to 12 s. |
| T2 | 4s | Maximum retransmission interval for non-INVITE request messages and INVITE response messages | 24s | It is recommended to set T2 ≥ 2 × SIP response transmission time (24 s), ensuring at least 1 retransmission. |
| T4 | 5s | Maximum lifetime of a SIP message in the network | 26s | The time to stop retransmissions must be greater than the T2 timer duration. |
| Timer A | T1 | Retransmission interval for INVITE messages when using UDP transport | 12s | Same as the T1 timer setting. |
| Timer B | 64T1 | INVITE transaction timeout duration | 96s | For the maximum total retransmission duration timer, the intervals are 12 s, 24 s, 48 s, 96 s, set to 96 s for a total of 4 retransmissions. Previously, 0.5 s, 1 s, 2 s, 4 s, 8 s, 16 s, 32 s allowed 7 retransmissions. |
| Timer D | >128s | Time to wait for retransmitted response messages after receiving a response | 96s | Same as the Timer B setting. |

If during an IMS voice call only one of the UEs is connected via GEO satellite, the UE not connected via GEO and the IMS elements serving that UE may also apply the set of SIP timers dedicated for GEO access to the session-related SIP messages. This will avoid risks of frequent re-transmissions of messages.

The dedicated SIP timers are only used for the session-related SIP messages while default timer values may be used for session-unrelated SIP messages.

The peer using GEO access informs the other peer when the RAT type in use is 3GPP-NB-IoT(GEO) via a new header.

NOTE:	A new header is required since the as per TS 24.229 [16] the IBCF removes the PANI header.

### 6.X.3	Procedures

#### 6.X.3.1	Overview

Two possible procedures for SIP timer configuration are shown in 6.X.3.2 and 6.3.X.3. For 6.X.3.2, the SIP timer values for the UE and IMS elements serving that UE are related to that UE’s RAT type. While for 6.X.3.3, the peer UE’s RAT type is also considered.

#### 6.X.3.2 Procedures for SIP timer adjustment based on UE’s RAT Type

Figure 6.X.3.2-1 provides an example procedure for the UE and IMS elements to adjust their SIP timers based on the UE’s RAT type.

UE-A

P-CSCF-A

AS-A

1. Registration Request

I/S-CSCF-A

4. 200 OK

3. Registration Request

2. 200 OK

Start SIP timers based on UE-A's RAT type

Figure 6.X.3.2-1: Session setup procedures and application of optimised SIP timers when UE is accessing via GEO.

1-2. The UE sends a SIP Registration Request to IMS network (P-CSCF, I-CSCF and S-CSCF), including a P-Access-Network-Info header field which includes the UE’s RAT type. The UE and the IMS shall start the SIP timers based on the UE’s RAT type. Then the IMS core responds with a 200 OK.

3-4. The S-CSCF initiates the third-party registration to AS (typically MMTel AS and SCC AS), to inform the AS of the UE’s RAT type. After receiving the Registration Request, the AS starts the SIP timers based on UE’s RAT type. Then the AS responds with a 200 OK.

#### 6.X.3.3	Procedures for the MO and MT UE Negotiating SIP Timers based on RAT Type

Figure 6.X.3.3-1 provides an example call setup procedure when the caller and/or called UE accesses through satellite. If either the UE-A or UE-B accesses via GEO satellite, the UEs and the IMS elements may both initiate the set of SIP timers dedicated for GEO access.

UE-A

P-CSCF-A

AS-A

UE-B

1. Registration Request

1. Registration Request

I/S-CSCF-A

4. 200 OK

3. Registration Request

2. 200 OK

AS-B

I/S-CSCF-B

P-CSCF-A

2. 200 OK

3. Registration Request

4. 200 OK

5. INVITE

6. 183 Session Progress

Start SIP timers based on UE-A's RAT type

Start SIP timers based on UE-B's RAT type

Start SIP timers based on UE-A and UE-B's RAT type

7. Remaining session setup procedures

5. INVITE

5. INVITE

5. INVITE

6. 183 Session Progress

6. 183 Session Progress

6. 183 Session Progress

6. 183 Session Progress

5. INVITE

Figure 6.X.3.3-1: procedures for the MO and MT UE negotiating SIP timers based on RAT type

1-2. The UE sends a SIP Registration Request to IMS network (P-CSCF, I-CSCF and S-CSCF), including a P-Access-Network-Info header field which includes the UE’s RAT type. The UE and the IMS core shall start the SIP timers based on the UE’s RAT type. Then the IMS core responds with a 200 OK.

3-4. The S-CSCF initiates the third-party registration to AS (typically MMTel AS and SCC AS), to inform the AS of the UE’s RAT type. After receiving the Registration Request, the AS starts the SIP timers based on UE’s RAT type. Then the AS responds with a 200 OK.

5.	The UE-A initiates the IMS voice call with a SIP INVITE message, including a PANI header field which consists of the UE-A’s RAT type. If Access-type field in PANI header is 3GPP-NB-IoT(GEO), then the IBCF shall carry the access type to the peer network via a new header.

6.	The UE-B response with a SIP 183 Session Progress message to UE-A, including a PANI header field which consists of the UE-B’s RAT type. If Access-type field in PANI header is 3GPP-NB-IoT(GEO), then the IBCF shall carry the access type to the peer network via a new header. If either the UE-A or UE-B accesses via GEO, the UEs and IMS elements may apply the set of timers dedicated for GEO access.

7.	UE-A and UE-B continue the remaining procedures for IMS call setup, based on the SIP timers determined in step 6.

NOTE 1:	The SIP timers unrelated to the session are not affected. For example, the SIP timers for Registration Request, OPTIONS, SUBSCRIBE remain unchanged.

NOTE 2: The value for the set of SIP timers dedicated for GEO access is FFS.

### 6.X.4	Impacts to Services, Entities and Interfaces

UE:

-	Support the set of SIP timers dedicated for GEO access.

-	If either the peer UE is accessing via GEO, it may apply the set of SIP timers dedicated for GEO access for session related SIP messages.

IMS:

-	Support the set of SIP timers dedicated for GEO access.

-	If the IMS of the peer UE is accessing via GEO, it may apply the set of SIP timers dedicated for GEO access for session related SIP messages.

-	Can send the RAT type used by the UE to the peer IMS (and UE) in the session

* * * End of Changes * * *