3GPP TSG-WG SA2 Meeting #171	S2-2509042

Wuhan, CN, 13th Oct – 17th Oct, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon, China Telecom, CSCN

Title:	[Sol#14 update] SIP procedure optimization for IMS voice call via NB-IoT GEO

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract: The contribution proposes to update solution #14 for KI#2.

# Introduction

This main update for solution#14 in this contribution includes:

1.	Change the E2E flow to MO/MT flow

2.	Update/Add the impact on terminating UE/terminating IMS

3.	Add Estimation of call setup time and delete related Editor’s notes

4.	Add description of CAT suppression on originating IMS and update/add the related impact on originating IMS

# Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-19.

# * * * * First change * * * *

## 6.14	Solution #14: Call Setup Procedure Optimization for Voice over GEO over NB-IoT

### 6.14.0	High-level solution Principles

This solution proposes to use as baseline the short call setup procedure without preconditions, but introduces safeguards to ensure the media starts to flow only when bearer is set up by the UE connecting to the network using 3GPP-NB-IoT(GEO). The solution also takes into account whether one of the parties does not support the proposed simplified call setup procedure.

### 6.14.1	Description

This solution addresses Key Issue #2: IMS enhancement for GEO NB-IoT NTN access.

1.	This solution aims to optimize call setup procedure for reducing call setup time in case one of the UEs is accessing network via NB-IoT GEO by proposing to use a simplified call set up time that uses as baseline the procedure in clause 5.7a of TS 23.228 [6] with difference that 180 Ringing carries SDP answer:

INVITE (SDP offer) -> 180 Ringing (SDP answer) -> PRACK -> 200 OK for PRACK-> 200 OK for INVITE -> ACK

2.	It is assumed that a UE capable of supporting IMS voice over GEO over NB-IoT GEO as IP-CAN (VoGoN) can support this simplified call setup procedure as default when connecting to the network via 3GPP-NB-IoT(GEO). This means for terminating UE is accessing network via 3GPP-NB-IoT(GEO) it should send 180 Ringing with SDP answer when it receives INVITE, and for originating UE is accessing network via 3GPP-NB-IoT(GEO) it sends PRACK as an acknowledgment of receiving SDP answer and resource reservation completion.

When the originating UE is accessing the network via GEO with NB-IoT RAT, the UE does not include Precondition in Supported header field in INVITE

The originating P-CSCF inserts originating UE's access network information and forwards the INVITE to the originating S-CSCF.

If originating S-CSCF determines originating UE is accessing the network via 3GPP-NB-IoT(GEO) based on the originating UE’s access network in the INVITE and originating UE has subscribed to the CAT service, the originating S-CSCF does not trigger CAT AS and forwards the INVITE to the terminating side.

If terminating S-CSCF determines originating UE is accessing the network via 3GPP-NB-IoT(GEO) based on the originating UE’s access network in the INVITE and terminating UE has subscribed to the CAT service, the terminating S-CSCF should not trigger CAT AS.

If terminating network (CAT AS or S-CSCF) determines terminating UE is accessing the network via 3GPP-NB-IoT(GEO) based on the terminating UE’s access network in the 180 Ringing and terminating UE has subscribed to the CAT service, the terminating network should not perform CAT AS related procedure.

If originating CAT AS determines terminating UE is accessing the network via 3GPP-NB-IoT (GEO) based on the terminating UE’s access network in the 180 Ringing and originating UE has subscribed to the CAT service, the originating CAT AS should not modify the SDP answer for CAT service.

3.	This solution examines two scenarios:

-	Case 1: Only originating UE (UE A) supports VoGoN and is accessing network via 3GPP-NB-IoT(GEO) while terminating UE(UE B) does not support VoGoN;

-	Case 2: Terminating UE (UE B) supports VoGoN and is accessing network via 3GPP-NB-IoT(GEO) regardless whether originating UE (UE A) supports VoGoN or not.

For case 2, the simplified call setup procedure can be performed E2E between two UEs because UE B is capable of sending 180 Ringing with SDP answer due to it supports VoGoN, while for case1, considering that UE B does not support VoGoN, this solution proposes that the originating P-CSCF acts as a B2B UA to perform simplified call setup procedure towards the UE A and normal call setup with the UE B.

Editor's note:	The case where Voice over GEO over NB-IoT is offered as subscription service is for further study.

- Estimation of call setup time:

For the calculation of the call setup time, the assumption of message size refers to annex A2 with the difference that in this solution the 180 Ringing message also includes the SDP answer.

- SIP INVITE: 2564(+52) Byte;

- 180 Ringing with SDP answer: 983+580(+52) Byte. (580 is an estimation of SDP answer)

So it is estimated total message size is to be 4231(2564+52+983+580+52) Bytes which requires 11.29~33.85s (data rate 1~3kbps) for the transmission before any message size compression mechanism is applied. The final call setup time depends on additional mechanism for message size reduction, e.g. SigComp and SDP pre-configuration.

### 6.14.2	Procedures

#### 6.14.2.1	Case 1: UE A Supports VoGoN and is Accessing the Network via 3GPP-NB-IoT(GEO), UE B does not support VoGoN

In this procedure the P-CSCF of UE A acts as B2B UA so that UE B, can use the normal call setup procedure while UE A which connects via 3GPP-NB-IoT(GEO) can use the simplified call setup procedure reducing the number of round trips over the satellite service link and enables more robust interworking with UE not supporting VoGoN.

Figure 6.14.2.1-1: Call procedure for Case1: UE-A supports VoGoN and is accessing network via 3GPP-NB-IoT(GEO) , UE B does not support VoGoN

1.	UE A sends a SIP INVITE request containing an SDP offer to P-CSCF A.

If UE A is accessing network via 3GPP-NB-IoT(GEO), it does not indicate precondition in Supported header.

2-3.	P-CSCF A forwards SIP INVITE request to the terminating network.

P-CSCF A interacts with EPC to retrieve UE A's access network information and inserts UE A's access network in the INVITE. P-CSCF A modifies the INVITE to introduce precondition supported.

NOTE:	If S-CSCF A determines UE A is accessing the network via 3GPP-NB-IoT (GEO) based on the UE A’s access network in the INVITE and UE A has subscribed to the CAT service, the S-CSCF A should not trigger CAT AS.

4.	UE B generates 183 Progress response with SDP answer due to UE B not supporting VoGoN, then sends it to S-CSCF A.

NOTE:	If S-CSCF B determines UE A is accessing the network via 3GPP-NB-IoT (GEO) based on the UE A’s access network in the INVITE and UE B has subscribed to the CAT service, S-CSCF B should not trigger CAT AS.

5.	S-CSCF A forwards 183 Progress response with SDP answer to P-CSCF A.

6.	P-CSCF A acts as a B2B UA and generates 180 Ringing with SDP answer based on the received 183 Progress response, then sends it to UE A.

7-10.	P-CSCF A acts as a B2B UA and generates PRACK as an acknowledgment of receiving 183 Progress messages, then sends it to the terminating network and receives 200 OK for PRACK.

11-12.	When UE A determines resource reservation is complete, it sends PRACK as an acknowledgment of receiving SDP answer and resource reservation completion, and UE A starts playing the ring back tone.

13-16.	P-CSCF A acts as a B2B UA and generates UPDATE as an acknowledgment of resource reservation completion, then sends it to the terminating network and receives 200 OK for UPDATE.

17.	P-CSCF A sends 200 OK response to UE A for PRACK acknowledgment.

18-19.	UE B sends 180 Ringing response to S-CSCF A, S-CSCF A forwards 180 Ringing response to P-CSCF A.

20-21.	When User B answers the call, UE B sends 200 OK response for INVITE to S-CSCF A, S-CSCF A forwards 200 OK response for INVITE to P-CSCF A.

22.	P-CSCF A sends 200 OK response to UE A.

23.	UE A sends ACK to P-CSCF A.

24-25.	P-CSCF A sends ACK to S-CSCF A, S-CSCF A forwards ACK to UE B.

#### 6.14.2.2	Case2: UE B supports VoGoN and is accessing network via 3GPP-NB-IoT(GEO)

In this case the simplified call setup procedure can be performed end to end because UE B is capable of sending 180 Ringing with SDP answer.

Figure 6.14.2.2-1: Call procedure for Case2: UE B supports VoGoN and is accessing network via NB-IoT over GEO

1.	UE A sends a SIP INVITE request containing an SDP offer to S-CSCF B.

2-3.	S-CSCF B forwards SIP INVITE request to the UE B.

NOTE:	If S-CSCF B determines either UE A is accessing the network via 3GPP-NB-IoT(GEO) based on UE A’s access network in the INVITE or UE B is accessing the network via 3GPP-NB-IoT(GEO) and UE B has subscribed to the CAT service, S-CSCF B should not trigger CAT AS.

4.	Based on the knowledge that UE B is accessing network via 3GPP-NB-IoT(GEO), UE B generates SIP 180 Ringing with SDP answer and UE B does not indicate precondition in Require header.

5.	P-CSCF B forwards 180 Ringing to S-CSCF B.

NOTE:	If CAT AS determines UE B is accessing the network via 3GPP-NB-IoT(GEO) based on the UE B’s access network in the 180 Ringing and UE B has subscribed to the CAT service, the CAT AS B should not modify the SDP answer for CAT service in case S-CSCF B has triggered CAT AS B.

6.	S-CSCF B forwards 180 Ringing to UE A via S-CSCF A.

NOTE:	If CAT AS A determines UE B is accessing the network via 3GPP-NB-IoT (GEO) based on UE B’s access network in the 180 Ringing and UE A has subscribed to the CAT service, the CAT AS A should not modify the SDP answer for CAT service in case S-CSCF A has triggered CAT AS A.

7-9.	UE A sends PRACK to S-CSCF B, S-CSCF B forwards PRACK to UE B.

10.	UE B alerts User B when UE B receives PRACK and UE B resource reservation is complete.

11-13.	UE B sends 200 OK response for PRACK to P-CSCF B, P-CSCF B forwards 200 OK response for PRACK to S-CSCF B, S-CSCF B forwards 200 OK response for PRACK to UE A.

14-16.	UE B sends 200 OK response for INVITE to P-CSCF B, P-CSCF B forwards 200 OK response for INVITE to S-CSCF B, S-CSCF B forwards 200 OK response for INVITE to UE A.

17-18.	UE A sends ACK to S-CSCF B, S-CSCF B forwards ACK to P-CSCF B.

19.	P-CSCF B sends ACK to UE B.

### 6.14.3	Impacts on Services, Entities and Interfaces

The following entities are impacted based on the solution above:

Originating UE:

-	Does not include Precondition in Supported header when it is accessing network via 3GPP-NB-IoT(GEO).

-	Sends PRACK as an acknowledgment of receiving SDP answer and resource reservation completion.

Terminating UE:

-	Determines generating SIP 180 Ringing with SDP answer in case at least one of UEs is accessing network via 3GPP-NB-IoT(GEO).

-	Does not include Precondition in Require header when it is accessing network via 3GPP-NB-IoT(GEO).

Originating IMS:

-	Indicates originating UE is accessing network via 3GPP-NB-IoT(GEO) in INVITE to the terminating network and suppresses CAT service.

-	Introduce Precondition in Supported header when originating UE indicates not supporting Precondition and is accessing network via 3GPP-NB-IoT(GEO)

-	Acts as a B2B UA in case the originating UE is accessing network via 3GPP-NB-IoT(GEO), but terminating UE does not support VoGoN.

-	Performs simplified call setup procedure to originating UE and normal call procedure to terminating side.

-	Can decode the indication that the terminating UE is accessing network via 3GPP-NB-IoT(GEO) contained in 180 Ringing and suppress CAT service.

Terminating IMS：

-	Can decode the indication that the originating UE is accessing network via 3GPP-NB-IoT(GEO) contained in the INVITE and suppress CAT service.

-	Indicates that terminating UE is accessing network via 3GPP-NB-IoT(GEO) in 180 Ringing to the originating network and suppresses CAT service.

# * * * * End of changes * * * *