3GPP TSG SA WG2#171	S2-2509125

Wuhan, China, October 13 – 17, 2025	(revision of S2-2508130)

Source:	Lenovo

Title:	Proposed scope update for WT3.1.1 and WT3.1.2

Document for:	Approval

Agenda Item:	20.6.3

Work Item / Release:	FS_6G_ARC/Release 20

Abstract of the contribution:

# 1	Discussion

The following is justification to update the postponed document from SA2#170 (S2-2508130). Two sub-work tasks have been defined. WT#3.1.1 focuses on studying for 6G new AI-related functionalities (such as AI agents) whereas WT#3.1.2 mainly focuses on enhancing existing AI/ML related functionalities defined during 5G.

The following provides justification to update WT3.1.1 and WT3.1.2.

Justification for WT#3.1.1: Evolving from an Analytics-Driven to an Intent-Driven Network

As also described in contribution S2-2507196, the 5G System introduced the NWDAF framework, which enables data collection and providing historical / predictive analytics to other network functions. The architecture supports distributed deployments (e.g., central, regional, edge) and collaborative model training (using FL), which are critical elements for network optimization.

While the NWDAF provides the necessary analytics and the NWDAF framework successfully distributes the analytics capability the decision to acts on such analytics still resides with the analytics consumer. This separation between "analysis" and "action" can introduce latency and complexity, limiting the network's ability to react in real-time to dynamic conditions.

To achieve deeper integration of intelligence, taking also into account that AI may be an inherent functionality in each NF in the core, it is essential to evolve from a network that is analytics-driven to one that is intent-driven where based on an intent (e.g. "goal") each NF in the core is able to determine the appropriate actions to achieve the goal. This requires a framework where intelligence is not just a service to be consumed, but a native capability with the power to act autonomously.

To meet the 6G goals of a lean, efficient, and fully autonomous network, the architecture must support "autonomous agency". This means embedding intelligent entities (e.g., AI Agents) directly into the core network architecture that can perceive the network environment, reason, and execute actions to achieve a high-level goal (an "intent").

The required evolution from 5G to 6G intelligence can be summarized as shown in the table below.

Table 1: Evolution from 5G to 6G intelligence

| Aspect | NWDAF in 5G/5GA | Evolution in 6G |
| --- | --- | --- |
| Primary role | Provides Analytics-as-a-Service. Other NFs consume analytics and then decide how to act. | Provides Intelligence-as-a-Native-Function. AI Agents are architectural components with the autonomy to act directly. |
| Operational model | Analytics-driven. The action is separated from the analysis. Intelligence is an "add-on" capability. | Intent-driven. An AI Agent receives a high-level goal and autonomously orchestrates NFs to achieve it in a fast, closed loop. |
| Architectural Focus | Focuses on distributing the data analytics capability. | Focuses on distributing the autonomous agency capability. |

Observation for WT#3.1.1:

-	It is important to highlight for WT#3.1.1 that the main goal is to evolve from an analytics-driven network to an intent-driven network. How to achieve intent driven network can be summarised in a solution, e.g. introduce AI agent or introduce a procedure where reinforcement learning is used to optimise an NF operation.

Justification for WT#3.1.2: Enhancing AI/ML Operations for Native AI

The NWDAF framework already supports various methods to provision trained AI/ML models to an NWDAF as well as, more importantly, defined methods to train AI/ML models, such as supporting model training using Federated Learning. In Rel-19, the NWDAF was also enhanced to support training of AI/ML models for other services such as train an AI/ML model for positioning to an AI-enabled LMF. Such LMF uses the trained AI/ML model to estimate location of a UE.

For 6G networks, AI is expected to be natively supported in multiple core network functions. As such the 6G network should support a framework to enable training of AI/ML models for different AI-enabled services within the 6G core. It is proposed to evolve the existing model training framework defined for the NWDAF. As part of the 6G study, additional model training mechanisms should be studied, such as online learning, reinforcement learning in order to have a comprehensive set of model training tools within the 6G network. In addition, for provisioning ML models to different AI-enabled functions in the core a framework supporting better model interoperability should be studied.

Observation for WT#3.1.2:

-	Highlight that main goal is to enhance the AI/ML related functionalities defined in 5G and introduce additional model training techniques (online learning, enhance vertical federated learning, reinforcement learning) and improve model interoperability.

# 2	Proposal

Based on the justification in the previous section the following are proposed to be included:

**** First Change ****

# Annex A.3. WT#3 Scope

WT#3: Study how to support and enable use of AI in 6G (e.g. AI agent, framework).

NOTE 4: 	The term AI agent refers to the general concept of agents autonomously performing tasks on behalf of users, systems, and/or applications. As the SA1 work is still in progress, adapting the definition of AI agent from SA1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

WT#3.1.1:	Study how to to evolve from a network that is analytics-driven to one that is intent-driven  in order to support customised services and achieve increased levels of performance, efficiency, scalability and resilience, while enabling multi-vendor interoperability. The following are part of the WT scope:

-	How to define an intent-driven request including defining the consumer of the intent-driven request, and which functions in the core can be recipients of such request.,

-	Based on the intent-driven request acting autonomously and controllably with acquiring contextual information, reasoning, explainability, self-learning, decision-making, generating results, interacting with other network functions, enabling closed-loop interactions,

WT#3.1.2:	Study whether and how to enhance AI/ML related functionalities from 5G including:

-	support AI-related functionalities (e.g. model training, model provisioning, performance monitoring) into 6G CN considering 5G CN AI-related functionalities as a starting point for discussion. Study support of additional model training techniques such as online learning, reinforcement learning and enhance existing defined techniques such as improving federated learning. Study support of better model interoperability ensuring reliable and sustainable AI.

NOTE:	For WT#3.1.2, aspects related to SA5 will be coordinated with SA5 during the study, aspects related to RAN will be coordinated with RAN during the study.

**** End of Changes ****