3GPP TSG-SA WG2 Meeting #171	S2-2509250

# October 13-17, 2025 Wuhan, China

Source:	Xiaomi

Title:	Interim agreement/Conclusion for KI#2 Authorization and Revocation to Support Sensing Service

Agenda item:	20.2.1

Document for:	Agreement

Work Item / Release: FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes Interim agreement/conclusions for KI#2 Authorization and Revocation to Support Sensing Service

1. Discussion

TS 23.700-14 v1.0.0 has documented 14 solutions relevant to KI#2 Authorization and Revocation to Support Sensing Service.

| Solutions | #1 | #2 | #3 | #4 | #5 | #6 |
| --- | --- | --- | --- | --- | --- | --- |
| #2 |  | X |  |  | X |  |
| #3 | X | X | X | X | X | X |
| #6 | X | X | X |  |  |  |
| #8 |  | X |  |  |  |  |
| #9 |  | X |  |  |  |  |
| #10 |  | X |  |  |  |  |
| #11 |  | X |  |  |  |  |
| #12 |  | X |  |  |  |  |
| #19 | X | X |  |  | X |  |
| #27 | X | X |  |  | X | X |
| #33 |  | X |  |  | X |  |
| #36 |  | X | X |  |  |  |
| #37 |  | X |  |  | X |  |
| #38 | X | X |  |  |  |  |

Based on the relevant 12 solutions, there are following observations and proposals.

Observation 1: sol#1, Sol#3, sol#6, sol#21, sol#31, sol#33, sol#34 propose new Gateway sensing function to support Sensing service Authorization, Sensing result exposure, discovery/selection for SF, etc. However, sol#3, sol#4, sol#5, #7, #8, #9, #10, #11, #12, #13, #14, #20, #21, #22, #24, #26, #27, #28, #29, #30, #32 and #36 propose to enhance NEF to support the discovery/selection of SF, and also can perform Sensing service authorization, sensing result exposure, not need to define one duplicated NF.

Proposal 1: to enhance NEF to support the following functionalities in principle 1:

- Authorization on Sensing Service Consumer (e.g., 3rd party AF);

- Performing Discovery and (Re-) Selection of Sensing Function (SF) or Sensing Control Function (SCF);

- Sensing Result and optional Sensing Contextual Information exposure to Sensing Service Consumer;

Observation 2: sol#3, #8, #9, #11, #12, #27 propose to support SF/SCF to authorize the Sensing Service request based on the parameters provided by 3rd party AF.

Proposal 2: Sensing Function (SF) or Sensing Control Function (SCF) is defined to support authorization for the Sensing Service Request based on the parameters provide by Sensing Service consumer (i.e., 3rd party AF) captured in Principle 2.

Observation 3: Sol#6, #11, #12, #36, #37, propose SF to revoke the sensing service authorization; Sol#8, #9 propose AF to revoke the sensing service authorization; Sol#6 proposes Gateway Sensing function (GSF) to revoke the sensing service authorization;

Proposal 3: authorized AF or SF (or SCF) can trigger to activate/de-activate/pause/revoke the authorization of the on-going Sensing Service in Principle 3.

Observation 4: Sol#4, #9, #11, #38, propose one storage NF to store the authorization information or profile.

Proposal 4: one storage NF (e.g., new NF, or enhanced NF) is defined to store the authorization profile or information to support authorization for Sensing Service. Captured in principle 4

Observation 5: Sol#11, for some case, house for intruder detection, body for sleep monitoring, or factory of AMR collision avoidance, in order to protect the interests or privacy of the owner of the sensing objects or required by regulations, additional check with the owner may still be needed to ensure the service request sent by the authorized sensing service consumer (e.g. the owner or the user authorized by the owner). Hence, it is proposed that:

-	permission/consent from the owner of the sensing target check is needed, as long as the information of user or owner is contained in the service request explicitly or implicitly. If the owner is a subscriber of the network receiving the service request, network obtains the permission/consent from the subscriber via the UE.

-	If the owner cannot be mapped to a subscriber of any mobile network, an owner list or allowed user list associated with the sensing object needs to be configured in the service profile.

Proposal 5: permission/consent from the owner of the sensing object may need to check, if the user/owner is the network subscriber, when the user/owner information is included in the Sensing Service Request.

2 Proposal

It is proposed to include the following changes in TR 23.700-14 V1.0.0.

* * * First Change * * * *

7	Interim Agreements

7.1	Agreed Principles

7.1.Y	Agreed Principles for KI#2 Authorization and Revocation to Support Sensing Service

To supports KI#2 Authorization and Revocation to Support Sensing Service, the following principles are concluded:

Principle 1: enhanced NEF is supported to provide authorization on Sensing Service Consumer (e.g., 3rd party AF) with following enhancements:

- Authorization on Sensing Service Consumer (e.g., 3rd party AF);

- Performing Discovery and (Re-) Selection of Sensing Function (SF) or Sensing Control Function (SCF);

- Sensing Result and optional Sensing Contextual Information exposure to Sensing Service Consumer;

Principle 2: Sensing Function (SF) or Sensing Control Function (SCF) is defined to support authorization for the Sensing Service Request based on the parameters: target sensing service area,

Principle 3: authorized AF or SF (or SCF) can trigger to activate/de-activate/pause/revoke the authorization of the on-going Sensing Service.

Principle 4: one storage NF (e.g., new NF, or enhanced NF) is defined to store the authorization profile or information to support authorization for Sensing Service.

Principle 5: permission/consent from the owner of the sensing object may need to check, if the user/owner is the network subscriber, when the user/owner information is included in the Sensing Service Request.

* * * End of Changes * * * *