SA WG2 Meeting #171	S2-2509456

13 – 17 October, 2025, Wuhan, China	was S2-2509408, S2-2508905

Source:	Futurewei, Samsung

Title:	New solution for KI#2: Support reliable time-based Data Collection of DO-A AIoT device Sensor with network trigger

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC/Rel-20

Abstract of the contribution: This contribution proposes a solution to support DO_A AIoT sensor time-based data collection with network assistance regarding Key Issues for WT#2.

Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-30.

* * * Start of Change (all new text) * * * *

### 6.X	Solution #6.X: Support reliable time-base Data Collection of DO-A AIoT sensor device with network trigger

#### 6.X.0	High-level solution Principles

This solution addresses KI #2 to support DO-A sensor data collection with network assistance, with the following key principles:

- The AF starts the time-based (e.g. periodic or at certain time) data collection service from the DO-A sensor device(s) by sending the AIoT command request to NEF, with the assumption that all the DO-A sensor device(s) for this operation have been registered with the 5G network before the service starts( e.g. DO-A sensor device can autonomously register to the network without the trigger from network)

NOTE: How the DO-A sensor device registers to the network is not covered in this solution and other DO-A device registration solutions can be applied.

- The data collection pattern of the AIoT sensor device(s) (e.g. periodic and aperiodic time-based data collection) is configured and aware by the AF. The AF provides the data collection pattern information to the AIoTF, which also provides the information to the NG-RAN / AIoT reader(s) to coordinate the data collection.

- The AIoTF provides the assistance information to the AIoT reader(s) or the NG-RAN to assist time-based (e.g. periodic or aperiodic) data collection (e.g. pre-allocate resource and send triggers for the DO-A sensor device(s)).  The assistance information includes periodic data collection information (e.g. start or end time of data collection, interval between periodic data collection, or certain point of time when the data collection is needed, so on).

- For periodic data collection, the AF and the AIoTF use single command procedure as defined in TS 23.369 to start the time-based data collection operation from DO-A AIoT device(s), without sending read command for every periodical data collection cycle.  The AIoT reader periodically trigger and allocated radio resource for the data collection of the AIoT sensor device(s), based on the assistance information from the AIoTF.

- This solution only focuses on time-based (e.g. periodic or at certain time) data collection of DO-A device(s) and doesn’t prevent DO-A device(s) transmitting data autonomously (e.g. event trigger transmission).

NOTE 1: The trigger and resource allocation mechanisms of AIoT reader are determined by RAN WGs and the coordination with RAN WGs is needed.

NOTE 2:  Security consideration for periodic data collection operation will be determined by SA3.

Editor’s note: How to support DO-A device mobility is FFS.

#### 6.X.1	Description

One of the key DO-A use cases is time-based (e.g. periodic ) data collection from AIoT sensor. Considering the nature of DO-A IoT device which is still a low energy and low complex device, keeping accuried and network-sychronized clock for periodic data transmission in the AIoT sensor is challenging. Although DO-A device can autonoumously sending data, a large number of AIoT sensor device sending periodic data at the promiximty time without network coordination can cause serious race condition for the network resource which lead to low effective of data collection.  This solution allows AIoT reader, with the time-based data collection assistance information provided by AIoTF,  to periodically or at-certain point to preallocate radio resouce and trigger the DO-A AIoT sensor device to send sensor data when the transmission time is come. With this solution, the DO-A sensor device doesn’t need to maintain a timer which is need to be synchronized with network or AF for periodic transmission.

#### 6.X.2	Procedures

Figure 1 periodic data collection

1.	Step 1 to step 6 of Procedure for Command in clause 6.2.3 of TS 23.369 [3] with additional parameters. AF sends AIoT Command Request to NEF. In includes Device(s) Info, Location (External), AF Transaction ID, Command Type Read, Length, Periodic data collection indication, Approximate number of AIoT Devices, and periodic data collection Parameters (periodic data collection start / end time, interval between each data collection cycle). NEF selects AIOTF, and then NEF sends AIoT Command Request to AIOTF. AIOTF checks AF Authorization by retrieving AF Authorization Profile in ADM. AIOTF sends AIoT Command Response to NEF. It includes Status (Success/Failure), Transaction ID. NEF sends AIoT Command Response to AF. It includes Status (Success/Failure), AF Transaction ID.

2.	Step 7 to step 10 of Procedure for Inventory in clause 6.2.2 of TS 23.369 [3]

3.	AIOTF sends AIoT Command Request to AIoT Reader via NG-RAN. This includes Device ID, (Reader ID), Command Type Read, Correlation ID, Length, and assistance information which includes Periodic data collection indication and periodic data collection Parameters (periodic data collection start / end time, interval between each data collection cycle). AIoT Reader stores Correlation ID with AIoT Device ID as context data as well as the assistance information.

4-5. When the periodic sensor data collection time starts, AIoT Reader sends AIoT Command Request to the AIoT Device. This includes Device ID, (Reader ID), Command Type Read, Correlation ID.

NOTE 3: AIoT reader or NG-RAN may allocate radio resource for the D2R command response from the DO-A Device. This is up to RAN WG decision.

6.	AIoT Device sends AIoT Command Response to AIoT Reader. This includes Device ID, and the data collected from the AIoT device(s).

7.	AIoT Reader sends AIoT Command Response to AIOTF. This includes Device ID, Sensing Result, and Correlation ID. AIoT Reader retrieves Correlation ID with AIoT Device ID from the stored context.

8.	AIOTF sends AIoT Command Notify to NEF. This includes Device ID, Sensing Result, and Transaction ID. AIOTF retrieves Transaction ID with Correlation ID from the stored context.

9.	NEF sends AIoT Command Notify to AF. This includes Device ID, Sensing Result, and AF Transaction ID. NEF retrieves AF Transaction ID with Transaction ID from the stored context.

10-14.	Every Period, AIoT reader sends AIoT command Request to the AIoT Device. Repeat step 5-9.

#### 6.X.3	Impacts on services, entities and interfaces

Impacts on existing entities:

AIOTF:

-	Receives time-based data collection information (e.g. periodic data collection information) from AF

AIoTF:

-	Add time-based data collection information to the assistance information, such as

- periodic data collection: periodic data collection indication, interval of each data collection cycle, start or end time of the periodic data collection.

- aperiodic data collection: time(s) when the data collection starts

AIoT Reader:

-    Maintain DO-A device context to conduct data collection procedure at certain point of time based on the assistance information.

-	Supports periodically start AIoT Command-only read procedure to collect data from AIoT Device(s) with triggering AIoT device(s) to send data.

-    For aperiodic data collection, supports triggering AIoT device to send data when the data collection time is up.

* * * End of Change * * * *