SA WG2 Meeting #171	S2-2509709

13 – 17 Oct, 2025, Wuhan, CN	(revision of S2-2509508)

Source:	Futurewei (penholder)

Title:	[WT#1.2, Enhanced User Plane] Scope and Key Issue for Enhanced User Plane

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: Enhanced User plane revision with updates to work task scope and key issue.

- Merged Documents

|  |  |  |  | User Plane architecture |  |  |  | Docs:=20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20.6.1.2 | S2-2508221 | P-CR | Approval | 23.801-01: [WT#1.2, Enhanced User Plane] WT Scope and Key Issue for Enhanced User Plane | CATT | Rel-20 | FS_6G_ARC | Revision of (Postponed) S2-2508121 |
| 20.6.1.2 | S2-2508350 | P-CR | Approval | 23.801-01: [WT#1.2, User Plane Architecture] User Plane Architecture for 6GS | vivo | Rel-20 | FS_6G_ARC | Revision of (Unhandled) S2-2506639 |
| 20.6.1.2 | S2-2508400 | P-CR | Approval | 23.801-01: [WT#1.2, UP Architecture] Scope update and Key Issue for User Plane Architecture | China Telecom | Rel-20 | FS_6G_ARC |  |
| 20.6.1.2 | S2-2508466 | P-CR | Approval | 23.801-01: [WT#1.2, User Plane Enhancement] User plane enhancement | Huawei, Hisilicon | Rel-20 | FS_6G_ARC |  |
| 20.6.1.2 | S2-2508522 | P-CR | Approval | 23.801-01: [WT#1.2, Enhanced User Plane and QoS] Scope and Key Issue for Enhanced User Plane and QoS | Lenovo | Rel-20 | FS_6G_ARC |  |
| 20.6.1.2 | S2-2508543 | P-CR | Approval | 23.801-01: [WT#1.2, Enhanced User Plane and QoS] Scope and Key Issue for Enhanced User Plane and QoS . | [Futurewei (pen holder), InterDigital Inc., Ofinno, Vivo, China Telecom, Ericsson, Cisco Systems, Deutsche Telecom, LG Electronics, Huawei, HiSilicon, Qualcomm, CATT, VODAFONE, NEC, AT&T, TNO], CMCC | Rel-20 | FS_6G_ARC | Revision of (Postponed) S2-2508121 |
| 20.6.1.2 | S2-2508552 | P-CR | Approval | 23.801-01: [WT#1.2, UP Architecture] Scope and New Key Issue for WT#1.2 User Plane Architecture | LG Electronics | Rel-20 | FS_6G_ARC | Revision of (Postponed) S2-2508121 |
| 20.6.1.2 | S2-2508613 | P-CR | Approval | 23.801-01: [WT#1.2, Enhanced User Plane] Scope and Key Issue for Enhanced User Plane | Futurewei | Rel-20 | FS_6G_ARC | Revision of (Postponed) S2-2508121 Handle |
| 20.6.1.2 | S2-2508769 | P-CR | Approval | 23.801-01: [WT#1.2, Enhanced User Plane and QoS] Key Issue for Enhanced User Plane | ETRI | Rel-20 | FS_6G_ARC | Revision of (Postponed) S2-2508121 |
| 20.6.1.2 | S2-2508847 | P-CR | Agreement | 23.801-01: [WT#1.2, UP] WT & KI for User plane architecture | Ericsson | Rel-20 | FS_6G_ARC |  |
| 20.6.1.2 | S2-2508907 | P-CR | Approval | 23.801-01: [WT#1.2, Enhanced User Plane and QoS] Scope and Key Issue for User Plane Architecture | Nokia, T-Mobile US, [Futurewei, InterDigital Inc., Ofinno, Vivo, China Telecom, Ericsson, Cisco Systems, Deutsche Telecom, LG Electronics, Huawei, HiSilicon, Qualcomm, CATT, VODAFONE, NEC, AT&T, TNO] | Rel-20 | FS_6G_ARC | Revision of (Postponed) S2-2508121 |
| 20.6.1.2 | S2-2508973 | P-CR | Approval | 23.801-01: [WT#1.2, User Plane Architecture] Key Issue on User Plane Architecture | NEC Corporation. | Rel-20 | FS_6G_ARC |  |
| 20.6.1.2 | S2-2508994 | P-CR | Approval | 23.801-01: [WT#1.2, user plane enhancements] User Plane Enhancement for 6G | Cisco Systems | Rel-20 | FS_6G_ARC |  |
| 20.6.1.2 | S2-2509049 | P-CR | Approval | 23.801-01: [WT#1.2, User Plane] Scope and Key Issue for 6G User Plane | Samsung | Rel-20 | FS_6G_ARC | Revision of (Postponed) S2-2508121 |
| 20.6.1.2 | S2-2509072 | P-CR | Approval | 23.801-01: [WT#1.2, Enhanced User Plane and QoS] Scope and Key Issue for Enhanced User Plane and QoS | [Futurewei (pen holder), InterDigital Inc., Ofinno, Vivo, China Telecom, Ericsson, Cisco Systems, Deutsche Telecom, LG Electronics, Huawei, HiSilicon, Qualcomm, CATT, VODAFONE, NEC, AT&T, TNO], Xiaomi | Rel-20 | FS_6G_ARC |  |
| 20.6.1.2 | S2-2509075 | P-CR | Approval | 23.801-01: [WT#1.2, Enhanced User Plane and QoS] Scope and Key Issue for Enhanced User Plane and QoS | Mediatek Inc. | Rel-20 | FS_6G_ARC |  |
| 20.6.1.2 | S2-2509111 | P-CR | Approval | 23.801-01: [WT#1.2, user plane architecture enhancement] Multi-vendor N4 interface | Vodafone | Rel-20 | FS_6G_ARC |  |
| 20.6.1.2 | S2-2509158 | P-CR | Approval | 23.801-01: [WT#1.2.X, User Plane] [WT#1.2.X, QoS] User Plane Architecture and QoS Framework in 6G | Ofinno | Rel-20 | FS_6G_ARC |  |
| 20.6.2 | S2-2509205 | P-CR | Approval | 23.801-01: Discussion and way forward for WT#2 (UP architecture) | Qualcomm | Rel-20 | FS_6G_ARC | Moved from 20.6.2 |
| 20.6.1.2 | S2-2509232 | P-CR | Approval | 23.801-01: [WT#1.2, Enhanced User Plane and QoS] Scope and Key Issue for Enhanced User Plane and QoS | Google, [Futurewei, InterDigital Inc., Ofinno, Vivo, China Telecom, Ericsson, Cisco Systems, Deutsche Telecom, LG Electronics, Huawei, HiSilicon, Qualcomm, CATT, VODAFONE, NEC, AT&T, TNO] | Rel-20 | FS_6G_ARC |  |

- Justifications

- - Application-Network Collaboration

A)View from majority of contributions is added (with markup). However, there are other views in (S2-250905, S2-2509075, S2-2509049): Is a user plane solution needed, or should it be only in WT 1.2 QoS, or not at all?

Motivation for this WT from proponents include the following:
- QoS is assigned to a PDU session and with encrypted packets it is not feasible for the network to know how to divide that QoS equitably between flows of the session that are dynamic (added/removed during session);
- relative priority of flows, application/flow characteristics (only application client side can provide this).
- immediate change in bandwidth by network can cause flow congestion/collapse (signal is sent to modem – PDU session modify, but is not available to the application).

B) Other clarifications:

(S2-2508350): “Whether and how to support a mechanism to address the dynamic change on the server IP address over UP”
This is the same as a new flow added in a PDU session: it is covered by the text in (1)
(S2-2508350): “Whether and how to support traffic identification and QoS differentiation for the QUIC traffic with dynamical change CID in 6G.”
This issue is identified in 5.X.1 (“ …QUIC connection migration which cause 5-tuple change ..”). CIDs may be negotiated E2E at some point/or frequently. But the negotiation is encrypted, and CIDs are encrypted in transmission.

(S2-2508400) “Whether and how to support negotiable data transmission mechanism between UP and data collection functions.
Only the general mechanism would be covered under WT#1.2 UP Arch.

(S2-2508522) “Whether and how to enhance application-network collaboration for 6G user plane traffic to enable the UE/AS to exchange information with the network that allows the network to adapt to application traffic or vice versa.”
“Exchange” of information has been noted by several contributions.

(S2-2509158) New clause “…a general unified framework on signaling exchange for session management via user plane path ..”
This would add a duplicate signalling mechanism. The “information exchange” in (1) should be sufficient to cover the identified gaps.

2 CP-UP Interaction

Simplify N4 vendor options and support simple services (e.g., FWA) (S2-2509111).
Alternate views: (S2-2508466, S2-2508847) suggest deletion.

3 SBA for N4

(S2-2508466) indicates that this is a solution.
(S2-2508847) indicates that SBI for N4 may complicate interworking. Enhancing PFCP should be considered.
The SBA aspect is general (i.e., not specific to user plane) and if needed should be studied in WT1.2 SBA.
The bullet point is deleted with change mark “8847-8466”.

4 Flexible User Plane

(S2-2509049) requests clarification for “Flexible User Plane”.
From (S2-2508613) Flexible in this context means that 3GPP system can measure and reselect to ensure optimal end-to-end performance. Added new “NOTE x4”.

(S2-2508466) has similar text/bullet point 4 for “service requirements” and bullet point 5 for “session continuity, mobility”. Combined both into the same clause for “flexible user plane”.

5 SSC and Mobility

(S2-2508613) identify gaps to mobility and session continuity related to HRLLC, (S2-2508466) to runtime and dynamic redundant user plane, and (S2-2508847 to most demanding services (e.g., URLLC).

6 Others:

(S2-2508221): “Whether and how to enhance the user plane handing for new capabilities or services(e.g., AI, Computing) in UPF.” 
Service specific issues should be addressed by that service. General UP enhancements seem to be captured already.

Many companies prefer enhancing the UP architecture, but a few companies seem to lean towards new UP architecture/ replace 3GPP UP protocol (GTP-U) with SRv6. (note that transport underlay can be SRv6, SR-MPLS or IGP with GTP-U already).
In line with the understanding from SA2#170: architectural aspects of the user plane will be studied in WT1.2 User Plane Architecture (and protocol aspects in CT4).
Therefore, the suggestion for all WT is to motivate and identify specific system aspects to be studied (just high level use cases/desired service characteristics are not enough). Companies can then bring solutions to these WT that are enhancements, or new UP architecture if there is no common understanding before the solution stage.

- (S2-2508221): “Whether and how to support replacing GTP-U with SRv6.”
This is a protocol issue for CT4.

- S2-2508350) “lot of signalling for SMF to relocate a UPF once a UPF suffers a heavy load”, “ .. SRv6 supports real-time adjustment of the UP path”.

- (S2-2508994): new bullet point 1: - “Explore new transport protocol or alternate methodologies which can meet performance, scalability, security and other needs of the future 6G network.”

(S2-2508994): bullet 1 on interworking and migration – addressed in WT 2 is added to the WT; SBA should be addressed in WT1.2 SBA item.

Key Issues:

As there were no KIs defined in the last meeting, companies have updated with multiple formats (sub-issues, different expansion from WT, etc.) and is difficult to converge/adopt a common format. This needs to be resolved.

In this first revision, the approach taken is to complement the “list” in WT section with additional KI details from various company contributions.

**** First Change ****

# Annex A.X. WT #1.2 User Plane Architecture

Items that have consensus:

In order to support 6G user plane for a diverse set of applications and traffic patterns, the following are studied:

- Whether and how to enhance CP-UP functional split for better multi-vendor interoperability.

- Whether and how to enhance flexible user plane (by UPF (re)selection) for different service requirements, session continuity and mobility, with consideration of path performance and capability between access network and data network.

NOTE y:  user plane Interworking and Migration will be studied in WT#2.

Items that need further discussion:

- (Qualcomm proposes to address this in WT1.2 QoS)
Whether and how to enable that UE/application and 6G CN can adapt the handling of application traffic (e.g. for packet detection, QoS, handling of headers) when e.g. application traffic is encrypted or IP addresses change dynamically, including:
- study whether and how UE/application and the 6G CN exchange information via the user plane path which can be used to apply PCC rules for the application traffic e.g. for the purpose for packet detection, QoS, charging, handling of headers, etc.

NOTE x1: applying PCC rules require visibility of E2E headers (packet detection, header enrichment, etc.)

NOTE x2: This WT needs to be coordinated with the 6G work for QoS related topics

4. 	 (request to remove by Huawei)
 Whether and how to extend the service-based architecture for the user plane control interface to UPF.

5. 	 (request to add from Interdigital, DT, Cisco, Charter, CATT)
 Study whether there are any architectural requirements that could influence the design or choice of user plane protocols in the 6G network.

NOTE: SA WG2 will not design or select user plane protocol(s). However, an output of this work task may be architectural requirements that stage 3 working groups may consider when making protocol related decisions.

**** Second Change ****

# 5.X. Key Issue #X: User Plane Architecture

5.X.1	Key Issue #X.1 Enhanced User Plane Architecture for 6G control interface towards UPF

This key issue is focused towards improving the interaction of UPF with control plane entities and include:

- Whether and how to enhance CP-UP interaction for better multi-vendor interoperability, e.g. by identifying options on the N4 interface and studying how to remove them in the 6G System.

5.X.2	Key Issue #X.2 Enhancements on User Plane Flexibility, mobility and session continuity

This key issue should focus on whether and how to enhance flexible user plane (by UPF (re)selection) for different service requirements, session continuity and mobility with consideration of path performance and capability between access network and data network:

- Study whether and how to establish/adjust user plane path by (re)selecting UPF(s) with consideration of service requirements and path performance and capability between access network and data network.

- Study whether and how to improve transmission quality by reducing impacts of N6 path.

- Whether and how to select/establish/adjust user plane path to satisfy service requirements, including latency, jitter, bandwidth, and packet loss for each segment between access network and data network.

- Whether and how to avoid sub-optimal routing caused by static anchoring or lack of path performance awareness.

- Study how to optimize control signalling to minimize the time for user plane path (re-)establishment between access network and data network.

- user plane handling for session continuity and mobility for demanding services and redundant user plane with consideration of network performance

NOTE x3a: Solutions should minimize the impact on re-establishment (i.e. release and creation) of resources/context in various entities (such as UE, RAN), triggered due to change of UP path.

**** End of Changes ****