3GPP SA WG2 Meeting #171	S2-2508219

13 – 17 Oct, 2025, Wuhan, China

Source:	NEC

Title:	[WT#2] Scope and key issue on Migration and Interworking for voice

Document for:	Agreement

Agenda Item:	20.6.2

Work Item / Release:	FS_6G_ARC/Rel-20

This contribution proposes scope and key issue to Work task 2 of the 6G study on Migration and Interworking for voice.

1. Justifications

This contribution discusses the scope and proposes a corresponding key issue to facilitate work on Work Task #2 of the approved 6G SID (SP-250806):

WT#2: Study migration and interworking, including

-	How to support migration to 6GS

-	How to support interworking with 5GS

-	Whether and how to support interworking with EPS

-	How to support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS.

NOTE 2:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is assumed to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 3:  The detailed migration study scope will be coordinated and aligned with RAN.

The contribution builds on the baseline of S2-2508129, which was postponed at the end of SA2#170.

2. Proposal

This contribution proposes to update the annex of TR 23.801-01 to include the scope of WT#2 and the related Key Issue for voice.

**** Tracked changes on top of S2-2508129 ****

**** First Change – all text new ****

Annex A.X. WT#2 Migration and Interworking Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#2: Study migration and interworking, including

1	How to support migration to 6GS, including Voice over 6GS (Vo6G).

2	How to support interworking with 5GS (including when using 5G NTN/satellite access), including the interworking between VoNR and Vo6G.

3	Whether and how to support interworking with EPS (including when using 4G NTN/satellite access) including the interworking between VoLTE and Vo6G.

NOTE 1:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is assumed to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 2: The detailed migration study scope will be coordinated and aligned with RAN.

NOTE 3: The Work Task should consider roaming aspects.

**** Second Change – all text new ****

5.X Key Issue Y: 6GS interworking and migration for voice

This key issue studies migration and interworking for voice service, including

1	How to support migration to Vo6G.

2	How to support Vo6G interworking with VoNR (including when using 5G NTN/satellite access).

3	Whether and how to support interworking with VoLTE (including when using 4G NTN/satellite access).

4	Whether and how to support Vo6G interworking with IMS voice service over NB-IoT NTN via GEO satellite connecting to EPC.

NOTE 1:	Interworking with 2G/3G and CS (Circuit switch) are not considered in this Key issue. Interworking between Vo6G and VoNR is assumed to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 2: The detailed Vo6G migration study scope will be coordinated and aligned with RAN.

NOTE 3: This Key issue should consider roaming aspects.

**** End of Changes ****