3GPP TSG-SA2 Meeting #171	S2-2508244

13- 17 October, 2025, Wuhan, China

Source:	China Telecom

Title:	23.700-19: KI#1 Solution to support reselection of P-CSCF for GEO access

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This paper proposes solution to support P-CSCF reselection in case the P-CSCF for GEO access is different from other access.

1 Discussion

Since the IMS signalling procedures for NB-IoT GEO access in R20 is significantly different from legacy IMS signalling procedure for other access, e.g. E-UTRA or NR, and the PGW may support non-IP/IP conversio, it is possible that some operators tend to deploy individual P-CSCF for GEO access seperated from the P-CSCF for other access, additionally with the possible deployment of different PGW of IMS PDN and/or different MME for the access. Some operators may deploy single P-CSCF and PGW for all accesses. Some operators may deploy different P-CSCF for different access, but same PGW for all accesses. Some operators may deploy different P-CSCF and PGW for different access. Some operators even more consider to deployment different MME for different access. For other PDN connection, e.g. internet, operator may use same PGW for all accesses to ensure service continue.

Considering above various deployments, this paper propose a solution to support reselection of P-CSCF in case of UE mobility between GEO and other access, by re-using existing procedures of P-CSCF restoration/re-selection and procedures of PDN connection re-establishment.

2 Proposal

It is proposed to agree the solution of support of reselection of P-CSCF in case of UE molility between GEO and other access into TR 23.700-19 on FS_5GSAT_Ph4_ARC as follows

* First change *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[7]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[8]	3GPP TS 22.261: "Service requirements for the 5G system".

[9]	3GPP TS 22.101: "Service aspects; Service principles".

[10]	3GPP TS 29.211: "Rx Interface and Rx/Gx signalling flows".

[11]	3GPP TS 29.213: "Policy and charging control signalling flows and Quality of Service (QoS) parameter mapping".

[12]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[13]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3".

[14]	3GPP TS 24.294: "IP Multimedia Subsystem (IMS) Centralized Services (ICS) protocol via I1 interface".

[15]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) centralized services; Stage 2".

[16]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[17]	3GPP TS 23.203: " Policy and charging control architecture".

[18]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[19]	IETF RFC 4566: "SDP: Session Description Protocol".

[20]	IETF RFC 5049: "Applying Signaling Compression (SigComp) to the Session Initiation Protocol (SIP)".

[21]	3GPP TS 24.228: "Signalling flows for the IP multimedia call control based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[22]	3GPP TS 23.003: "Numbering, addressing and identification".

[23]	3GPP TR 22.887: "Study on satellite access - Phase 4".

[24]	IETF RFC 3312: "Integration of Resource Management and Session Initiation Protocol (SIP)".

[25]	3GPP TS 23.380: " IMS Restoration Procedures".

* Second change *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 |
| #X | X |  |  |  |  |

* Third change *

6.X Soultion #X: Reselection of P-CSCF between GEO access and other access

6.X.0	High-level solution Principles

This solution addresses KI#1, by proposing EPC enhancements to support reselection of P-CSCF in case that UE changes its access, e.g. from RAT of NB-IoT GEO to RAT of E-UTRA, or vice versa, considering that operator may deploy different P-CSCF, PGW and MME, or different combination of the three Network Elements for different access, since the IMS signalling procedures for NB-IoT access in KI#2 is significantly different from legacy IMS signalling procedure for other access.

This solution reuses existing mechanism of P-CSCF restoration/re-selection and procedures of PDN connection re-establishment as described in 3GPP TS 23.380, for transferring to UE the new P-CSCF for the new access in either the existing or new established IMS PDN connection.

6.X.1	Description

In the proposal, if the RAT type is changed during UE mobility, the MME and PGW jointly perform re-slection/relocation for P-CSCF/PGW to handle the various deployment case for P-CSCF, PGW and MME. It also applies to roaming case where the MME is located in different PLMN from PGW.

As described in Annex L.2.2.1 of 3GPP TS 24.229, there exists 4 methods for P-CSCF discovery. In practice operators mainly use method II by which P-CSCF address is transfered within the EPS bearer context activation procedure. With this reality this proposal also uses core network controlled approach based on method II.

6.X.2	Procedures

During the UE mobility between different RAT type, especially between NB-IoT GEO and E-UTRA, the MME receives the new RAT type of UE during TAU or attach following exiting mobility procedure. The MME decides to disconnect existing IMS PDN connection due to RAT change, taking into account the capability of the PGW, e.g. whether the existing PGW of IMS PDN supports both the new RAT and old RAT, whether the existing PGW is a legacy PGW. The MME can have the knowledge of that PGW by configuration, or during DNS procedure for PGW selection.

- If the MME decides to disconnect existing IMS PDN connection, it uses the procedure of 5.10.3 of TS 23.401, with PDN connection reactivation request. During the IMS PDN connection re-establishment the P-CSCF(s) for the new RAT type is transferred to UE.

- If the MME decides to continue existing IMS PDN connection, it sends the new RAT type to PGW in Modify Bearer Request following exiting mobility procedure. After receiving the new RAT type, the PGW decides to disconnect or continue existing IMS PDN connection, by taking into account some factors, e.g. capability of existing P-CSCF supporting both RAT types, deployment of individual P-CSCF for different RAT type, UE capability to support P-CSCF re-selection.

- If the PGW decides to disconnect existing IMS PDN connection, it uses the bearer deactivation procedure of        5.4.4.1 of TS 23.401, with PDN connection reactivation request. During the IMS PDN connection re-establishment the P-CSCF(s) for the new RAT type is transferred to UE.

- If the PGW decides to continue existing IMS PDN connection, it transfers address of new P-CSCF(s) for new RAT type to UE in PCO of Update Bearer Request following the procedure of Annex L.2.2.1C of 3GPP TS 24.229.

After receiving the P-CSCF address in existing or new established IMS PDN connection, the UE performs registration to the P-CSCF following specific IMS procedure for the corresponding RAT type.

NOTE 1: To avoid interruption, the MME and PGW can delay the above procedures of P-CSCF restoration/re-selection and procedures of PDN connection re-establishment when there is an ongoing voice communication transfered from GEO access to E-UTRA access.

6.X.3	Impacts to Services, Entities and Interfaces

PGW:

-	Disconnect the UE's IMS PDN connection and instructs the UE to establish a new IMS PDN connection by using the procedure of 5.4.4.1 of TS 23.401, based on UE capability and P-CSCF capability.

-  Transfer address of P-CSCF(s) for new RAT type in existing or new established IMS PDN connection, if the PGW supports the new RAT.

MME:

- Disconnect IMS PDN connection and instructs the UE to restablish a new IMS PDN connection by using the procedure of 5.10.3 of TS 23.401, based on RAT type change and capability of existing PGW

* End of change *