SA WG2 Meeting #171	S2-2508372

13 – 17 Oct, 2025, Wuhan, CN

Source:	NTT DOCOMO

Title:	[WT#6] Scope and key issue on computing

Document for:	Approval

Agenda Item:	20.6.6

Work Item / Release:	FS_6G_ARC/Rel-20

This contribution proposes WT scope (with changes on the baseline of the postponed paper S2-2508133 at SA2#170) and Key Issue description for Work task 6 (computing) of the 6G study.

# 1. Justifications

This contribution discusses the scope and proposes a corresponding key issue to facilitate work on Work Task #6 of the approved 6G SID

WT#6: Study aspects on support of computing for UE, core network and application server in 6G (e.g. coordination between UE, core network and application server, exposure of computing service in the core network, etc.).

The contribution builds on the baseline of S2-2508133, which was postponed at the end of SA2#170.

## 1.1 Architectural Motivation

3GPP SA1 TR 22.870 defines 6G Computing Service as “a service provided by 6G network utilizing computing resources in Service Hosting Environment, which can be used by a subscriber (via UE)/3rd party”, with the following note:

“NOTE 1: The computing resources can refer to hardware and/or software that provides the required processing, storage capability etc. to perform computational tasks (e.g. XR rendering)”.

According to TS 22.261, the Service Hosting Environment is “the environment, located inside of 5G network and fully controlled by the operator, where Hosted Services are offered from”.

6G Computing services can enable immersive communication use cases based on compute-intensive tasks such as multi-media services (3GPP TR 22.870 section 9.2), 3D video streaming (3GPP TR 22.870 section 9.3) and immersive gaming (3GPP TR 22.870 section 9.1). The compute capacity available in the user device may be limited, whereas the available compute capacity in the 6G network may provide acceleration capabilities, hence applications can benefit from offloading compute load (e.g., XR rendering offload, AI workload split) from the user devices to the Service Hosting Environment.

Application providers developing applications that require to execute compute-intensive tasks on the user device may, based on an SLA with an MNO, deploy their applications on the user device while offloading application workload from the user device to the operator’s network; this way, the applications can be supported also by small form factor user devices that have battery limitation and limited compute capabilities.

Observation 1: by offloading application workload from the user device, compute-intensive applications can be supported also by small form factor user devices that have battery limitation and limited compute capabilities.

In terms of utilization of compute resources and energy consumption, it is more efficient to aggregate the application workload in the telco cloud platform instead of executing it in the user devices. The telco cloud platform can be exploited to host compute-demanding application workload, making efficient use of high-performance computing resources (with acceleration capabilities). The telco cloud platform may be used to host application workload in addition to virtualized network functions (e.g., Core NFs).

Observation 2: offloading compute-intensive application workload from the user device to the telco cloud platform enables efficient use of high-performance computing resources, which may be shared between network workload (Core NFs) and application workload.

Finally, it is experimentally demonstrated [1][2] that executing UPF and application data processing in the same computing instance (e.g., Kubernetes Pod) enables performance gains like higher throughput and lower packet jitter in comparison to extracting the PDU payload in the UPF and routing the application data packets via N6 interface to an application server. Applications involving video streaming benefit from high and stable throughput and low jitter.

Conclusion: offloading compute-intensive application workload from the user devices to the Service Hosting Environment enables innovative services as well as efficient use of energy and high-performance computing resources offered by the telco cloud platform.

[1] H. Baba, S. Hirai, K. Hayashi, and T. Takeda, "6G In-Network Computing Architecture for Service Acceleration Prototype and Evaluation," 2025 IEEE Wireless Communications and Networking Conference (WCNC), Milan, Italy, 2025, pp. 1-6

[2] S. Hirai, K. Hayashi, H. Baba, and T. Takeda, "DPU-Based Telecom Network Architecture Toward 6G and AI-Native," 2025 IEEE 11th International Conference on Network Softwarization (NetSoft), Budapest, Hungary, 2025, pp. 463-467

## 1.2 Gap Analysis

As observed in the previous section, MNOs can make efficient use of telco cloud resources by executing application workload in the telco cloud platform (observation 2). Moreover, MNOs can enable innovative services by exploiting application performance gains that can be achieved by joint selection or even integration of UPF and compute node.

5G EC features do not allow performing joint selection of UPF and compute resources in the Service Hosting Environment to fulfil joint requirements on communication QoS and compute resources. Edge Computing (EC) features in 5G and 5G Advanced (Rel-15-Rel-20) only allow for routing traffic (with distributed anchor point or session breakout) to an Edge Application Server (EAS), where application workload can be hosted. The selection of the EAS is based on resolving DNS queries, which may be influenced by the 5G core network using 5G EC solutions. This way, the coordination between communication QoS and allocation of compute resources is loosely coupled, bringing to suboptimal application performance.

The application workload that can be offloaded from a user device may depend on:

- The communication QoS that can be supported between the UE and the UPF and between the UPF and the EAS.

- The availability of compute resources in the Service Hosting Environment, in the EAS in the data network, and in the user device.

The optimal trade-off between communication QoS and allocation of compute resources in the user device, in Service Hosting Environment and in the EAS should take into considerations application constraints. Some examples of application constraints are: maximum application latency (e.g., maximum summation of the time to execute the compute tasks given the allocated compute resources plus the time to transfer the application data between UE, Service Hosting Environment and EAS), maximum energy consumption in the user device, or maximum total end-to-end energy consumption (e.g., summation of the energy consumption in the user device, the Service Hosting Environment and the EAS in the data network).

Coordinated control of communication QoS and compute resources is essential to effectively fulfil application requirements (on communication QoS and compute resources) and constraints (e.g., latency, energy consumption). Joint control of communication QoS and compute resource allocation should be supported also in mobility scenarios, and at UP changes or changes in the availability of compute resources.

Observation 3: to fulfil application requirements and constraints, the 6G network should support joint control of communication QoS and compute resources in the Service Hosting Environment, also in mobility scenarios.

In 5G, the application layer can issue communication QoS requirements for PDU sessions (like in TS 23.502 clause 4.15.6.6 on setting up an AF session with required QoS) but it cannot issue compute requirements or application constraints (e.g., maximum latency or energy consumption). The 5G network enables only loosely coupled coordination between the 3GPP system and the application layer upon changes in the QoS that can be supported by the network (by notifying the AF that the required QoS cannot be supported) or at UP changes and EAS server relocation (by using early and late notifications to the AF about UP changes, like in TS 23.502 clause 4.3.3.6).

Observation 4: the 6G network should enable the application layer to issue QoS requirements for a PDU session (like in 5G) as well as compute requirements and application constraints.

In the S2-2508133 baseline compiled in SA2#170, it is observed that application layer mechanisms defined by SA6 (such as the EDGEAPP framework in TS 23.558) may be considered to realize the computing service in 6G. In fact, also the EDGEAPP framework does not allow to perform joint control of 3GPP User Plane and compute resources. The EDGEAPP framework provides support to the UE to perform selection of the edge application server, but coordination with the 3GPP User Plane is performed via exposure framework. When standardizing the 5G EC framework, there was an explicit agreement (S2-1912771, S6-200357) to keep separated the 5G Core architecture specified by SA2 and the application layer architecture of SA6 (the only exception is that the 5G Core may provision information about the SA6 Edge Configuration Server to the UE). The SA6 application layer components that reside in the Edge DN may consume (via SA6-specified interfaces EDGE-2 and EDGE-7) the network function services exposed by the 5G Core via NEF (Network Exposure Function), resulting in loosely coupled coordination with the 3GPP User Plane.

Moreover, the EDGEAPP framework poses a technical challenge to the implementation and wide scale adoption of compute services: the User Equipment and the cloud platform need to support 3GPP-specific functionalities, i.e. respectively the Edge Enabler Client (that should be supported by the OS of the user device) and the Edge Enabler Server (that should be implemented by the cloud framework adopted by the MNO). These dependencies between different technical domains (3GPP network, OS of the user device, cloud platform) should be avoided to enable interoperability and global adoption of 3GPP-enabled compute services.

## 1.3 Technical Impact

Based on the gap analysis, WT#6 scope should address the following key issues:

- How to make the 6G network aware of compute resources available in the Service Hosting Environment, in the user device and in the data network.

- How the 6G network performs joint control of communication QoS and compute resources offered by the Service Hosting Environment to fulfil application requirements and constraints, also in mobility scenarios.

- How the 6G network enables application functions to issue QoS requirements for a PDU session as well as compute requirements and application constraints (e.g., on energy consumption or latency).

## 2. Proposal

This contribution proposes to update the annex of TR 23.801-01 to include the scope of WT#6 and the related Key Issue. The text proposed for the scope of WT#6 is based on the WT baseline discussed in SA2#170 and reflected in S2-2508133. On top the S2-2508133 baseline, the current contribution proposes few changes:

- Clarify that the scope of WT#6 is to study how to support “computing services”.

- In WT#6.2, clarify the bullet: “Whether and how to improve the service experience/QoS(e.g., by defining new metrics or enhancing the existing metrics for QoS)”. The text “improve the service experience/QoS” is unclear because an improvement should be defined against a term of comparison. Moreover, defining new metrics is not sufficient to improve the service experience, if the 6G system does not enforce the fulfilment of service requirements. In our understanding, the intention of this bullet is to study “whether and how to fulfil service experience/QoS requirements”.

- In WT#6.2, the term “compute site” is undefined. It is proposed to introduce a note to define the term and clarify that a compute site may be within the Service Hosting Environment in the 3GPP network or in the data network.

- On NOTE 3 in the baseline text, it is intended that the scope of WT#6 identifies the aspects to be studied in the scope of SA2: other aspects for enabling computing services in the scope of SA5 or SA6 may be addressed in a SA5 study (if any) or a SA6 study (if any); the outcome of those studies can be taken into consideration as input to the SA2 study.

The proposal for the Key Issue to study “support of computing services” reflects the scope of WT#6 by describing the 3 aspects that need to be studied.

**** First Change – all text new ****

# Annex A.X. WT#6 Computing

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#6: Study aspects on support of computing for UE, core network and application server in 6G (e.g. coordination between UE, core network and application server, exposure of computing service in the core network, etc.).

In order to support computing services, the following aspects need to be studied:

WT#6.1: Enablement/authorization of computing service for UE or AF.

  - Whether and how to enable UE/AF to request computing service.

  - Whether and how to authorize UE/AF request for computing service.

WT#6.2: Coordination of communication and computing, service continuity and QoS aspects:

  - Whether and how to coordinate communication QoS and computing resource control (e.g., within the core network by enhancing the existing NFs or introducing a new NF, outside the core network by operator or a 3rd party owned AF).

  - Whether and how to identify and/or collect the computing resource related information (e.g., computing resource type and/or status, location of the computing resource, etc.).

  - Whether and how to expose the computing resource information and/or network metrics.

NOTE 1:	 Potential SA2 defined exposure of network metrics/computing resource related information will follow the exposure framework defined in WT#1.2.

  - Whether and how to fulfil service experience/QoS requirements, which may be characterized by new metrics or enhancement of existing metrics for QoS.

  - Whether and how to support service continuity for computing service upon change of computing site and/or user plane function (e.g., due to UE mobility, computing load balancing, AF influence, etc.).

NOTE 2:	Compute site represents the compute resources in a specific location of network topology. The compute site can offer compute services by allocating its compute resources to run application workloads upon demand of a compute service consumer. From networking perspective, the compute site may be in the Service Hosting Environment inside the 3GPP network or in a location of a data network identified by a DNAI.

WT#6.3: Discovery and (re-)selection of compute site(s)/Application Server(s) for UE/AF requested computing service:

  - Whether and how to discover and (re-)select computing site(s)/Application Server(s) for UE/AF requested computing service.

  - Whether and how to re-use and/or enhance Edge Computing features specified in 5G (i.e., EAS discovery procedure specified in TS 23.548 can be used as starting point for discussion) to enable discovery or (re-)selection of computing site/Application Server to provide computing services  requested by UE/AF.

NOTE 3:  The outcome of a SA5 study (if any) or a SA6 study (if any) for enabling computing services can be taken into consideration as input to this study.

NOTE 4: Potential gaps on 5G Edge Computing design will be identified in study phase.

**** Second Change – all text new ****

# 5.X. Key Issue #X: Support of computing services

This key issue is to study support of computing services. To support computing services, the following aspects need to be studied:

1) Enablement/authorization of computing service for UE or AF.

  - Whether and how to enable UE/AF to request computing service.

  - Whether and how to authorize UE/AF request for computing service.

2) Coordination of communication and computing, service continuity and QoS aspects:

  - Whether and how to coordinate communication QoS and computing resource control (e.g., within the core network by enhancing the existing NFs or introducing a new NF, outside the core network by operator or a 3rd party owned AF).

  - Whether and how to identify and/or collect the computing resource related information (e.g., computing resource type and/or status, location of the computing resource, etc.).

  - Whether and how to expose the computing resource information and/or network metrics.

NOTE 1:	 Potential SA2 defined exposure of network metrics/computing resource related information will follow the exposure framework defined in WT#1.2.

  - Whether and how to fulfil service experience/QoS requirements, which may be characterized by new metrics or enhancement of existing metrics for QoS.

  - Whether and how to support service continuity for computing service upon change of computing site and/or user plane function (e.g., due to UE mobility, computing load balancing, AF influence, etc.).

NOTE 2:	Compute site represents the compute resources in a specific location of network topology. The compute site can offer compute services by allocating its compute resources to run application workloads upon demand of a compute service consumer. From networking perspective, the compute site may be in the Service Hosting Environment inside the 3GPP network or in a location of a data network identified by a DNAI.

3) Discovery and (re-)selection of compute site(s)/Application Server(s) for UE/AF requested computing service:

  - Whether and how to discover and (re-)select computing site(s)/Application Server(s) for UE/AF requested computing service.

  - Whether and how to re-use and/or enhance Edge Computing features specified in 5G (i.e., EAS discovery procedure specified in TS 23.548 can be used as starting point for discussion) to enable discovery or (re-)selection of computing site/Application Server in the data network to provide computing services requested by UE/AF.

NOTE 3:  The outcome of a SA5 study (if any) or a SA6 study (if any) for enabling computing services can be taken into consideration as input to this study.

NOTE 4: Potential gaps on 5G Edge Computing design will be identified in study phase.

**** End of Changes ****