SA WG2 Meeting #171	S2-2508452

13 – 17 October, 2025, Wuhan, China

Source:	OPPO, China Unicom

Title:	[WT#1.2] 6G QoS support

Document for:	Approval

Agenda Item:	20.6.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution is proposed to update the WT#1.2 QoS for close collaboration between applications and communications to achieve a satisfied user experience

1. Justifications

  - Motivation

In 6G, it needs considering the shifting from a QoS-centric approach to a QoE-centric approach. I.e., it no longer always pursues the ultimate QoS at the communication layer, but rather achieve deterministic QoE (i.e. less or no impact to QoE) through close collaboration between applications and communications. This helps to resolve the bandwidth limitation for some realtime service (such as XR rendering) and widely deploy the XR services for a high density of end-users.

As the application (e.g., XR rendering and video streaming) becomes more and more powerful to recover some kind of information missed due to data transmission via network which provides the network with some flexibility on QoS control. In particular, when connection quality becomes poor (e.g., due to bandwidth limitation or network congestion), the network lowers QoS with a certain level (such as increasing the transmission error rate or lowering the data rate for specific service data flow) while the receiving end/UE application can still achieve a deterministic user experience (i.e. less or no impact to QoE) by recovering the missed data in application layer (e.g., in use case in TR22.870 clause 9.10 and 9.14).

Figure-1: UE recovers data transmitted with high error rate in SA1 use case (TR22.870 clause 9.10)

Figure-2: network performs user experience-aware resource allocation (b), in SA1 use case (TR22.870 clause 9.14)

However, because recovering information in application needs to consume more power on the receiving end/UE. it is a trade-off between power consumption and communication resource consumption, and the lowering QoS by network should apply only when it is necessary (e.g., when communication link is poor).

Proposal:

Study how to make application and network to collaborate to achieve deterministic user experience, specifically how to improving the QoS framework to provide the network with flexibility on QoS control to deal with limited communication resource (e.g. when congestion or bandwidth limitation happens) which achieving a deterministic user experience.

The according updates (in cyan) is as below, which is made on top of S2-2508122 (baseline paper) postponed from SA2#170.

**** First Change****

Annex A.X WT#1.2.X: Evolution of QoS Framework for 6G

Editor’s Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

- Investigate if any new functionality in 6G QoS framework considering the emerging new traffic characteristics and application needs.

-	How to support a close collaboration between application and communication to achieve a satisfied user experience.

- How to enable the 6G CN with QoS control flexibility to deal with limited communication resource with no or little impacting to application user experience.

NOTE 1: The definition of 6GS QoS concepts in the 6G System provider perspective and in the 6G System usage perspective respectively should have clear boundary (enable clear & measurable connectivity service descriptions for SLAs).

- Whether and how to enhance QoS control mechanisms to

  - better support QoS targets than the best effort approach of existing non-GBR.

  - to improve QoS for applications with dynamic requirements.

- Whether and what to monitor in relation to QoS Framework.

- Study whether and how to support the UE and network QoS collaboration to improve awareness in the 6GS of connectivity performance needs.

  - UE provided information to the network for QoS collaboration

  - Application and Network QoS collaboration including what information shall be exchanged by means of ‘application/UE and network collaboration’ [see WT#1.2 User Plane].

NOTE 2: Applications typically consist of an Application Client in the UE and application on the server side.

- Study how to ensure interworking with 5GS.

NOTE 3: The content of the 5GS QoS Framework that will be used as a starting point for discussion on 6G QoS will be determined by moderated discussion process.

NOTE 4: Aspects of traffic detection are assumed to be handled by the WT#1.2 User Plane.

NOTE 5: Impacts on QoS originating from new services will be considered based on and when relevant input is identified and provided by the relevant WTs.

NOTE 6: This WT will coordinate with SA WG4 and RAN WGs.

**** End of Changes ****