3GPP TSG SA WG2 Meeting #171	S2-2508627

Wuhan, China, 13 – 17 October 2025	(revision of S2-2506361, S2-2508133)

Source:	China Mobile

Title:	[WT#6] WT and key issues for supporting 6G computing

Document for:	Agreement

Agenda Item:	20.6.6

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper proposes the discussion of 6G architecture to support computing, and proposes the WT and KI for 6G computing

1. Justifications

1.1 Gap analysis: 5G edge computing vs 6G Computing

During the SA2#170 meeting, that exists some views that whether 6G computing is a new architecture or just the enhancement of 5G edge computing architecture.

In R15 and R16, fundamental edge computing features are introduced including traffic impact, UL CL and etc to support traffic steering to edge. And in R17, new network function EASDF is introduced to act as the DNS resolver inside 5GC to assist to resolve the FQDN in UE DNS query to the EAS IP address which the edge application server near the UE in location.

But there are some differences between 5G edge computing and 6G computing:

(1) The purpose of 5G edge computing is to address the issues of local breakout to 3rd party’s application server.

5G edge computing focuses on the edge application server discovery near the UE location, and requires the 5GC to establish the traffic routing between UE and EAS. In 5G, the 3rd party application doesn’t have the requirements on computing, and whether the edge server satisfies the computing or not is also not discussed.

But in 6G, based on several SA1 requirements, some high computing required services, e.g.: AI services, have the requirements on computing, and the edge application server selected by 5G edge computing mechanism may not be the optimal on computing.

For example, as indicated in section 6.25.6 and 6.29.6 of TR 22.870[x], the potential requirements are shown below:

[PR 6.25.6-2]	Subject to operator’s policy, the 6G network shall be able to take into account third party’s requirement(s) (e.g. latency) and the computing resource availability for the AI/ML model training and inference within service hosting environment.

[PR 6.49.6-1] The 6G system shall be able to provide the communication and AI service (assuming AI inferencing in Service Hosting Environment) for user with the KPI requirements in the Table 6.49.6-1 below. Table 6.49.6-1: KPIs for physical AI robot to perform cognitive functions for to interact with human user via 6G network Use case; Traffic type; Average packet size (Byte); Transfer interval (ms); Data Rate (Mbps); Joint E2E latency (ms) (NOTE 4); Reliability. Support robot conscious awareness for interacting with human user; UL camera data (NOTE 1); [<1000]; [10]; [20-60] (NOTE 2); [<200] (NOTE 3) (NOTE 5); [99.9 %]. NOTE 1: 6 RGB cameras are equipped for robot “Figure 02” [180]. NOTE 2: 20 Mbps is for six 1080p cameras, and 60 Mbps is for four 1080p cameras plus two 4K cameras. Data Rate is calculated from video resolution*(Bits per colour*3) *Refresh rate/Compression ratio. Compression ratio of 240 and 8 bits per colour is assumed, thus data rate is around 3 Mbps for 1080p 15Hz, and around 24Mbps for 4K 30Hz. Compression ratio, bits per colour and refresh rate for 1080p refers to the similar cases for real-time video uploading of a vehicle as per YD/T 4778-2024 [182]. NOTE 3: For physical AI robot interacting with a human user, the robot is expected to mimic the similar basic human brain reaction time including conscious awareness/recognition and decision-making based on various stimuli. Such human brain reaction time ranges from mean auditory reaction time 140-160ms, touch 155ms, to visual reaction time 180-200ms [279]. NOTE 4: Joint E2E latency (i.e. round-trip communication latency, and AI inference latency in Service Hosting Environment). and UE is only considered to contribute to the communication service latency. NOTE 5: The human to robot latency (vice versa) is not included..

These requirements indicate that the for the AI services, the 3rd party’s requirements of computing resources are included and the new introduced E2E latency includes the AI inference latency. So, for current 5G edge computing architecture, it can not handle the 3rd party’s computing requirement and also, it is unaware of the latency that related to AI services. What 5G computing define is to select the edge application server which is topological near to the UE locally according to the FQDN resolution. It depends on that the 3rd party has already deployed application instance locally and operator cannot control this.

Observation 1: The 5G edge computing architecture only focus to select the application server locally deployed near the UE and has no perception of computing requirements. So the 5G edge computing can’t meet the new 6G computing requirements.

(2) Latency on N6 interface is uncontrollable.

Considering that the edge server is deployed in data network that behind the N6 interface, and the N6 interface is always not controllable by operator. It means that the time delay on N6 interface is also not controllable. So, the E2E service experience between UE and edge server is also uncontrollable. This cannot meet the requirements of 6G new services, such as 6G system should guarantee the QoS of computing Service requirement, e.g.: E2E time delay. For example, in section 6.29.6 of TR 22.870[x], the potential requirements indicate the joint E2E latency of AI inference:

NOTE 4: Joint E2E latency (i.e. round-trip communication latency, and AI inference latency in Service Hosting Environment). and UE is only considered to contribute to the communication service latency.

(3) According to the SLA with all kinds of application provider, the operator-controlled computing nodes can deploy the application instance to serve the UE at several location.

In 5G edge computing, the edge application server is deployed by 3rd party, and if the service provider doesn’t deploy the server on this location, there will be no edge computing. While compared with the 5G, in 6G we are discussing about the computing resources in core network. That is to say the operators can control the edge application deployment (based on the SLA) and provide the services to UE based on UE/application request.

Conclusion: New architecture that support computing in 6G should be provided.

**** First Change, Update based on Postponed version ****

Annex A.X. WT#X Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#6: Study aspects on support of computing for UE, core network and application server in 6G (e.g., coordination between UE, core network and application server, exposure of computing service in the core network, etc.).

NOTE 1: Application layer mechanism and exposure framework may require coordination with SA6.

In order to support computing, the following aspects need to be studied:

WT#6.1: Enablement/authorization of computing service for UE or AF.

  - Whether and how to enable UE/AF to request computing service.

  - Whether and how to authorize UE/AF request for computing service.

WT#6.2: Coordination of communication and computing, service continuity and QoS aspects:

  - Whether and how to coordinate communication and computing resource (e.g., within the core network by enhancing the existing NFs or introducing a new NF, outside the core network by operator or a 3rd party owned AF).

  - Whether and how to identify and/or collect the computing resource related information (e.g., computing resource type and/or status, location of the computing resource, etc.).

  - Whether and how to expose the computing resource information and/or network metrics.

NOTE 2:	 Potential SA2 defined exposure of network metrics/computing resource related information will follow the exposure framework defined in WT#1.2.

  - Whether and how to improve the service experience/QoS (e.g., by defining new metrics or enhancing the existing metrics for QoS).

  - Whether and how to support service continuity for computing service upon change of computing site and/or user plane function (e.g., due to UE mobility, computing load balancing, AF influence, etc.).

WT#6.3: Discovery and (re-)selection of compute site(s)/Application Server(s) for UE/AF requested service:

  - Whether and how to discover and (re-)select computing site(s)/Application Server(s) for UE/AF requested service.

NOTE 3:  Which of above aspects should fall into SA5 and/or SA6 scope will be further identified in study phase, coordination with SA5 and SA6 may be needed.

**** Second Changes, same as the WT above ****

5.X. Key Issue #X: Support of computing services in 6G network

This key issue will address the system architecture and function required to provide end-to-end support for computing.

The aspects to be studied in this key issue include:

-	Enablement/authorization of computing service for UE or AF.

-	Whether and how to enable UE/AF to request computing service.

-	Whether and how to authorize UE/AF request for computing service.

-	Coordination of communication and computing, service continuity and QoS aspects:

-	Whether and how to coordinate communication and computing resource (e.g., within the core network by enhancing the existing NFs or introducing a new NF, outside the core network by operator or a 3rd party owned AF).

-	Whether and how to identify and/or collect the computing resource related information (e.g., computing resource type and/or status, location of the computing resource, etc.).

-	Whether and how to expose the computing resource information and/or network metrics.

NOTE 2:	 Potential SA2 defined exposure of network metrics/computing resource related information will follow the exposure framework defined in WT#1.2.

-	Whether and how to improve the service experience/QoS (e.g., by defining new metrics or enhancing the existing metrics for QoS).

-	Whether and how to support service continuity for computing service upon change of computing site and/or user plane function (e.g., due to UE mobility, computing load balancing, AF influence, etc.).

-	Discovery and (re-)selection of compute site(s)/Application Server(s) for UE/AF requested service:

-	Whether and how to discover and (re-)select computing site(s)/Application Server(s) for UE/AF requested service.

NOTE 3:  Which of above aspects should fall into SA5 and/or SA6 scope will be further identified in study phase, coordination with SA5 and SA6 may be needed.

**** End of Changes ****