SA WG2 Meeting #171	S2-2508695

13 - 17 October, 2025, Wuhan, China

Source:	InterDigital Inc

Title: 	KI #3, New Agreed Principles: Agreed Principle for Key Issue #3

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract: This document proposes principles for KI#3.

# 1.	Introduction

At SA2 #171, S2-2507677 was a proposal on agreed principles for KI#3 that was discussed and noted. We assume that discussion of a revised version of S2-2507677 will continue at SA2 #172. InterDigital thinks that S2-2507677 is a good baseline for further discussion.

The purpose of this contribution is to propose to continue to use the text from S2-2507677 as a baseline for further discussion, with the following addition to the baseline KI#3 agreed principles:

The NRF may store NF Profiles for N3IWFs and TNGFs. N3IWF and TNGF profiles may include Energy Information (e.g. Energy Priority Information or Schedule of Energy Priority Information). The AMF may query the NRF for N3IWF and TNGF profiles so that the information can be considered for N3IWF or TNGF reselection.

This addition is based on Solution #29 and proposed to be part of the agreed principles so that an N3IWF or TNGF can be selected based on Energy Information (e.g. Energy Priority Information or Schedule of Energy Priority Information).

In the Rel-19 specifications, the AMF can not consider energy information when deciding whether to redirect the UE to a different TNGF or N3IWF. Selection of the N3IWF and TNGF influences what UPFs can eventually be selected when PDU Sessions are established. This enhancement will further optimize user plane energy consumption by allowing for energy information to be configured for N3IWFs and TNGFs and considered by the AMF.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-67 V1.0.0.

# * * * * First change (all new text) * * * *

### 7.1.3	Agreed Principles for KI#3

The following principles are agreed for KI#3:

-  The NRF may store NF Profiles for N3IWFs and TNGFs. N3IWF and TNGF profiles may include Energy Information (e.g. Energy Priority Information or Schedule of Energy Priority Information). The AMF may query the NRF for N3IWF and TNGF profiles so that the information can be considered for N3IWF or TNGF reselection. The N3IWF and TNGF can provide their profiles to the NRF by invoking the Nnrf_NFManagement_NFRegister service or the N3IWF and TNGF profiles can be provided to the NRF via OAM.

# * * * * End of change * * * *