3GPP TSG-WG SA2 Meeting #171	S2-2508701

13th – 17th October 2025, Wuhan, China	  (is revision of XXXX)

Source:	Lenovo

Title:	KI#1 Interim Agreements on UE reader selection (AF requesting UE case)

Document for:	Approval

Agenda Item: 	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: This paper proposes the interim agreements principles for Key Issue 1 UE reader selection, especially on the AF requesting specific UE reader case.

1	Introduction

This paper proposes the interim agreements for the key issue 1, and mainly covers the aspects of protocol stack of the T2, and the UE reader selection when 3rd party AF is requesting specific UE reader. The details of the solution that the agreement is made on can refer to the contribution (S2-2508699).

1.1 Protocol Stack for Topology 2 RRC

Figure 1.1 Protocol Stack for T2 RRC scheme (Indirect Connectivity)

The protocol stack for the RRC scheme of the topology 2 indirect connectivity, is proposed and shown above in the Figure 1.1. Similar to the design of T1 indirect connectivity as described in clause 4.6.2.3 of TS 23.369, for T2 indirect connectivity, we are proposing that the NG-RAN and AIOTF also communicate with each other via the NGAP AIOT information/container. It is assumed that AMF transparently relays the NGAP AIOT information between the NG-RAN and the AIOTF and it is included in the NGAP messages and the AIOT3 messages.

Proposal 1: For Topology 2, the AIOTF and NG-RAN communicate with each other via the NGAP AIOT information/container, and it is transparent to the AMF.

1.2 UE reader selection (AF requesting specific UE reader scenario)

1.2.1 UE reader selection use case

For UE reader selection, from SA2 point of view, there can be 3 use cases:

Use case 1: AF requesting specific UE reader (s) for the AIoT service operation.

This is mainly for the hand-held UE reader case, where AF has already indicated the UE (s) it wants to use. Therefore, there is no need for the AIOTF or the NG-RAN to perform any further selection of the UE reader if the AF request has been authorized.

Use case 2: Fixed UE reader scenario

This is for the case when a fixed UE reader is deployed, e.g., in the warehouse or the supermarket. The fixed UE reader is quite stable physically and is assumed to have a fixed connection with the NG-RAN. Therefore, a similar pre-configuration mechanism specified in the Release 19 for the NG-RAN and RAN reader information at the AIOTF, can be reused for the fixed UE reader. AIOTF can select the fixed UE reader based on the OAM configuration of the UE reader information locally.

Use case 3: Area based UE reader selection

Since the UE reader can be moving, and the radio conditions between the UE reader and the serving NG-RAN is time varying and only available at the NG-RAN side, a reasonable approach would be that the NG-RAN performs the final selection of the UE reader. Under this use case, the AIOTF can forward the requested area received from the AF via NEF (in the form of AIOT area) to the NG-RAN, and NG-RAN will further select the UE reader based on the requested area, which is under the remit of RAN 3.

This paper will mainly discuss the use case 1 for AF requesting specific UE reader (s).

1.2.2 AF authorization enhancement

For the Release 19 study when NG-RAN is serving as the AIoT reader, the AF will not indicate or request a specific RAN reader to perform the AIoT service operation (e.g., inventory, command), and the authorization for the AF request is based on the AF authorization data stored in the ADM/UDR, which includes the AF ID, allowed area, allowed service operation and allowed target AIoT Devices.

However, for T2, when it comes to the case that a 3rd party AF specifically requests UE reader(s) for an AIoT service operation, the core network (i.e., AIOTF) needs to firstly check whether this AF is authorized to use this UE reader(s) to perform the AIoT service operation. Here, the UE reader can have 2 different options, e.g., the UE reader belongs to/is owned by the operator, or the UE reader belongs to the 3rd party (e.g., Lenovo). For both options, it is necessary for the AIOTF to firstly check whether this AF can ask to use this UE reader before assigning the task, and if not authorized, the AF request shall be rejected, or the AIOTF may recommend other available UE readers to the 3rd party AF.

Proposal 2: The AF authorization data needs to be enhanced with allowed UE reader information.

1.2.3 UE reader ID and information forwarding

As shown in the protocol stack, the NGAP AIOT information between the AIOTF and NG-RAN is transparent to the AMF. Under this assumption, which UE ID should AIOTF use to communicate with the NG-RAN will become a problem. For example, when AF requests a specific UE reader in the form of GPSI, after AIOTF or NEF resolves it into SUPI (via UDM), AIOTF cannot directly use SUPI as the UE reader ID and send it to the NG-RAN since AMF in the between cannot read and let alone translate (e.g., translate SUPI into the ID that NG-RAN knows), and NG-RAN certainly cannot understand the SUPI due to security concern.

On the other hand, if the UE ID is outside the AIOT NGAP information and readable to the AMF, the relay AMF shown in the Figure 1.1 between the AIOTF and NG-RAN may not always be the AMF that currently serves the UE (i.e., serving AMF). Following the design in the T1 as described in clause 5.3.4 of TS 23.369, the AMF between the NG-RAN and AIOTF is selected by the AIOTF, based on the local configuration that corresponds to the NG-RAN node(s). Under this case, the middle/relay AMF may not be able to translate the UE ID (SUPI) into the ID that RAN can understand (e.g., NGAP UE ID, 5G-TMSI), and it will also cause the misalignment of the UE ID between NG-RAN and AIOTF.

To solve this problem, the UE reader ID needs to be firstly aligned at both the NG-RAN and AIOTF, understood and used by both sides to exchange the messages related to UE reader. What’s more, in order to forward the message towards the UE from AIOTF, the current serving NG-RAN of the UE reader, needs to be firstly discovered, especially considering the UE mobility. Therefore, it is proposed with the following 2 options:

Option 1: During UE reader registration procedure, the AMF actively sends the authorized UE reader information to the AIOTF, that contains the UE reader ID, the serving NG-RAN information (e.g., NG-RAN ID) and reader allowed/authorization indication.

The NGAP UE ID is reused as the UE reader ID and sent to the AIOTF, which is associated with SUPI, and stored at AIOTF. The reason to choose NGAP UE ID (AMF NGAP UE ID or gNB NGAP UE ID) is that this UE ID has already been supported and transmitted at both the NGAP interface and XN interface (e.g., during UE Handover procedure among source and target NG-RAN), and the NG-RAN can understand it naturally. Compared to the new allocated UE reader ID option (either by AMF or by AIOTF), reusing NGAP UE ID will have less standard impact. When the NGAP UE ID is sent to the NG-RAN, as long as it is associated with the UE reader authorization indication, the NG-RAN can understand that this represents the UE reader. What’s more, the relay AMF does not need to understand the NGAP UE ID used between the AIOTF and the NG-RAN since the NGAP UE ID as the UE reader ID will be contained in the NGAP AIOT information.

NOTE: The use of NGAP UE ID as UE reader ID is not limited to AF requesting UE reader case.

Based on this, later when a UE reader is determined (either by AIOTF, or indicated by AF), instead of using SUPI, the AIOTF will check the stored information for UE reader, use the NGAP UE ID as the UE reader ID and send it to the serving NG-RAN of the UE, together with the service request.

During the UE HO procedure, the current serving AMF always updates the latest UE reader related information to the AIOTF.

Option 2: AIOTF obtains the serving AMF of the UE from UDM using UE ID and sends the UE reader related information retrieval request to the serving AMF, when a UE reader is determined (either by AIOTF, or indicated by AF). The serving AMF will send UE reader ID (NGAP UE ID) associated with SUPI, the serving NG-RAN information (e.g., NG-RAN ID) to the AIOTF.

Based on the options listed above, the AIOTF can send the AIOT service request to the UE via the serving NG-RAN, that contains NGAP AIoT information (including UE reader ID (NGAP UE ID)), NG-RAN ID and AIOTF ID.

Proposal 3: NGAP UE ID is reused as UE reader ID and aligned to both AIOTF and NG-RAN by AMF.

Proposal 4: AIOTF receives the UE reader related information that contains the serving NG-RAN information, and UE reader ID, from the serving AMF of the UE, and is associated with SUPI.

The below Figure 1.2 gives the procedure for AF requesting the specific UE reader (s). Some of the key principles are highlighted:

- AIOTF authorizes the AIoT service request from AF that further includes authorizing the use of specific UE reader (s) for AIoT service operation.

- AIOTF obtains the UE reader related information, that includes the UE reader ID, the serving NG-RAN and serving AMF information of the UE, either via the information received from serving AMF after UE reader registration procedure and handover procedure, or via AIOTF actively retrieving the serving AMF information (from UDM using UE ID) and serving NG-RAN information and UE reader ID from serving AMF. The UE reader related information will be used by the AIOTF to forward the AIoT service operation towards the UE reader via NG-RAN correctly.

- The NGAP UE ID is used as the UE reader ID. AMF sends the NGAP UE ID as the UE reader ID, to the AIOTF (which is associated with SUPI) and it is used between AIOTF and NG-RAN for identifying the UE reader that is being selected/used.

Figure 1.2 Procedure of AF requesting specific UE reader (s) for AIOT Service Operation

2. Text Proposal

It is proposed to capture the following changes to TR 23.700-30.

# * * * * First change all new texts* * * *

7	Interim agreements

7.1	Agreed Principles

    - Agreed Principles for KI#1

7.1.1.X Protocol Stack for Topology 2 RRC scheme

Figure 7.x RRC protocol stack for Topology 2 (Indirect Connectivity)

The protocol stack for the RRC scheme of the topology 2 indirect connectivity, is shown above in the Figure 7.x. Messages between the UE reader and the AIOTF are delivered using RRC between UE and NG-RAN, and NGAP between NG-RAN and AMF, and using an SBI interface between AMF and AIOTF.

The NGAP AIOT information: It is the subset of NGAP information that the AMF transparently relays between the NG-RAN and the AIOTF and is included in the NGAP messages and the AIOT3 messages. For T2, it further contains the UE reader ID. The rest of the protocol stack details can be referred to the clause 4.6.2.3 of TS 23.369.

7.1.1.Y UE reader selection

For the use case when AF requesting specific UE reader (s) to perform the AIoT service operation:

- AF sends the UE ID (e.g., GPSI) to the core network as the requested reader information, together with the AIoT service operation.

- AIOTF authorizes the AF request as described in clause 5.6 of TS 23.369, that further includes the allowed UE reader information (e.g., UE ID in the form of GPSI or SUPI) as AF authorization data, meaning whether the 3rd party AF is allowed to assign the AIoT service operation to this specific UE reader (s). If not authorized, the AIOTF may also send the recommended UE reader information to the requested AF, which is based on its own determination.

- NGAP UE ID is used as the UE reader ID and aligned to both the NG-RAN and AIOTF by AMF, together with UE reader authorization indication.

- The AIOTF determines the UE reader related information that contains the UE reader ID (NGAP UE ID) associated with SUPI, the serving NG-RAN information (e.g., NG-RAN ID) of the UE reader, and UE reader authorization indication.

- The UE reader related information at AIOTF, can either be provided and updated by the serving AMF after UE reader registration procedure and UE Handover procedure, or by AIOTF actively requesting the serving AMF using UE ID (SUPI).

- AIOTF determines the serving AMF information (e.g., AMF ID) of the UE reader from the UDM using UE ID (SUPI) as the key.

- AIOTF sends the AIoT service operation that contains the UE reader ID (NGAP UE ID) to the serving NG-RAN of the UE via AMF, together with NG-RAN ID, correlation ID and AIOTF ID.

- NG-RAN sends the AIOTF service operation results to the AIOTF via AMF, that contains the UE reader ID (NGAP UE ID), NG-RAN ID, AIOTF ID and correlation ID.

# * * * * End of changes * * * *