SA WG2 Meeting #171	S2-2508720

13 – 17 Oct, 2025, Wuhan, China

Source:	Samsung

Title:	KI#1 Interim Agreements

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC/Rel-20

Abstract of the contribution: This contribution proposes Interim Agreements for KI#1.

1. Discussion

Rel-20 Architecture and precoedures for AmbientIoT Topology 2 can be derived based on Rel-19 partial conclusions and Rel-20 Architecture assumptions and KI agreements.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 23.700-30.

* * * Start of first Change (all new text) * * * *

7.1.Y	Agreed Principles for KI#1

The following principles apply for Topology 2:

-	The architecture and procedures for Ambient IoT defined in TS 23.369 [3] are used as a baseline for topology 2 support, with the difference that a UE Reader performs AIoT Service Operation requests from an AIOTF instead of a RAN Reader.

-	Subscription Aspects:

-	The UE subscription in the UDM will be extended with UE Reader subscription information, which consists of the following:

-	information indicating whether the UE is allowed to operate as a UE Reader.

-	Additonal subscription information, e.g. validity information, for the UE Reader is also needed for indicating whether the UE is allowed to operate as a UE Reader.

-	UE Reader subscription information is available to AMF and AIOTF.

-	If AMF receives, as part of the subscription information, the indication that the UE is authorized to operate as a UE Reader, then the AMF informs NG-RAN that the UE is authorized to operate as a UE Reader.

-	Radio resource management for UE Reader operation:

-	If the gNB has received the indication that a UE is authorized to operate as a UE Reader, then the gNB may assign radio resources to the UE for UE Reader operation.

-	The UE Reader is assumed to request radio resources for Reader operation (e.g., taking into account assistance information received from the AIOTF) from the gNB.

NOTE 1:	Assistance information is specified in TS 23.369 [3].

NOTE 2:	The details of UE Reader radio resource management will be defined by RAN WG2.

In the indirect connectivity architecture messages between the UE Reader and the AIOTF are delivered using RRC between UE and RRC and NGAP between gNB and AMF, and using an SBI interface between AMF and AIOTF. The related protocol stack is shown in Figure 7.1.y-1.

Figure 7.1.y-1: Protocol Stack for the RRC option

The following interim conclusion are agreed for the UE Reader authorization in Topology 2:

-	The subscription data of UE Reader includes the indication that whether the UE is allowed to act as an AIoT Reader.

-	The AMF provides the UE Reader authorization indication to the NG-RAN serving the UE Reader.

The following interim conclusion are agreed for the UE Reader selection in Topology 2:

-	UE Reader selection by AIOTF:

-	The AIOTF selects the UE readers (e.g. candidate or final UE readers) and provides the selected UE Reader list to the RAN.

-	If a single UE Reader ID is provided by the AF via the NEF for the operation, then that is used as the selected Reader.

-	If reader selection information or UE Reader ID is not provided, then how the AIOTF determines which readers to use is based on implementation. The AIOTF may be e.g. preconfigured with which readers to use, or take the requested target AIoT Devices last known location into account, etc.

-	Information to be used for UE AIoT reader selection:

-	At least one of the UE reader ID or the target area information.

* * * End of Change * * * *