SA WG2 Meeting #171	S2-2508978

13 - 17 Oct, 2025, Wuhan, CH

Source:	Nokia, T-Mobile USA, Deutsche Telekom, ZTE, IIT Bombay, AT&T

Title:	[WT#1.1, all topics] Work task #1.1 Support of 6G Control signalling

Document for:	Approval

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: Scope of Work task 1.1.

# 1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

Work task 1.1 scope as agreed in 6G System Study item S2-2506096:

.1.	Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

# Options considered for Work tasks:

OPTION 1:

- Work task 1.1 scope as in the agreed SID with just the removal of “or” from top level bullet.

OPTION 2:

- Replacing connectivity services and beyond connectivity services with NAS functionality and operator services.

- WT#1.1 Study the support for control signalling for 6G System for NAS functionalities and operator services (offered by 3GPP System), including the following:

- a.	Whether and how to enable the introduction of a new non-access stratum functionality with minimal or no impact to other non-access stratum functionalities.

- b. Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

- c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

- NOTE 1: 	It is assumed that this WT covers impacts to 6G System procedures for NAS functionalities such as mobility management, session management and UE NAS identifiers.

OPTION 3:

- WT#1.1 Study the support of control signalling for 6G System including the following:

- a.	Whether and how to enable the introduction of a new non-access stratum functionality with minimal or no impact to other non-access stratum functionalities.

- b. Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

- c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

- NOTE 1: 	It is assumed that this WT covers impacts to 6G System procedures for NAS functionalities such as mobility management, session management and UE NAS identifiers.

Based on majority preference, Option 2 is proposed as the way forward. Some companies also prefer to go with Option 3 or Option 1.

Options considered for Key issues:

- A single key issue for WT1.1

- Two key issues for WT1.1 – one key issue for WT1.1a, WT1.1b and another key issue for WT1.1c.

The paper reflects both options – single key issue and two key issues as there are still diverse views that need to be consolidated during the meeting.

**** First Change ****

# Annex A.X. WT#1.1

WT#1.1 Study the support for control signalling for 6G System for NAS functionalities and operator services (offered by 3GPP System), including the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality with minimal or no impact to other non-access stratum functionalities.

b. Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

NOTE 1: 	It is assumed that this WT covers impacts to 6G System procedures for NAS functionalities such as mobility management, session management and UE NAS identifiers.

**** Second Change (all text new – [Option 1]) ****

# 5.X. Key Issue #Z: Support for non-access stratum system procedures and UE-CN Control Signaling for Operator Services offered by 3GPP System

This Key issue studies the following aspects:

1)	Whether and how to enable the introduction of a new non-access stratum functionality with minimal or no impact to other non-access stratum functionalities.

NOTE: Coordination with CT1 could be required

2)	NAS termination, transport of services via NAS and NAS messages routing

3)	Whether and how to identify a minimum, common set of NAS functionalities

- Study whether and how to define generic mechanism including architecture, procedures and UE-core network interactions to support operator services offered by 3GPP system

  - Study how to support transfer and routing of UE-CN signaling (i.e. service-specific messages).

4)	Study system procedures for non-access stratum functionality, UE-CN signaling procedures, including impact to RAN-CN interface and 6G CN interfaces related to these system procedures.

NOTE 1: Any impact on the UE and RAN interaction aspects will be coordinated with RAN3 and the decision for RAN-Core interface will be aligned with RAN3.

NOTE 2: Any impact on the RAN and core network interaction aspects will be coordinated with RAN2.

NOTE 3: Interworking and migration aspects, especially inter-PLMN aspects (e.g. roaming), where applicable, are covered by WT#2.

NOTE 4: Potential security and privacy impacts require coordination with SA3.

NOTE 5: 	It is assumed that this key issue covers impacts to 6G System procedures for functionalities such as mobility management, session management and UE NAS identifiers.

NOTE 6: Coordination with KIs related to WT1.2, WT2, WT#3, WT#4, WT#5, WT#6 is required.

NOTE 7: The key issue includes studying mechanism for transporting signaling for operator services. It is not precluded that signaling for operator services may use transport over NAS.

**** Second Change (all text new – [Option 2]) ****

# 5.X. Key Issue #X: Support for non-access stratum system procedures

This Key issue studies the following aspects:

1)	Whether and how to enable the introduction of a new non-access stratum functionality with minimal or no impact to other non-access stratum functionalities.

NOTE: Coordination with CT1 could be required

2)	NAS termination, transport of services via NAS and NAS messages routing

3)	Whether and how to identify a minimum, common set of NAS functionalities

4)	Study system procedures for non-access stratum functionality, UE-CN signaling procedures, including impact to RAN-CN interface and 6G CN interfaces related to these system procedures.

NOTE 1: Any impact on the UE and RAN interaction aspects will be coordinated with RAN3 and the decision for RAN-Core interface will be aligned with RAN3.

NOTE 2: Any impact on the RAN and core network interaction aspects will be coordinated with RAN2.

NOTE 3: Interworking and migration aspects, especially inter-PLMN aspects (e.g. roaming), where applicable, are covered by WT#2.

NOTE 4: Potential security and privacy impacts require coordination with SA3.

NOTE 5: 	It is assumed that this key issue covers impacts to 6G System procedures for functionalities such as mobility management, session management and UE NAS identifiers.

NOTE 6: Coordination with KIs related to WT1.2, WT2, WT#3, WT#4, WT#5, WT#6 is required.

# 5.Y. Key Issue #Y: Support of UE-CN Control Signaling for Operator Services offered by 3GPP System

This Key Issue focuses on studying the support for UE-Core Network control signalling for operator services offered by 3GPP System, including the following:

- Study whether and how to define generic mechanism including architecture, procedures and UE-core network interactions to support operator services offered by 3GPP system

  - Study system procedures to support generic mechanism for supporting operator services, UE-CN signaling procedures, including impact to RAN-CN interface and 6G CN interfaces related to these system procedures.

  - Study how to support transfer and routing of UE-CN signaling (i.e. service-specific messages).

NOTE 1: The key issue includes studying mechanism for transporting signaling for operator services. It is not precluded that signaling for operator services may use transport over NAS.

NOTE 2: Any impact on the UE and RAN interaction aspects will be coordinated with RAN3 and the decision for RAN-Core interface will be aligned with RAN3.

NOTE 3: Any impact on the RAN and core network interaction aspects will be coordinated with RAN2.

NOTE 4: Potential impact on Interworking to be coordinated with KIs related to WT#2.

NOTE 5: Coordination with KIs related to WT1.2, WT2, WT#3, WT#4, WT#5, WT#6 is required.

NOTE 6: Potential security and privacy impact to be coordinated by SA3.

**** End of Changes ****