3GPP TSG-WG SA2 Meeting #171	S2-2509039

Wuhan, CN, 13th Oct – 17th Oct, 2025	(revision of S2-2507290)

Source:	Huawei, HiSilicon

Title:	[Sol2 & 6 update] IMS dedicated bearer pre-establishment

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract: Enabling IMS dedicated bearer is pre-established on demand.

# 1. Introduction/Discussion

This updates solutions for Key Issue #1: Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC.

TR 23.300-19 Solution #2 and Solution #6 proposed EPS enhancement to support dedicated bearer pre-establishment for the IMS voice service over NB-IoT (GEO).

However, if the NB-IoT UE does not have IMS voice call service for a long time after IMS Registration, the pre-established dedicated EPS bearer will cause 1) waste of radio resource. The UE may not have IMS voice call before entering into RRC_IDLE, which causes useless of pre-established dedicated bearer. 2) the NB-IoT UE cannot perform any other services due to the bearer occupation.

In TS 24.301 and TS 36.331, it describes that the UE sets RRC establishment cause to “MO Voice Call” if the UE is establishing the RRC connection for mobile originating MMTEL voice.

# 2. Text Proposal

It is proposed to capture the following changes to 3GPP TR 23.700-19.

# * * * * First change * * * *

## 6.2	Solution #2: Supporting establishing EPS bearers for IMS voice via user plane with IP type PDN connection

### 6.2.0	High-level solution Principles

This solution aims to resolve KI#1 by supporting the IMS voice call using the user plane of NB-IoT (GEO) satellite access, the following principles are followed in this solution:

-	UE camps on an eNB that supports NB-IoT (GEO) satellite access

-	The UE indicates its support of IMS voice over NB-IoT in attach/TAU request

-	The MME indicates IMS voice over PS session is supported to UE.

-	UE requests to establish an IP type PDN connection with IMS APN via user plane.

-	IMS signalling is transmitted via the default EPS bearer.

-	IMS voice data is transmitted via a dedicated EPS bearer.

-	The dedicated EPS bearer may be established together with the default EPS bearer.

### 6.2.1	Description

This solution aims to resolve KI#1 by supporting the IMS voice call over NB-IoT (GEO) via one IMS PDN connection, with default and dedicated bearers as existing IMS voice support over WB-E-UTRAN.

Figure 6.2.2-1: IMS signalling via default EPS bearer and voice data via a dedicated EPS bearer

The PDN connection is based on IP type, therefore, the protocol stack could be as follows:

Figure 6.2.2-3: Protocol stack for IMS signalling

Figure 6.2.2-4: Protocol stack for IMS voice data

### 6.2.2	Procedures

#### 6.2.2.1	Attach and IMS PDN connection establishment

The following figure depicts the attach and PDN connection establishment procedure for IMS session over the UP-based NB-IoT (GEO) deployment.

Figure 6.2.2.1-1: Attach and IMS PDN connection establishment Procedure

1.	UE camps on a GEO eNB cell.

2.		The UE sends Attach Request to the MME, where the eNB indicates RAT type as NB-IoT (GEO) satellite access. During attach, the UE indicates its support of IMS voice over NB-IoT, and optionally an ESM container is included where the PDN type is set as IP.

3.	Same as step 3-6 as specified in clause 5.3.2.1 of TS 23.401 [5].

The UE may send the IMS APN to MME in this step. It is up to the operator's policy whether the well-known IMS APN is used as default APN in the attach procedure. Using well-known IMS APN during UE attach can reduce the time for UE to be ready for IMS voice call and reduce the needed resources (e.g. number of DRBs), but consequently there is risk that UE may not get data service or even not authorized to attach to the network.

4.	The MME verifies with HSS if the subscription data allows the UE to use IMS voice over NB-IoT (GEO), and indicates IMS voice over PS session is supported to the UE in attach accept.

5.	If an ESM container was not included in the Attach Request, step 5 is skipped. Otherwise, step 12-16 as specified in clause 5.3.2.1 of TS 23.401 [5] are executed to establish the default EPS bearer for the IMS signalling, the P-CSCF address is provided to the UE when activating the default bearer.

One optimization to reduce the call setup time is to pre-establish the dedicated bearer prior to the IMS voice call setup, i.e. the MME may indicate the APN, Access Info, UE capability and subscription of voice over GEO to the P-GW in order to allow the P-GW to determine to set up dedicated EPS bearer for the IMS voice call, and the P-GW establishes a dedicated EPS bearer for IMS media plane in association with the establishment of the default bearer. Depending on how the packet filter for the dedicated bearer is handled, there are 3 alternatives:

-	Alt#1: the P-GW does not provide TFT containing uplink packet filters to the UE for the dedicated bearer and later during call setup procedure, the UE filters/identifies the uplink voice packets based on the port number negotiated during SIP call setup procedure. In this case no RRC messages are used between UE and eNB.

-	Alt#2: the P-GW creates uplink packet filter (see clause 15.3.3.4 of TS 23.060 [12] for examples) effectively stopping any packet flow in the uplink direction in this pre-established dedicated bearer for voice media, and during call setup procedure, the P-GW initiates a dedicated bearer modification procedure (see clause 5.4.3 of TS 23.401 [5]) for updating the TFT. In this case two RRC messages (direct transfer) are used between UE and eNB.

-	Alt#3: The MME selects a P-GW with pre-configured TFT information. Both the uplink and downlink packet filters of the TFT for dedicated EPS bearer specific for GEO is determined based on operator policy or implementation (e.g. (list of) AGW IP address, or range of destination IP address and a range of destination port numbers) that will be provided to the UE during the dedicated bearer pre-establishment. In this case no RRC messages are used between UE and eNB during the IMS call setup.

Editors' note:	The above EPC optimization requires E2E evaluation together with the rest of the system. For example, the IMS SIP signalling transmission currently contributes most of the call setup time (CST), given the SIP signalling message size is larger than RRC message, whether saving of RRC messages would make comparable impact to CST is to be assessed.

Editor's note:	Whether additional information UE capability and subscription of voice over GEO need to be sent to P-GW is FFS.

6-8.	Same as step 17-22 as specified in clause 5.3.2.1 of TS 23.401 [5] if only default EPS bearer is to be established.

If the P-GW decides to establish a dedicated EPS bearer for the IMS PDN connection in step 5, a Dedicated Bearer Setup Request with the corresponding the EPS Bearer QoS, the UE-AMBR etc will also be included in the Initial Context Setup Request, the radio bearer establishment of the default and dedicated bearer for IMS is performed in the same RRC message.

If attach without PDN connection is enabled and used between UE and network (i.e. an ESM container is not included in Attach Request), step 5 is skipped and step 6/7/8 is replaced with Downlink NAS transport/RRC direct transfer, and the following procedures proceed:

9.	The UE sends a PDN Connectivity Request to MME to indicate the requested PDN type is IP, and the APN is IMS.

10.	Same as step 5.

11-13.	Same as step 7-10 as specified in clause 5.10.2 of TS 23.401 [5].

If the PDN GW decides to establish a dedicated EPS bearer for the IMS PDN connection in step 10, a Dedicated Bearer Setup Request with the corresponding the EPS Bearer QoS, the UE-AMBR etc will also be included in the S1-AP Bearer Setup Request, the radio bearer establishment of the default and dedicated bearer for IMS is performed in the same RRC message.

The default EPS bearer for IMS signalling is established, and optional the dedicated EPS bearer for IMS voice is also established.

#### 6.2.2.2	MO call

The following figure depicts the mobile originating procedure for IMS session establishment over the UP-based NB-IoT (GEO) deployment.

Figure 6.2.2.2-1: MO call Procedure

1.	The MO UE camps on the eNB that supports NB-IoT (GEO) satellite access and performs attach procedure and establishes the PDN connection for IMS service. The detailed procedure can refer to clause 6.2.2.1. Optionally, the dedicated EPS bearer for IMS voice can be established.

2.	The MO UE performs IMS registration procedure as specified in clause 5.2.2 of TS 23.228 [6], which is via the default EPS bearer of the IMS PDN connection. P-CSCF can obtain RAT type of the IP-CAN by subscribing to the IP-CAN Type Change with PCRF during the SIP registration, as described in Annex B, clause B.1a of TS 29.213 [11].

After steps 1 and 2 the MO UE may return to RRC_IDLE.

3-4.	If there is no available dedicated EPS bearer in step 1 and the MO UE has a pending IMS voice call, if the MO UE is in RRC_IDLE, the MO UE may trigger to pre-establish the dedicated EPS bearer using the RRC establishment cause ("MO Voice Call") during the process of entering RRC_CONNECTED from RRC_IDLE. Otherwise, if the MO UE is in RRC_CONNECTED, the MO UE may trigger to pre-establish the dedicated EPS bearer using the Bearer Resource Allocation procedure as described in clause 6.5.3 of TS 24.301 [17].

The MO UE initiates a voice call using SIP INVITE message with an SDP offer towards the P-CSCF, which is over the default EPS bearer of the IMS PDN connection. The P-CSCF forwards the SIP INVITE message towards IMS core and receives SIP 183 response with an SDP answer to the SIP INVITE message as specified in clause 5.6 of TS 23.228 [6].

NOTE:	Any IMS optimizations, including compression or substitution mechanisms, will be addressed in Key Issue#2 study.

5-6.	The P-CSCF sends diameter AAR according to the SDP offer and the SDP answer received to the PCRF as specified in clause 8.1 of TS 29.211 [10]. The PCRF sends a PCC decision provision (QoS policy) message to the PDN GW as specified in step 1 of clause 5.4.1 in TS 23.401 [5].

7.	If a dedicated EPS bearer is pre-established in step 1 or step 3, the QoS requirement for voice data is configured for the dedicated EPS bearer. The packet filter will be updated based on the alternatives listed in clause 6.2.2.1 step 5.

If the dedicated EPS bearer is not established in step 1 or step 3, the PDN GW triggers the dedicated EPS bearer creation procedure as specified in step 2-11 in clause 5.4.1 of TS 23.401 [5], where 3 RRC messages (reconfiguration/complete/direct transfer) are used between UE and eNB.

Editor's note:	Whether and how to resolve the conflict with maximum 2-DRB restriction if concurrently supporting data and IMS services for NB-IoT UEs is FFS and needs coordination with RAN WGs.

Editor's note:	Which QoS parameters are applicable for the dedicated EPS bearer is FFS and need coordination with RAN WGs.

8-9.	The PDN GW responds to the PCRF as specified in step 12 in clause 5.4.1 of TS 23.401 [5]. The PCRF send diameter AAA to the P-CSCF as specified in clause 8.1 of TS 29.211 [10].

10.	The P-CSCF sends the SIP 183 response with the SDP answer to the MO UE, which is over the default EPS bearer of the PDN connection.

For Alt#1, the UE updates the packet filter locally based on the SDP negotiation result.

11.	The procedure is continued and the IMS session is established.

The IMS voice data are transmitted over the dedicated EPS bearer.

#### 6.2.2.3	MT call

The following figure depicts the mobile terminating procedure for IMS session establishment over the UP-based NB-IoT (GEO) deployment.

Figure 6.2.2.3-1: MT call Procedure

The same procedures as in clause 6.2.2.2, apart from the following steps:

3-4.	The UE-1 receives an incoming voice call using SIP INVITE message with an SDP offer, which is over the default EPS bearer of the PDN connection. The UE sends SIP 183 response with an SDP answer to the SIP INVITE message as specified in clause 5.7 of TS 23.228 [6], which also is over the default EPS bearer of the PDN connection.

10.	The P-CSCF sends the SIP 183 response with the SDP answer to the IMS core.

The IMS voice data are transmitted over the dedicated EPS bearer.

### 6.2.3	Impacts on Services, Entities and Interfaces

PGW:

-	Support determination of dedicated EPS bearer pre-establishment based on IMS APN, and NB-IoT (GEO) satellite access;

-	Support pre-establishing dedicated EPS bearer during Create Session, preconfiguring the packet filter or updating the packet filter locally during call set up; or

-	Support creating the dedicated EPS bearer for voice for UP-based NB-IoT (GEO).

MME:

-	Support verification of IMS voice call subscription considering NB-IoT (GEO) satellite access, UE preferences;

-	Support indicating IMS Voice over PS session supported to UE;

-	Support the dedicated EPS bearer pre-establishment based on the RRC establishment cause ("MO Voice Call").

UE:

-	Support creation of the dedicated EPS bearer for voice, and optionally together with during the Attach or PDN connection establishment procedure, or Service Request procedure, or Bearer Resource Allocation procedure;

-	Indicating to pre-establish the dedicated EPS bearer for voice using the RRC establishment cause ("MO Voice Call") during Service Request procedure;

-	May support update packet filter locally based on the SDP negotiation result;

-	Indicating support of IMS voice over NB-IoT in Attach/TAU request.

RAN:

-	Support dedicated bearer pre-establishment.

HSS:

-	Subscription data indicating whether IMS voice over NB-IoT (GEO) is allowed.

# * * * * Second change * * * *

## 6.6	Solution #6: Support establishing dedicated bearer for QoS differentiation for CP based IMS voice service over NB-IoT (GEO)

### 6.6.0	High-level solution Principles

This solution addresses KI#1 to support IMS voice call over NB-IoT via GEO connecting to EPC.

The following principles are applied to this solution:

-	Procedure on Data Transport in Control Plane CIoT EPS Optimisation is used as baseline.

-	EPS enhancement to support dedicated bearer for the IMS voice service over NB-IoT(GEO).

-	IMS signalling is transmitted via the default bearer, IMS voice data is transmitted via the dedicated bearer.

-	Dedicated bearer can be pre-established during IMS PDN connection establishment to reduce delay for the IMS voice service over NB-IoT(GEO).

### 6.6.1	Description

The TS 23.401 [5] specifies dedicated bearer is not supported over NB-IoT. GBR bearer is not supported in the NB-IoT RAT. These restrictions may not enable NB-IoT to support the QoS requirement of an IMS voice service considering the IMS signalling and the IMS voice data transmission have different QoS requirements (i.e. current IMS voice transmission using GBR bearer with QCI = 1, while IMS signalling using non-GBR bearer with QCI = 5).

The solution proposes to remove those restrictions mentioned above to achieve IMS voice service over NB-IoT via GEO connecting to EPC. The solution is based on CP CIoT EPS Optimisation.

To enable the IMS voice transmission via NB-IoT control plane, an IMS PDN connection containing one default bearer and one dedicated bearer is proposed to transmit the IMS voice service, which can be depicted in figure 6.6.1-1.

Figure 6.6.1-1: IMS signalling via default EPS bearer and voice data via a dedicated EPS bearer

The IMS PDN connection based on IP type, therefore, the protocol stack could be as follows:

Figure 6.6.1-2: Protocol stack for IMS signalling

Figure 6.6.1-3: Protocol stack for IMS voice data

The following procedure is an example to create one default bearer and one dedicated bearer to support IMS voice service.

### 6.6.2	Procedures

Figure 6.6.2.1: IMS signalling and voice data delivery in CIoT EPS Optimisation over NB-IoT via GEO satellite

Attach:

0.	A UE, supporting IMS voice service over NB-IoT(GEO), camps on a NB-IoT(GEO) cell.

1.	The UE initiates the Attach procedure by the transmission of an Attach Request to the MME, including the Preferred Network Behaviour indicates "Control Plane CIoT EPS Optimisation is preferred", its supporting of IMS voice over NB-IoT indication.

2.	The MME process the Attach request from the UE. If the attach is successful, it sends an Attach Accept message to the UE. The MME indicates Control Plane CIoT EPS Optimisation is supported in the Supported Network Behaviour information. If IMS voice service over NB-IoT is supported when the UE is using the NB-IoT(GEO) and the UE is allowed to use IMS voice over NB-IoT(GEO) based on the UE subscription information , the IMS voice over NB-IoT Support indication is contained in the message as well.

If an ESM container is included in the attach request and IMS related APN is provided, an IMS PDN connection is established during the attachment. Otherwise, a separate PDN connection establishment procedure is performed to establish the IMS PDN connection.

PDN connection establishment for IMS voice service:

3.	Based on the IMS voice over NB-IoT(GEO) Support indication, the UE can initiate a PDN connection establishment for IMS service by using an APN related to the IMS service, and the request PDN type is set as IP.

4.	If the IMS PDN connection is allowed to establish, a default bearer for the IMS PDN connection is established.

Moreover, considering the dedicated bearer establishment triggered by IMS call setup will cause additional delays, the dedicated bearer can be pre-established in combination with the default bearer establishment. The NB-IoT (GEO) RAT type is sent to PGW via the create session request. The PGW can make use of this information to grant dedicated bearer to be established for the IMS APN PDN connection. In this case, two EPS bearers (one default bearer identified by EBI-1 and one dedicated bearer identified by EBI-2) are established for the IMS PDN connection over NB-IoT(GEO). To prevent the use of the dedicated bearer before IMS voice session is established, the TFT binding to the dedicated bearer shall be set to block all packets from the UL and DL.

Editor's note:	Whether new QCI is needed for supporting IMS voice service over NB-IoT NTN via GEO is FFS.

5.	The steps 7-12 in Annex F of TS 23.401 [5] are reused. If both the default bearer and dedicated bearer are activated, the EBI-1 and EBI-2 are provided to the UE.

After the IMS PDN connection is established, the UE can perform IMS registration as specified in clause 5.2.2 of TS 23.228 [6], which is via the default bearer of the IMS PDN connection. Then the UE can initiate IMS voice call setup.

After the above steps, the UE may return to RRC_IDLE.

IMS call setup:

6.	If there is no available dedicated bearer in step 2 or 4 and the UE has a pending IMS voice call, if the UE is in RRC_IDLE, the UE may trigger to pre-establish the dedicated bearer using the RRC establishment cause ("MO Voice Call") during the process of entering RRC_CONNECTED from RRC_IDLE. Otherwise, if the UE is in RRC_CONNECTED, the UE may trigger to pre-establish the dedicated bearer using the Bearer Resource Allocation procedure as described in clause 6.5.3 of TS 24.301 [17].

The UE can initiate IMS signalling negotiation with remote UE by using CP CIoT EPS Optimisation as defined in clause 5.3.4B.2. The NAS PDU carries the encrypted uplink IMS signalling and EBI-1 to the eNodeB of the NB-IoT(GEO). The UE maps to NAS PDU carrying the IMS signalling to the SRBx,

6a.	The eNodeB of the NB-IoT(GEO), based on configuration, may retrieve the EPS negotiated QoS profile from the MME. The QoS profile may be applied to prioritise between requests from different UEs.

7.	The NAS PDU sent in step 6 is relayed to the MME by the eNodeB using a S1-AP Initial UE message.

8.	The MME checks the integrity of the incoming NAS PDU and decrypts the data it contains.

9.	Based on the received EBI-1, the MME maps the uplink IMS signalling data to the default bearer and deliveries it to the P-GW.

10.	The Downlink IMS signalling is delivered via the default bearer, details can refer to steps 9-12 of Figure 5.3.4B.2-1 of TS 23.401 [5]. The eNB maps the DL traffic carrying the IMS signalling to the SRBx.

11.	The P-CSCF sends diameter AAR to the PCRF which provides the QoS requirement based on the SIP negotiation. The PCRF generates the PCC rule for the IMS voice data and provides it to the P-GW.

12.	If the dedicated bearer has already established during the IMS PDN connection establishment, the PCC rule installation may trigger P-GW to initiate dedicated bearer modification as described in clause 5.4.3 of TS 23.401 [5] to update the DL TFT. The UL TFT can be updated by the UE locally based on the IMS SDP negotiation result.

Otherwise, if there is no dedicated bearer for the IMS PDN connection, the PCC rule installation may trigger P-GW to initiate the dedicated bearer creation procedure as specified in step 2-11 in clause 5.4.1 of TS 23.401 [5].

13.	When the P-GW receives a downlink IMS voice data, it delivers the downlink data to the S-GW and further to MME via the dedicated bearer.

14.	The MME encrypts and integrity protects Downlink IMS voice data.

15.	The MME sends the encrypt downlink IMS voice data along with EBI-2 using a NAS PDU carried by a Downlink S1-AP message. The eNB maps the DL traffic of IMS voice packet to the SRBy.

16.	The NAS PDUwith IMS voice data is delivered to the UE via a Downlink RRC message.

17.	The uplink IMS voice data transmission via the dedicated bearer can refer to steps 16-19 of Figure 5.3.4B.3-1 of TS 23.401 [5]. The UE maps to UL traffic of IMS voice packet to the SRBy.

Editor's note:	Whether new SRB is needed for IMS signalling or IMS voice data transmission is FFS and depends on RAN implementation.

Editor's note:	Whether and how to provide QoS to the IMS voice service via SRB is FFS and depends on RAN implementation.

Editor's note:	Whether and how to enhance or optimize IMS registration, IMS call setup procedures to enable IMS voice over NB-IoT NTN via GEO is FFS.

### 6.6.3	Impacts on Services, Entities and Interfaces

UE:

-	Request for PDN connection establishment for IMS voice service when using NB-IoT NTN via GEO.

-	Support dedicated bearer when using NB-IoT NTN via GEO for IMS voice service.

-	Support to update UL packet filter locally based on the IMS SDP negotiation result.

-	Indicating to pre-establish the dedicated bearer for voice using the RRC establishment cause ("MO Voice Call") during Service Request procedure.

-	Indicating to pre-establish the dedicated bearer for voice, and optionally together with during the PDN connection establishment procedure, or Service Request procedure, or Bearer Resource Allocation procedure.

MME:

-	Support verification of IMS voice call subscription considering NB-IoT (GEO), UE and network capability for supporting IMS voice over NB-IoT(GEO).

-	Provide IMS voice service over NB-IoT support indication to UE.

-	Support for PDN connection establishment for IMS voice service when using NB-IoT NTN via GEO.

-	Support to pre-establish dedicated bearer (GBR bearer) for UE using IMS service over NB-IoT NTN via GEO and using the RRC establishment cause ("MO Voice Call").

S-GW/P-GW:

-	Support pre-creating the dedicated bearer (GBR bearer) for IMS voice service over NB-IoT (GEO).

eNB:

-	Support pre-creating new SRB for IMS signalling or IMS voice data (depends on RAN study).

HSS:

-	Subscription data indicating whether IMS voice over NB-IoT (GEO) is allowed.

# * * * * End of changes * * * *