3GPP TSG-WG SA2 Meeting #171	S2-2509136

Wuhan, CN, 13th – 17th Oct, 2025

Source:	CEWiT

Title:	Interim Conclusion proposal for KI#3

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: To provide a basis for future discussion, interim conclusions have been incorporated into Clause 7.

# 1.	Discussion

This contribution proposes the interim agreements for KI#3. The interim agreements are drafted based on the common points across all solutions proposed for KI#3 in 3GPP TR 23.700-14 V1.0.0.

The proposed solutions #1, #3, #6, #13, #14, #15, #16, #17, #35 and #36 address KI#3. The solutions are summarized as follows,

- Solution #1

The Gateway Function performs discovery and selection of Sensing Function. The Sensing Function performs discovery and selection of Sensing Entity.

- Solution #3

The NEF performs discovery and selection of Sensing Function (Control and Processing) based on the request from AF. The Sensing Function (Control and Processing) performs discovery and selection of Sensing Entity based on, e.g. the sensing service requirement, and sensing capability supported by sensing entity.

- Solution #6

The Gateway Sensing NF performs discovery and selection of a (distributed) Sensing NF for the target sensing service area. The (distributed) Sensing NF performs the discovery and selection of Sensing Entity(ies) based on the based on the Target Sensing Service Area and capabilities of Sensing Entity(ies). A Sensing Entity registers its supported capability at the Sensing NF. The Sensing NF may communicate with Gateway Sensing NF to gather the candidate list of Sensing Entity(ies) with their capabilities.

- Solution #13

The NEF performs discovery and selection of SeMF (based on required QoS and Target Sensing Area) either based on local configuration or by querying the NRF. The SeMF discovers and selects AMF(s) (based on Target Sensing Area) and requests/subscribes the Sensing Entity(ies) information in the Target Sensing Area from the AMF. Further, the SeMF selects Sensing Entity(ies) from the candidate list obtained from the AMF based on Sensing Entity capability(ies). The sensing entity capability can include supported sensing mode, and supported sensing area.

- Solution #14

The Sensing Function registers itself to NRF with its NF profile that contains NF Type, supported Sensing Service Area, supported Sensing modes and supported Sensing types. When sensing service is requested by a Trusted AF, then the trusted AF performs Sensing Function discovery and selection via NRF. When sensing service is requested by an Untrusted AF, then the NEF performs Sensing Function discovery and selection via NRF.

- Solution #15

The Sensing Function (SF) registers to NRF with its NF profile that may include supported area, sensing service type. The Sensing Entity registers/de-registers itself to SF along with its capabilities such as supported sensing area and sensing transmitter and/or receiver. The Sensing Entity(ies) discover(s) SF via local configuration or use NRF for the same. The NEF performs discovery and selection of a SF based on the sensing service consumer’s request. The SF from its available list of Sensing Entities, selects Sensing Entity(ies) based on the sensing service request.

- Solution #16

The Sensing Function (SF) performs Sensing Entity (SE) discovery and selection for a specific sensing service. The SF may, (i) select one or more Sensing Entities and the sensing mode (ii) select one primary sensing entity and delegates further decisions like sensing mode, sensing transmitter/receiver selection to primary sensing entity. The SF may obtain SE information based on SE association procedure, which provides sensing capability information, sensing entity ID and served sensing area.

- Solution #17

The SeMF registers to NRF with its NF profile. The AF/NEF performs SeMF discovery and (re-)selection. The SeMF (re-)selection is based on SeMF target service area, supported QoS, response time, supported AF ID(s), SeMF load and reporting mode. The SeMF discovers and selects AMF(s) and requests the Sensing Entity(ies) information in the Target Sensing Area from the AMF. Further, the SeMF (re-)selects Sensing Entity(ies) from the candidate list obtained from the AMF based on Sensing Entity(ies) capability(ies), requested Sensing Area and Sensing Entity location.

- Solution #35

Based on the target sensing area in the sensing service request, and the NG-RAN information configured by OAM, the Sensing Service Function (SSF) performs discovery and selection of sensing entity(ies) (i.e. a single NG-RAN node or multiple NG-RAN nodes).

- Solution #36

The Sensing RAN node obtains SF information by pre-configuration or from AMF. It sends a sensing association request to SF, containing the RAN node ID and sensing capability information. The sensing capability information may include the supported sensing mode, accuracy of sensing, confidence level, sensing resolution etc.

Based on the above provided summery for all the solutions, we propose the following interim agreements for KI#3:

- Sensing NF registers to NRF and whose NF profile contains NF Type, supported service area and service type etc.

- AF/NEF performs Sensing NF discovery via NRF or based on local configuration and sensing NF selection is based on the parameters like target sensing area, sensing service type, qos etc.

- Sensing NF discovers the Sensing Entity(ies) using AMF and selects SE(s) based on its sensing capabilities and other parameters, where sensing capabilities are provided by Sensing Entity(ies) prior to the network (AMF).

# 2.	Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

## 7.1	Agreed Principles

### 7.1.A	KI#3

The following are the interim agreements for KI#3:

- Sensing Management Function (SeMF) discovery and (re-)selection

The SeMF registers itself to NRF with its NF profile that contains,

- NF Type

- supported Sensing Service Area

- supported Sensing types

- supported Sensing QoS

AF/NEF performs SeMF discovery via NRF or based on local configuration.

The SeMF selection is performed based on key parameters such as,

- Target Sensing Area

- QoS parameters

- Sensing Service Type(s)

There could be other relevant parameters that can be considered for SeMF selection.

The AF/NEF may perform SeMF re-selection, when necessary, e.g. due to unavailability of SeMF to serve a specific area during a periodic sensing procedure.

- Sensing Entity discovery and (re-)selection

The sensing capability information of SE(s) may be provided to SeMF either directly or through the AMF.

NOTE:	The exact information provided by the SE is dependent on RAN WG3 study.

Upon receiving a sensing request, the SeMF discovers SE(s) by using AMF or directly.

The SeMF performs SE selection based on SE sensing capabilities, and other parameters. The SE capabilities can be,

- supported sensing area

- supported sensing mode

The SeMF may perform re-selection of Sensing Entity, when necessary, e.g. dynamic load conditions of gNB(s).

# * * * * End of Changes * * * *