SA WG2 Meeting #171	S2-2509137

13 – 17 Oct 2025, Wuhan, China	revision of S2-2508130

Source:	Huawei, HiSilicon, Vivo, China Mobile, Oppo, Boost Mobile Network, , NEC, , Futurewei, Deutsche Telekom, Oracle

Title:	[WT#3.1] AI for 6G architecture (baseline from SA2#170)

Document for:	Agreement

Agenda Item:	20.6.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This document summarises the proposals for WT#3 regarding the use of AI for 6G architecture. It is a resubmission from the final version of SA2#170.

**** First Change ****

# Annex A.3. WT#3 Scope

WT#3: Study how to support and enable use of AI in 6G (e.g. AI agent, framework).

NOTE 4: 	The term AI agent refers to the general concept of agents autonomously performing tasks on behalf of users, systems, and/or applications. As the SA1 work is still in progress, adapting the definition of AI agent from SA1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

WT#3.1.1:	Study how to introduce AI technologies (e.g. AI Agent/NF-based Distributed AI, Federated Learning, Reinforcement Learning, etc.) into the 6G CN to support customised services and achieve increased levels of performance, efficiency, scalability and resilience, while enabling multi-vendor interoperability, including:

1.	understanding requests or events,

2.	acting autonomously and controllably with acquiring contextual information, reasoning, explainability, self-learning, decision-making, generating results, interacting with other network functions, enabling closed-loop interactions,

3.	accessing trusted external capabilities and network AI capabilities,

4.	selecting AI native or non-AI native behaviour.

WT#3.1.2:	Study whether and how to:

5.	support AI-related functionalities (e.g. model training, model provisioning, performance monitoring) into 6G CN considering 5G CN AI-related functionalities as a starting point for discussion, which includes model interoperability, reliable and sustainable AI.

6.	coordinate between 5G CN AI and 6G CN AI, if needed, e.g. for purpose of consistent service across 5G and 6G.

NOTE:	For WT#3.1.2, aspects related to SA5 will be coordinated with SA5 during the study, aspects related to RAN will be coordinated with RAN during the study.

**** End of Changes ****