SA WG2 Meeting #171	S2-2509157

13 – 17 OCT, 2025, Wuhan, China	(Revision of S2-2508109)

Source:	[Nokia, AT&T, T-Mobile US, NEC, IIT Bombay, Lenovo, Qualcomm, Google, vivo, Interdigital, Deutsche Telecom, ZTE, MediaTek, Charter Communication], Ofinno

Title:	[WT#1.1, all topics] Work task #1.1 Support of 6G Control signaling

Document for:	Approval

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper proposes new KIs associated with WT#1.1 to support control signalling for 6G.

# 0. Background

At SA2 #170, SA2 discussed proposals on updating WTs for FS_6G_ARC. For SA2 #170, S2-2508109 related to 6G Control signaling as baseline documents and finally were postponed.

In this contribution, we propose further KIs for WT#1.1

# 1. Discussion

6G services will include both basic connectivity services (e.g., registration, mobility management, paging, session management) and beyond connectivity services (e.g. operator services over N6 such as AI/ML or sensing services). In 5GS, NAS functionality operates uniformly regardless of service type, with all NAS messages anchored in the AMF for mobility management, or routed via the AMF to SMF for session management. While this approach simplifies architecture, it creates processing bottlenecks in AMF and prevents service-specific optimization. For 6G, there is an architectural opportunity to introduce extended NAS functionality that can terminate in existing or new NF types, adapt NAS message paths based on service requirements, and minimize unnecessary AMF involvement to reduce control-plane congestion. From Rel-15 to Rel-20, the NAS architecture has remained as AMF-centric, with no mechanism to route NAS messages directly to service-specific NFs or to bypass AMF when its involvement is not required. This limitation restricts flexibility in supporting diverse service requirements and increases control-plane load.

Furthermore, current specifications also lack a framework to selectively define minimal NAS functionality for different types of UEs, e.g., while allowing reduced NAS capabilities for specific UE types (e.g., for IoT) or supporting extended NAS capabilities operator services. In the development of 5GS, supports for specific types of UEs and supports for specific verticals were introduced after basic features and service flows are defined. As a result, minimum NAS functionalities may be already too big for a later introduced UE types.

When considering NAS functionalities and UE-CN control signalling, issues arising from various deployment scenarios and different supports of different networks need to be studied as well.

Following areas needs to be studied for the WT#1.1:

- Support in heterogeneous deployment scenario: 
When a UE accesses its home network, operator services subscribed by the UE may be fully supported by the home network. Otherwise, having subscription for the operator services does not have meaning. However, when support for the operator services is different per different operators/networks, the UE may end up accessing a network in which the operator services are not supported.  
Basic operator services like SMS/Voice are assumed to be universally supported. So, for these operator services, there may be no issue regardless of in which networks the UE starts access to the operator services. However, for emerging 3GPP operator services (e.g., avatar service, AIoT service, sensing service), there may not be no universal supports by different operators. In this case, how a UE identifies which operator services are supported by which networks, and/or what NAS functionalities should be minimally supported by all UEs, as a basic framework (e.g., discovery) to support any operator services.

- Support for flexible interworking with lower layer: 
Though 6G architecture should aim for reducing inter-dependency between NAS layer AS layer, it cannot be completely removed. For example, in previous generation, there are any procedure requiring information exchange between NAS layer and RRC layer. (e.g., access control, network selection, establishment control). If 6GS NAS supports flexibility and intends to minimize impacts to basic functionality, the interaction between the NAS and the RRC needs similar considerations.

- Supporting resiliency:
Many OTT services today relies on stable network connection. An interruption in communication layer causes bad QoE from user perspective. To provide reliable communication, in 5Gs, many features were introduced. However, the features introduced in 5GS are focused on user plane, e.g., redundant PDU sessions, duplication of data at lower layers. On the other hand, mechanisms to make control plane robust were not much discussed.
In 5GS, NAS signalling used only control plane. So, when this control plane is congested and/or a node (e.g., AMF) in the control plane breaks down, all signalling passing through the node also gets interrupted.
Maybe, some operators may use virtualization in the cloud or other implementation specific solution to address this issue. However, if multiple vendors are involved in the core network, then standardized way providing robustness to NAS signalling is required.

- Minimum NAS functionality per device type:
It may not be possible to categorize all potential device types. However, based on characteristic, there are some specific categories of the UE. For example, an IoT UE (e.g., sensors) has specific characteristic compared to typical UEs (e.g., smartphones). A UE for FWA or an NTN UE may have unique characteristic. For these UEs, minimum requirements or common requirements may be different. This warrants to investigate whether different minimum NAS functionality is needed per device type.

# 2. Proposal

It is proposed to agree on following text proposal.

**** First Change (baseline S2-2508978) ****

# 5.X. Key Issue #X: Aspects for non-access stratum functionality and UE-CN control signalling

This Key issue studies the following aspects:

1)	Whether and how to enable the introduction of a new non-access stratum functionality with minimal or no impact to other non-access stratum functionalities.

NOTE: Coordination with CT1 could be required

2)	NAS termination, transport of services via NAS and NAS messages routing

- Study how to provide robustness, and resiliency to UE-CN signaling procedures, e.g., providing multiple signalling path, how to distribute NAS messages among the multiple signalling paths.

3)	Whether and how to identify a minimum, common set of NAS functionalities per UE type (e.g., IoT, FWA, satellite)

4)	Study system procedures for non-access stratum functionality, UE-CN signaling procedures, including impact to RAN-CN interface and 6G CN interfaces related to these system procedures.

NOTE 1: Any impact on the UE and RAN interaction aspects will be coordinated with RAN3 and the decision for RAN-Core interface will be aligned with RAN3.

NOTE 2: Any impact on the RAN and core network interaction aspects will be coordinated with RAN2.

NOTE 3: Interworking and migration aspects, especially inter-PLMN aspects (e.g. roaming), where applicable, are covered by WT#2.

NOTE 4: Potential security and privacy impacts require coordination with SA3.

NOTE 5: 	It is assumed that this key issue covers impacts to 6G System procedures for functionalities such as mobility management, session management and UE NAS identifiers.

NOTE 6: Coordination with KIs related to WT1.2, WT2, WT#3, WT#4, WT#5, WT#6 is required.

**** Second Change (baseline S2-2508978) ****

# 5.X. Key Issue #Y: Support of UE-CN Control Signaling for Operator Services offered by 3GPP System

This Key Issue focuses on studying the support for UE-Core Network control signalling for operator services offered by 3GPP System, including the following:

- Study whether and how to define generic mechanism including architecture, procedures and UE-core network interactions to support operator services offered by 3GPP system

  - Study system procedures to support generic mechanism for supporting operator services, UE-CN signaling procedures, including impact to RAN-CN interface and 6G CN interfaces related to these system procedures.

  - Study how to support transfer and routing of UE-CN signaling (i.e. service-specific messages).

  - Identify and address the minimal NAS functionality that the UE needs to support including where the operator services are heterogeneous among operators/networks (e.g. connecting to different PLMNs or roaming cases).

NOTE 1: The key issue includes studying mechanism for transporting signaling for operator services. It is not precluded that signaling for operator services may use transport over NAS.

NOTE 2: Any impact on the UE and RAN interaction aspects will be coordinated with RAN3 and the decision for RAN-Core interface will be aligned with RAN3.

NOTE 3: Any impact on the RAN and core network interaction aspects will be coordinated with RAN2.

NOTE 4: Potential impact on Interworking to be coordinated with KIs related to WT#2.

NOTE 5: Coordination with KIs related to WT1.2, WT2, WT#3, WT#4, WT#5, WT#6 is required.

NOTE 6: Potential security and privacy impact to be coordinated by SA3.

**** End of Changes ****