3GPP SA WG2 Meeting #171	 S2-2509163

Wuhan, China, 13-17 October 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated

Title:	[WT#1.2, Network Slicing] Work task and Key issue proposals on Network Slicing for 6G

Document for:	Approval

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution elaborates on the specifics of Network Slicing scope within WT#1.2.

# 1. Justifications

Network slicing is one of the features in WT#1.2 subject to study whether and how to support and/or enhance in 6G.

The current network slicing design has a main drawback:

Drawback of 5G Network Slicing #2: Lack of awareness of Network Slice capabilities at the UE

In 5G Network Slicing, the UE is unaware of the KPIs and capabilities that are provided by a PLMN for a given S-NSSAI.

This is an issue for scenarios where the binding of applications to slices is not done based on AppID but using other means.

For instance, there have been efforts to enable operators to bind slices to traffic categories (NSSP in URSP), and the HLOS to then bind applications to those traffic categories. GSMA has attempted to define semantics (KPIs and capabilities) for those traffic categories, see for example GSMA NG.141 [1], NG.116 [2] and NG.135 [3] for more details. However, a standardized approach turned out to be too inflexible as it prevents operators from deciding which KPIs to support per traffic category based on deployment, area, etc.

Proposal #1: It is proposed to study solutions for PLMNs to provide the UE with the KPIs allocated to each network slice, and this information to be exposed to the UE’s upper layers to facilitate binding of applications to network slices.

# 2. Proposal

It is proposed to include the following changes to TR 23.801-01:

**** First Change ****

# Annex A.X. WT#1.2 Scope on Network Slicing

Taking network slicing for 5G system as defined in TS 23.501 [X] as a baseline, the scope of the network slicing work task includes the following potential enhancements:

1.	Identify and address improvements of the application traffic mapping to network slice(s).

2.	Study impacts, if any, of network slicing for 6G system on interworking and migration.

NOTE: Interworking and migration is studied in conjunction with WT#2.

**** Second Change ****

## 5.X	Key Issue #X: Key Issue on Network Slicing support in 6G System

This Key issue covers Network Slicing aspects of WT#1.2.

Taking Network Slicing for 5G System as defined in TS 23.501 [X] as a baseline, this key issue includes the following potential enhancements to Network Slicing in 6G System:

- Identify and address improvements of the application traffic mapping to network slice(s).

2.	Study impacts, if any, of network slicing for 6G system on the interworking and migration.

NOTE: Interworking and migration is studied in conjunction with WT#2.

**** End of Changes ****

# 3. References

[1] GSMA NG.141: Guidelines for URSP

[2] GSMA NG.116: Generic Network Slice Template and NESTs

[3] GSMA NG.135: E2E Network Slicing Requirements