SA WG2 Meeting #171	S2-2509196

Wuhan, China, 13-17 October 2025

Source:	Qualcomm Incorporated

Title:	Interim agreements for FS_Sensing_ARC KI#2

Document for:	Agreement

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This contribution proposed the interim agreements for KI#2 of ISAC study.

# 1.  Discussions

Based on the discussion in the SA2 and the solutions documented in TR 23.700-14, this contribution proposed the interim agreements for KI#2 Authorization and Revocation to Support Sensing Service.

# 2.  Proposals

To apply the following change to the TR 23.700-14.

**** First Change ****

### 7.1.Y	Agreed Principles for KI#2

The following principles are agreed in this release to address the key issue on Authorization and Revocation to Support Sensing Service:

-	sensing service request is authorized by the Sensing Gateway Function, based on the parameters of the sensing service request;

-	the sensing service consumers in the trusted domain, e.g. NFs or AFs, are authorized by the Sensing Gateway Function, and sensing service consumers in untrusted domain, e.g. external AFs, are authorized by the NEF;

-	sensing service authorization revocation can be triggered by Sensing Function;

-	sensing service request can be cancelled by the requesting NF or AF;

-	the gNB based sensing entity is authorized for sensing service based on OAM configurations.

**** Next Change ****

### 7.2.Z	Topics for further consideration for KI#2

The following topics requires further considerations and would not be pursued in normative phase of this release:

-	support of UE based sensing consumer directly requesting sensing service to Sensing Function or NEF.

NOTE: UE based sensing consumer requesting sensing service via an AF is allowed.

**** End of Changes ****