3GPP SA WG2 Meeting #171	S2-2509337

# 13- 17 October, 2025, Wuhan, China	         		       	 (revision of S2-2507243, S2-2508610)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6408 | rev | 2 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications and corrections on usage of MoQ for encrypted media flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-09-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The metadata in an MoQ object header is used to transfer the ciphered MRI (Media Related Information). Currently, the clause 5.37.9.1 specifies that only the PDU Set Information and the EoDB Indication (of the MRI) are supported. Now, the IETF MoQ draft (https://datatracker.ietf.org/doc/draft-ietf-moq-transport/) has provided the standardized extension to MoQ Object header, via multi-extension header type(s), to accommodate more generic information. As quoted from the MoQ draft: the object extension headers are defined in external specifications and registered via an IANA table which is also explained in the same IETF draft. These external specifications define the type and value of the header, along with any rules concerning processing, modification, caching and forwarding of the corresponding object. For a MoQ object, the number of extensions can be derived from the ‘Extension Headers Length’. With this extension, the MoQ would be able to, in the more generic way, handle all the MRI as specified in the current clause 5.37.9.1, aligned with the other two on-path N6 singalling methods (i.e., Connect-UDP and UDP-option). - This CR also provides some clarifications and editorial updates on the MoQ usage for encrypted XR and media flows to improve the readability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Align the supported MRI by MoQ with the other two on-path N6 signalling methods in 5.37.9.1. - Clarifications on interactions between SMF and UPF for MoQ; - Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description of MoQ in the specification is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.9.1, 5.37.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of changes * * * *

#### 5.37.9.1	General

This clause describes the procedure to support transferring of media related information (i.e. information related to the media being transmitted, either explicitly provided by the AS or inferred from protocol headers or payload, e.g. PDU Set Information, Data Burst Size, etc.) over N6. Such support is necessary is when the end-to-end connection for XR and media traffic between the UE and Application Server is fully encrypted thus UPF not being able to identify, for example, PDU set information.

On-path N6 signalling methods are used to transfer media related information over N6 where media related information is sent together with the encrypted payload allowing media related information to be received synchronously with the media packet. The following on-path N6 signalling methods are supported:

-	Using Media over QUIC (MoQ), as defined in IETF draft-ietf-moq-transport [201], whose usage is further defined in clause 5.37.9.2.

-	Use of connect-UDP for Using Proxying-UDP-in-HTTP, as defined in IETF RFC 9298 [170] and QUIC-Aware Proxy, draft-ietf-masque-quic-proxy [200], whose usage is further defined in clause 5.37.9.3

-	Using UDP-Options, as defined in IETF draft-ietf-tsvwg-udp-options [202], whose usage is further defined in clause 5.37.9.4.

For all the above listed on-path N6 signalling methods, the following media related information are supported:

-	PDU Set Information (as described in clause 5.37.5.2).

-	End of Data Burst Indication (as described in clause 5.37.8.3).

-	Expedited Transfer Indication (as described in clause 5.37.10).

-	Data Burst Size (as described in clause 5.37.10).

-	Time to Next Burst (as described in clause 5.37.10).

The mechanisms defined in clause 5.37.9 address only UE to XRM server (AS) communication and do not address UE-UE XRM communications. The mechanisms defined in clause 5.37.9 support only downlink media related information delivery to the UPF.

The media related information is encrypted and integrity protected between the UPF and the AS, as defined in TS 33.501 [29].

# * * * *Next change * * * *

#### 5.37.9.2	Usage of Media Over QUIC in order to Handle end-to-end encrypted XR and media flows

When a PSA UPF that supports MoQ relay functionality has been selected, encrypted Metadata associated with traffic can be transported between the UPF and the AS via Media over QUIC (MoQ), IETF draft-ietf-moq-transport [201].

A PSA UPF supporting the MoQ relay functionality can be selected by SMF with the consideration of the DNN and S-NSSAI provided by UE.

NOTE 1:	The application client in the UE uses MoQ Transport protocol and establishes a MoQ connection using the address of MoQ relay between the application client and MoQ relay in UPF as defined in MoQ, IETF draft-ietf-moq-transport [201].

The SMF may get the address of MoQ relay from the UPF during N4 session management procedure as defined in the clause 4.4.1 of TS 23.502 [3]: the SMF indicates the PSA UPF to provide MoQ relay address in N4 Session Establishment/Modification Request and the PSA UPF returns MoQ relay address in N4 Session Establishment/Modification Response.

NOTE 2:	The application client can obtain the MoQ relay address via application layer-based schemes.

In the case the EASDF based DNS procedure as described in clause 6.2.3.2.2 of TS 23.548 [130] is used, after SMF has received FQDN in DNS message report from EASDF, if SMF determines that the FQDN is for MoQ traffic, then SMF gets the address of MoQ relay from UPF and then SMF provides DNS message handling rule with the address of MoQ relay to instruct EASDF to return the address of MoQ relay in PSA UPF by setting the Forwarding Action as Respond directly to the DNS request according to TS 23.548 [130]. In this case a local PSA UPF supporting MoQ relay maybe selected by SMF.

NOTE 3:	It is assumed an application client can send a DNS query to resolve FQDN.

NOTE 4:	The FQDN can be the FQDN for MoQ traffic and preconfigured in SMF or it can be the FQDN of the MoQ relay and preconfigured in SMF and application client. It is assumed there is SLA between operator and application service provider for using MoQ relay functionality provided by operator.

NOTE 5:	In some other case, the application client can also obtain the MoQ relay FQDN or IP address from the MoQ application server via the query during the MoQ session establishment between the application client and the MoQ application server as in MoQ, IETF draft-ietf-moq-transport [201], which is out of the scope of 3GPP. The MoQ application server can get the MoQ relay FQDN or IP address via local configuration.

NOTE 6:	Different MoQ application servers can correspond to different MoQ relays that are deployed on the same PSA UPF.

The AF provided Protocol Description indicating Media over QUIC Transport, IETF draft-ietf-moq-transport [201] can be used in determining the PCC Rule by the PCF as defined in clause 6.1.3.27.4 of TS 23.503 [45]. Based on the PCC rule, the SMF generates the PDU Set Information marking Indicator to instruct the PSA UPF to perform PDU Set Information identification. Upon receiving the MoQ SUBSCRIBE message from UE application client, the MoQ relay can establish an upstream MoQ subscription to the MoQ application server if there is no existing subscription yet.

For the downlink service data flow packet using MoQ Transport, based on the Indicator received from SMF, the PSA UPF can identify the PDU Set Information from the MoQ Metadata as defined in clause 9 of MoQ, IETF draft-ietf-moq-transport [201] that is extracted from the received data packet by the MoQ relay and provide the PDU Set Information to the RAN by adding it in the GTP-U header.

NOTE 7:	The format of the MoQ Metadata carrying the media related information and transported in the MoQ object extention header(s) is defined in TS 29.561 [132]. Morever, the mappings between the MoQ Metadata and the media related information can be based on local configuration, or operator-determined implementations.

# * * * * End of changes * * * *