3GPP TSG-WG SA2 Meeting #171	S2-2509414

Wuhan, CN, 13th – 17th Oct, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	KI#2: Conclusion

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: Conclusion on KI#2 on authorization is proposed in this document.

1. Introduction

Based on the discussions and analyses in S2-2508235 and the architectural conclusion of KI#1 in S2-2508236, this paper provides the conclusion on KI#2 on Authorization and Revocation to Support Sensing Service.

Rev 1:

As per the guidance from the rapporteur, the revision of the document also includes all/part of the proposals of the following documents:

Table 1-1: List of merged documents of KI#2.

| S2-2508237 | 23.700-14: KI#2: Conclusion. | Huawei, HiSilicon | Handle as baseline |
| --- | --- | --- | --- |
| S2-2508366 | 23.700-14: Proposed agreement of principles for Key Issues 2. | Nokia | Merge into S2-2508237 |
| S2-2508415 | 23.700-14: Interim agreements on KI#2: Authorization and Revocation to Support Sensing Service. | Vivo | Merge into S2-2508237 |
| S2-2508520 | 23.700-14: Evaluations and proposed Interim conclusion for KI#2. | Lenovo | Merge into S2-2508237 |
| S2-2508811 | 23.700-14: Proposal for Interim Agreements on KI#2. | Interdigital | Merge into S2-2508237 |
| S2-2508829 | 23.700-14: Interim conclusion proposal to KI#2 Authorization and Revocation to Support Sensing Service. | ZTE | Merge into S2-2508237 |
| S2-2508855 | 23.700-14: Interim agreement for KI#2. | Ericsson, AT&T | Merge into S2-2508237 |
| S2-2509099 | 23.700-14: Proposal on Interim agreements for KI#2. | Samsung | Merge into S2-2508237 |
| S2-2509135 | 23.700-14: Interim Conclusion proposal for KI#2. | CEWiT | Merge into S2-2508237 LATE DOC: RX 04/10, 07:45 |
| S2-2509196 | 23.700-14: Interim agreements for FS_Sensing_ARC KI#2. | Qualcomm Incorporated | Merge into S2-2508237 |
| S2-2509250 | 23.700-14: Interim agreement/Conclusion for KI#2 Authorization and Revocation to Support Sensing Service. | Xiaomi | Merge into S2-2508237 |
| S2-2508604 | 23.700-14: Interim Agreement Proposals for FS_Sensing_ARC. | Apple |  |

Key issue #2 should address the following aspects:

1.	Storage of the authorization information

a)	Whether a Dedicated NF other than Sensing (Control) Function is used to store the sensing authorization information;

Majority of the companies prefer to use Sensing (Control) Function to store authorization information, and three companies support both (pre-configure or using a dedicated storage), and one companies support UDM/UDR only.

Proposal 1a: it is proposed to make use of the Sensing (Control) Function to store the sensing authorization information.

b)	How to provide the sensing authorization information to Dedicated NF/Sensing (Control) Function;

Proposal 1b: Based on proposal 1a, if it is the Sensing (Control) Function that stores the sensing authorization information, such information can be either pre-configured or configured by OAM, as mentioned by several proposals (e.g., S2-2508415, S2-2509099, S2-2509135).

2.	Sensing authorization information

a)	What is the parameter(s) included in sensing authorization information;

The following parameters are mentioned by different documents:

| Parameters | Contributors | Contributions |
| --- | --- | --- |
| location-aware profile (NOTE 1) | Nokia | S2-2508366 |
| object-aware profile (NOTE 2) | Nokia | S2-2508366 |
| allowed/unallowed service area | Vivo, Lenovo, InterDigital, Ericsson, AT&T, Samsung, Xiaomi, CEWiT, Apple, Huawei | S2-2508415, S2-2508520, S2-2508811, S2-2508855, S2-2509099, S2-2509250, S2-2508604, S2-2508237 |
| allowed/unallowed service time | Vivo, InterDigital, Ericsson, AT&T, Samsung, CEWiT, Apple, Huawei | S2-2508415, S2-2508811, S2-2508855, S2-2509099, S2-2509135, S2-2508604, S2-2508237 |
| (Allowed) sensing service type | Lenovo, InterDigital, Ericsson, AT&T, Samsung, CEWiT, Apple | S2-2508520, S2-2508811, S2-2508855, S2-2509099, S2-2509135, S2-2508604 |
| sensing service requirement | Lenovo, InterDigital | S2-2508520, S2-2508811 |
| Allowed AF IDs | Ericsson, AT&T, CEWiT, Huawei | S2-2508855, S2-2509135, S2-2508237 |
| NOTE 1: The location-aware profile might be equal to allowed/unallowed service area. NOTE 2: The service type may include the ability of object-aware, e.g., for service type of UAV detection, it needs to identify the detected object(s) is a UAV other than a bird. |  |  |

The object-aware profile denotes whether the required object type (e.g., UAV, pedestrian) in the sensing service request is authorized, the service type may include the ability of object-aware, e.g., for service type of UAV detection, it needs to identify the detected object(s) is a UAV other than a bird. In addition, from the author’s point of view, this depends on the capability on whether it is feasible to determine the object type, or just a rough description (e.g., height, width, length). The details can be further clarified in the normative phase.

Proposal 2a: It is proposed to consider 1) allowed/unallowed service area; 2) allowed/unallowed service time; 3) (Allowed) sensing service type and 4) Allowed AF IDs as the information for Sensing authorization.

The NFs that make use of those information will be discussed in the following part.

3.	Call-flow related

a)	Which NF(s) is used to proceed the authorization;

The proposals on the table are in general in two directions:

Direction 1: NEF authorizes the AF request, SCF performs authorization of the sensing service request;

Direction 2: NEF authorizes the AF request, Gateway performs authorization of the sensing service request;

The first step of both directions is exactly the current NEF functionality as defined in TS 23.502, i.e., NEF authorizes the AF request based on the AF ID in the sensing service request and the Allowed AF IDs locally.

The differences regarding the second step on this KI are actually "which entity conducts the sensing service request authorization", note that the used parameters should be the 1)-4) in proposal 2a.

Proposal 3a: It is proposed to let NEF authorizes the AF request, based on the AF ID in the sensing service request and the Allowed AF IDs locally. Sensing (Control) Function/Sensing Gateway authorizes the sensing service request based on the parameters in proposal 2a.

The exact form of "Sensing (Control) Function/Sensing Gateway" can be determined.

b)	Which NF(s) can trigger the revocation.

Proposals from all contributions are in principle same. From the author's observation it is mainly the term issue: The NF who conduct the authorization can trigger the "reverse execution" i.e., revocation. As such, Sensing (Control) Function and NEF can trigger the revocation, while AF can actually "cancel" the sensing service request.

Proposal 3b: Sensing (Control) Function and NEF can trigger the revocation, while AF can actually "cancel" the sensing service request.

4.	Others

a)	Whether the authorization of the Sensing Entity (i.e., gNB) is needed;

Observation 4a: According to the proposals, it is more related to "discovery", i.e., how to select the gNB as the sensing entity as per the request.

b)	How to enforce the authorization (e.g., the procedure when authorization is failed/partially failed);

Observation 4b: Detailed-level stuff, can be either clarified in the revision of this document, or in the normative phase.

c)	Whether and what special aspects to consider for regulatory services;

Observation 4c: Seems to be no specific enhancements on the framework.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-14.

# * * * * First change * * * * all new text

7.1.2	Agreed Principles for KI#2 Authorization and Revocation to Support Sensing Service

The following principles are agreed for KI#2:

a)	The Sensing Service authorization consists of two steps:

1)	Authorisation of the AF request for Sensing Service is performed by the NEF, if the AF is outside the trusted domain, as defined in TS 33.501 [9];

2)	Authorisation of the AF's Sensing Service Request performed by the Sensing (Control) Function/Sensing Gateway;

NOTE 1:	The selection of Sensing Function, Sensing Control Function, and Sensing Gateway will be determined by KI#1.

b)	Sensing (Control) Function/Gateway determines whether the Sensing Service Request from the AF is authorised, considering the Sensing authorisation information described in table 7.1.2-1. Additional parameters, if any, and further details on Sensing authorization information can be determined during the normative phase.

Table 7.1.2-1: Sensing Authorization information for Sensing Service

| AF Authorization Data | Description |
| --- | --- |
| AF ID | Identifier used to identify the AF. |
| Allowed/Not allowed area | Indicate the allowed/not allowed area for the indicated AF to trigger the sensing services operations. (Note 1) |
| Allowed/Not allowed time period | Indicate the allowed time period within which AF can trigger a particular sensing service operation |
| (Allowed) sensing service type | Indicate the allowed sensing service type (e.g., object tracking, detection) for the indicated AF to trigger the sensing services operations. |
| Note 1: the Allowed area and the Not allowed area may be both present or only one of them can be present, for example if the AF is allowed to make a request for the majority of PLMN coverage excluding the not allowed area only the not allowed area can be present. If both are not present all area is allowed. |  |

c)	The Sensing Authorization information is pre-configured or configured by OAM, and stored in Sensing (Control) Function/Sensing Gateway for authorisation of the sensing service request;

d)	At any time the Sensing (Control) Function/Sensing Gateway may determine to revoke the Sensing Service Request when the authorisation criteria are no longer met.

e)	There is no need for authorisation for the gNB as the Sensing Entity, as the gNB is deployed by operator.

# * * * * End of changes * * * *