3GPP TSG-WG SA2 Meeting #171	S2-2509485

Wuhan, China, October 13 – 17, 2025

# Title:	Reply LS on clarification of UAV regulation from SA2

# Response to:	S2-2508188/R3-255899

# Release:	Rel-19

# Work Item:	UAS_Ph3

Source:	SA2

To:	RAN3

Cc:

Contact Person:

Name:	Aleksejs Udalcovs

E-mail Address:	aleksejs.udalcovs@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: NA

1.	Overall Description

SA2 thanks RAN3 for the Reply LS on clarification of UAV regulation. Regarding the requested clarification on altitude reporting for aerial UEs, i.e.:

“RAN3 seeks for further clarification from SA2 on whether the gNB can be provided with multiple aerial UE reporting configurations from the AMF for the same aerial UE. If so, RAN3 asks whether this would require the gNB to configure the aerial UE with multiple aerial UE flight information configurations to the concerned aerial UE, and whether the gNB is expected to report multiple flight information reporting to AMF in accordance with each configuration.”,

SA2 has discussed the matter and provides the following inputs on UAV UE’s altitude reporting.

Configuration of an aerial UE to perform altitude reporting “can be used in conjunction with the NEF-assisted pre-flight planning, in-flight monitoring and USS changeover as specified in clauses 5.12.2, 5.12.3 and 5.13, respectively”, which can ultimately require that different altitude thresholds are configured in the aerial UE while it moves, for instance, across different TAs according to its flight plan, see clause 5.16.2 of TS 23.256.

#---excerpt from clause 5.16.2 of TS 23.256---

2.	Based on the flight path information included in step 1 and other information obtained as a result of either of the procedures (i.e. NEF-assisted pre-flight planning specified in clause 5.12.2, NEF-assisted in-flight monitoring specified in clause 5.12.3, or USS changeover specified in clause 5.13.2), the UAS NF/NEF determines minimum and maximum values of altitudes (i.e. altitude range) for UAV's flight across each of the determined tracking areas (TAs) from the starting point to the destination point.

The determined minimum and maximum altitude values for each of the UAV UE is used by the UAS NF/NEF to derive altitude thresholds for altitude reporting events.

NOTE 1:	Event H1 and H2 for altitude-based reporting specified in clause 5.5.4.21 and clause 5.5.4.22 of TS 38.331 [23] can be used for altitude thresholds and event-based reporting.

Additionally, the UAS NF/NEF determines whether to request the aerial UE to report its altitude information periodically, threshold-based (e.g. altitude becomes a higher/lower than the provided threshold, multiple threshold values can be provided) or event-based (e.g. change of a tracking area/NG-RAN node).

#---

Based on the SA2 specified procedures and considering that a single aerial UE reporting configuration consists of one pair of derived altitude thresholds (i.e., minimum and maximum altitude thresholds) as indicated in the LS(R3-255899), SA2 provides the following clarifications.

- On “whether the gNB can be provided with multiple aerial UE reporting configurations from the AMF for the same aerial UE”: Aerial UE location reporting configuration shall enable altitude reporting whenever the UE’s altitude meets any of the configured altitude threshold values. SA2 does not exclude that different pairs of altitude thresholds (i.e., minimum and maximum altitude thresholds) can be derived by the UAS NF / NEF for each of the TA/Cell/NG-RAN in the assigned trajectory (see clause 5.2.2.3.1 of TS 23.502). Whether to use multiple LOCATION REPORTING CONTROL messages with one pair of derived altitude thresholds each to configure the NG-RAN node or to accommodate multiple pairs of derived altitude thresholds is a single aerial UE reporting configuration is outside SA2 scope.

#---excerpt from clause 5.2.2.3.1 of TS 23.502---

Table 5.2.2.3.1-1: Example of Event Filters for AMF exposure events

| Event ID | Event Filter (List of Parameter Values to Match) |
| --- | --- |
| […] |  |
| Assigned Trajectory | <Parameter Type = Notify when the UE deviates from the assigned trajectory, Value=TAI, Cell ID, or RAN Node ID, Scheduled Time in a particular TA, Cell, or gNB and/or Height, acceptable deviation (e.g. Distance, Time)> (to report only when the UE deviates from the assigned values of TAI, Cell ID, or RAN Node ID over the acceptable deviation of distance, Scheduled Time in a particular TA or Cell and/or moves outside the altitude range based on the assigned flight plan over the acceptable deviation on time). The assigned trajectory may consist of multiple segments identified by geographical areas that, for instance, are served by different UAV Service Suppliers. In this case, the information of each segment shall be provided. <Parameter Type = Notify when the altitude reporting conditions are met, Value=altitude thresholds, reporting periodicity> (NOTE 4) |
| […] NOTE 4: Filter information for the events is provided as a set of TAI, Scheduled Time and/or Height values for each of the trajectory segments assigned to the UE/UAV, see clauses 5.12 and 5.13 of TS 23.256 [80]. NOTE 5: Procedures related to reporting altitude information of aerial UEs are specified in clause 5.16 of TS 23.256 [80]. |  |

#---

- On “whether this would require the gNB to configure the aerial UE with multiple aerial UE flight information configurations to the concerned aerial UE”: Yes, if a single aerial UE flight information configuration includes only one pair of the derived altitude thresholds (i.e., minimum and maximum altitude thresholds) to the concerned aerial UE.

- On “whether the gNB is expected to report multiple flight information reporting to AMF in accordance with each configuration”: AMF is expected to detect the configured Assigned Trajectory event whenever the event conditions are met (see clause 5.2.2.3 of TS 23.502) so that the UAS NF /NEF can be notified when the aerial UE deviates from the assigned trajectory, which includes moving outside the assigned altitude range while present in a certain TA/cell/NG-RAN.

2.	Actions

To RAN3:

ACTION: 	SA2 respectfully asks RAN3 to take the above information into account and align.

3.	Date of Next TSG SA WG2 Meetings

SA2#172	November 17 – 21, 2025	Dallas, US

SA2#173	February 9 – 13, 2026	TBD, India