3GPP TSG-SA WG2 Meeting #171	S2-2509615

Wuhan, China, 13 – 17 October 2025											 (revision of S2-2509351)

Source:	Ericsson

Title:	KI#2, New Solution: SMS over NAS delivery to Emergency Response Centre with emergency numbers

Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract of the contribution: This contribution provides a solution for SMS over NAS delivery to Emergency Response Centre KI#2.

# 1.	 Proposal

It is proposed to update TR 23.700-65 with below changes:

* * * Start of change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |
| #x |  | x |  |
| - |  |  |  |
| - |  |  |  |
| - |  |  |  |
| - |  |  |  |

* * * Next change (all new)* * * *

## 6.X	Solution #X: SMS over NAS delivery to Emergency Response Centre based on emergency call numbers

### 6.X.0	High level principles

This solution aims to address KI#2 with the following key principles (considering the solutions for KI#1):

-	The UE uses emergency call numbers/serving PLMN SM service centre address as RP-DA-destination.

-	The SMSF/MME is responsible to deliver the SMS to the SM service centre based on the identification of the emergency number/serving PLMN SM service centre address.

-	The serving SMS service centre is responsible to forward the SMS to the local Emergency Response Centre.

### 6.X.1	Description

In this proposal, it is also assumed that UE is aware of the supporting capability of the network based on the solutions provided for KI#1 (e.g. UE is informed that SMS over NAS delivery to Emergency Response Centre is supported by the network during the UE Registration/Attach procedure).

The main change is SMSF/MME deliver the SMS to the SMS service centre based on the RP-DA field.

### 6.X.2	Procedures

#### 6.x.2.1	MO SMS over NAS delivery procedures

##### 6.x.2.1.1	MO SMS over NAS in CM-IDLE in 5GS

The following procedures are based on clause 4.13.3 in TS 23.502 [3] with addition in flowing steps.

Figure 4.13.3.3-1: MO SMS over NAS

1.	The UE performs domain selection for UE originating SMS as defined in clause 5.16.3.8 of TS 23.501 [2] if SMS delivery via non 3GPP access is allowed and possible. If an UE under CM-IDLE state is going to send uplink SMS message, then UE and network perform the UE Triggered Service Request procedure firstly as defined in clause 4.2.3.2 to establish a NAS signalling connection to AMF.

2a.	The UE builds the SMS message to be sent as defined in TS 23.040 [7] (i.e. the SMS message consists of CP-DATA/RP-DATA/TPDU/SMS-SUBMIT parts). The SMS message is encapsulated in an NAS message with an indication indicating that the NAS message is for SMS transporting. The UE send the NAS message to the AMF. When UE builds the SMS message to be sent to Emergency Response Centre, the RP-DA field includes the SMS service centre address/the emergency service number provided by the AMF when AMF sets the “SMS over NAS delivery to Emergency Response Centre allowed” indication during UE registration procedure.

2b.	The AMF forwards the SMS message and SUPI to the SMSF serving the UE over N20 message by invoking Nsmsf_SMService_UplinkSMS service operation. In order to permit the SMSF to create an accurate charging record, the AMF adds the IMEISV, the current UE Location Information (ULI) of the UE as defined in clause 5.6.2 of TS 23.501 [2] and if the UE has sent the SMS via 3GPP access, the local time zone.

When the AMF determines that the UE has the MPS for Messaging indication set (enabled) in the UE context, the AMF includes a Message Priority header to indicate priority information. Other NFs relay the priority information by including the Message Priority header in service-based interfaces, as specified in TS 29.500 [17].

2c.	The SMSF invokes Namf_Communication_N1N2MessageTransfer service operation to forward SMS ack message to AMF.

2d.	The AMF forwards the SMS ack message from the SMSF to the UE using downlink unit data message.

3-5.	The SMSF checks the SMS management subscription data. If SMS delivery is allowed, the procedure defined in TS 23.040 [7] or TS 23.540 [84] applies. If the Message Priority header received in step 2 contains a value that indicates MPS, the SMSF may set the transport priority (e.g. the DRMP) to a value appropriate for MPS. If the RP-DA includes the serving SMS service centre address/emergency service number, the SMSF delivers the SMS to the SMS service centre of the serving PLMN. The SMS service centre delivers the SMS to corresponding Emergency Response Centre based on emergency numbers.

6a-6b.	The SMSF forwards the submit report to AMF by invoking Namf_Communication_N1N2MessageTransfer service operation which is forwarded to UE via Downlink NAS transport. If the SMSF knows the submit report is the last message to be transferred for UE, the SMSF shall include a last message indication in the Namf_Communication_N1N2MessageTransfer service operation so that the AMF knows no more SMS data is to be forwarded to UE.

NOTE:	The behaviour of AMF based on the "last message indication" is implementation specific.

If the UE has more than one SMS message to send, the AMF and SMSF forwards SMS /SMS ack/submit report the same way as described in step 2a-6b.

6c-6d.	When no more SMS is to be sent, UE returns a CP-ack as defined in TS 23.040 [7] to SMSF. The AMF forwards the SMS ack message by invoking Nsmsf_SMService_UplinkSMS service operation to SMSF.

Editor's note:	Undetectable SMS delivery to Emergency Response Centre is FFS.

##### 6.x.2.2	MO SMS over NAS delivery procedures in EPS

The SMS over NAS delivery to Emergency Response Centre is similar to 5GS procedure with the change that the MME takes to role of AMF/SMSF.

Editor's note:	SGs interface based on SMS delivery is FFS.

### 6.X.3	Impacts to Services, Entities and Interfaces

UE:

-	Support the capability of SMS over NAS delivery to Emergency Response Centre during registration procedure, e.g., accept “SMS over NAS delivery to Emergency Response Centre allowed/supported” indication in Registration Accept.

-	Formulates the RP-DA in the RP-DATA with serving PLMN SMS service centre or emergency number provided by the AMF.

AMF/MME:

-	Support the capability of SMS over NAS delivery to Emergency Response Centre during registration procedure, e.g., accept “SMS over NAS delivery to Emergency Response Centre allowed/supported” indication in Registration Accept.

SMSF/MME:

-	Support of SMS delivery to SMS service centre of the serving PLMN based on the identification of the SMS service centre address/emergency number.

* * * End of change * * * *