SA WG2 Meeting #172	S2- 2509911

17 – 21 November 2025, Dallas, US

Source:	LG Electronics

Title:	KI#4: Conclusions and Interim Agreements

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This paper proposes conclusions and interim agreements for KI#4.

Proposal

It is proposed to agree the following changes into TR 23.700-14.

* * * * Start of 1st Change * * * *

7.1	Agreed Principles

7.1.4	Agreed Principles for KI#4: Sensing Data and the Associated Information Collection and Transport

The following principles are agreed for KI#4:

-	Based on the received sensing service requirements from the Sensing Function (SF), the Sensing Entity (i.e., gNB) can determine how to perform the sensing measurement to collect the sensing data.

-	The sensing data is transmitted from the SE to the SF via direct connection.

-	The Sensing Entity can support one-time, periodical sensing data transmission to the Sensing Function according to sensing service request.

Editor's note: The type (one time, periodical, etc.) and content of sensing data that will be provided by gNB needs to be coordinated with RAN WGs in the normative phase.

-	Sensing data can be collected from one or multiple gNBs (i.e. SEs) by the Sensing Function for the same sensing service request.

-	The Sensing Function can generate sensing results based on the sensing data and sensing associated information provided by the Sensing Entity(ies) (i.e., gNB node(s)).

* * * * Start of 2nd Change * * * *

!! All New Texts !!

7.2	Topics for further consideration

7.2.4	Topics for further consideration for KI#4

Topics for further consideration for KI#4 are as below:

-	What sensing data and associated information are transferred via a direct connection.

-	How to support the transport of large volumes of sensing data without affecting other services, and how to ensure QoS.

-	Where and how sensing data is aggregated and processed (e.g., at Sensing Entity(ies), Sensing Function(s)).

* * * * Start of 3rd Change * * * *

!! All New Texts !!

8.4	Conclusion on KI#4

The key issue #4 covers “The Sensing Data and the Associated Information Collection and Transport”.

The following aspects common to the sensing service are concluded as principles for normative work:

-	Based on the received sensing service requirements from the Sensing Function (SF), the Sensing Entity (SE) (i.e., gNB) can determine how to perform the sensing measurement to collect the sensing data.

-	Sensing data is transmitted from the SE to the SF via a direct connection.

-	The SE can support one-time, periodical sensing data transmission to the SF according to the sensing service request.

-	Sensing data can be collected from one or multiple SEs by the SF for the same sensing service request.

-	The SF can generate sensing results based on the sensing data and sensing associated information provided by the SE(s) (i.e., gNB node(s)).

NOTE 1:	The type (one time, periodical, etc.) and content of sensing data that will be provided by gNB needs to be coordinated with RAN WGs in the normative phase.

NOTE 2:	How to support the transport of large volumes of sensing data without affecting other services, and how to ensure QoS will be discussed during the normative phase.

NOTE 3:	How the sensing data is aggregated and processed (e.g., at Sensing Entity(ies), Sensing Function(s)) will be discussed during the normative phase.

NOTE 4:	What sensing data and associated information are transferred via a direct connection will be discussed during the normative phase.

* * * * End of Changes * * * *