SA WG2 Meeting #172	S2-2510625

# November 17 - 21, 2025, Dallas, USA

Source: 	NTT DOCOMO

Title: 	KI#2: New Sol: Emergency registration and SMS over NAS with confidentiality

Document for: 	Approval

Agenda Item: 	20.8.1 (FS_SMS2EC_ARC)

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract of the contribution: This contribution proposes a new solution for KI#2.

1	Introduction

This contribution proposes a solution for KI#2 on "Support of SMS Delivery to Emergency Response Centre via NAS" for the UE detectable scenario in 5GS when the UE is about to send an emergency SMS to a network to which the UE cannot normally register (i.e., the cases of c, d in TS 23.401 clause 4.3.12.1), in particular defining the emergency registration for the emergency SMS and a mechanism that enables confidentiality of SMS.

With this solution, UE can send a confidential emergency SMS via a network to which the UE cannot normally register if the network allows it.

2	Proposal

It is proposed to add the following text to TR 23.700-65 v0.2.0.

********** First Change **********

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |
| X |  | X |  |

********** Next Change (All new text) **********

6.x	Solution #x: Emergency registration and SMS over NAS with confidentiality

6.x.0	High-level solution Principles

This solution is for KI#2 on "Support of SMS Delivery to Emergency Response Centre via NAS" for the UE detectable scenario in 5GS when the UE is about to send an emergency SMS to a network to which the UE cannot normally register (i.e., the cases of c, d in TS 23.401 clause 4.3.12.1), in particular defining the emergency registration for the emergency SMS and a mechanism that enables confidentiality of SMS.

With this solution, UE can send a confidential emergency SMS via a network to which the UE cannot normally register if the network allows it.

6.x.1	Description

6.x.1.1	General

The assumptions of this solution are as follows:

-	This solution is intended to be used for NTN, while it does not have any mechanism for that restriction.

The principles of this solution are as follows:

-	5GS supporting the emergency registration for the emergency SMS broadcasts a new indication to show that the emergency SMS is supported for unregistered UEs.

-	UE sends the Registration request by setting 5GS registration type = emergency registration and indicating the emergency SMS is requested and the UE is capable of the confidential emergency SMS. AMF sends to UE the Registration accept containing the PLMN public key.

-	UE creates an emergency SMS, encrypts it by using the PLMN public key, and sends it to AMF. AMF forwards the received confidential emergency SMS to UDM/SIDF, which returns the decrypted emergency SMS to AMF.

The advantages of this solution are as follows:

-	UE can send a confidential emergency SMS via a network to which the UE cannot normally register if the network allows it.

6.x.2	Procedures

Figure 6.x.2-1: Emergency registration and MO SMS procedure

1.	gNB broadcasts a new indication in SIB to show that the emergency SMS is supported for unregistered UEs.

2.	UE determines to send an emergency SMS via the network the gNB belongs to. UE sends to AMF the Registration request containing 5GS registration type = emergency registration, 5GS update type = unregistered emergency SMS services requested (a new value), and 5GMM capability = unregistered secure emergency SMS services supported (a new value).

3.	Based on the value of 5GMM capability, AMF decides to send the public key of the PLMN to UE. AMF sends to UE the Registration accept containing the public key and 5GS network feature support = unregistered secure emergency SMS services supported (a new value).

4.	UE generates CP-DATA and encrypts it by using the public key.

5.	UE sends to AMF the UL NAS transport containing Payload container type = encryptedEmergencySMS and Payload container that holds CP-DATA.

6.	Based on Payload container type = encryptedEmergency SMS, AMF recognizes that the AMF has received a confidential emergency SMS, which needs to be decrypted. AMF sends to UDM/SIDF the Nudm_SmsDeconceal request containing the encrypted CP-DATA. (a new message)

7.	UDM/SIDF decrypts the received encrypted CP-DATA.

8.	UDM/SIDF sends to AMF the Nudm_SmsDeconceal response containing the decrypted CP-DATA. (a new message)

9.	AMF selects a new SMSF, which the AMF has not accessed during the Registration procedure, capable of handling the emergency SMS. AMF sends to SMSF the Nsmsf_SMService_UplinkSMS request containing 3gpp-Sbi-Message-Priority header and SmsRecordData. The SmsRecordData contains CP-DATA, emergencySmsIndication (a new indication), and UE location. The SmsRecordData also contains UE ID, e.g., PEI.

10.	MO SMS procedure continues.

6.x.3	Impacts to Services, Entities and Interfaces

UE:

-	Support an indication to show that the emergency SMS is requested and an indication to show that the UE is capable of confidential emergency SMS.

-	Support capability to create the confidential emergency SMS by using the PLMN public key.

-	Support a new payload container type for the confidential emergency SMS or a new indication to show a confidential emergency SMS is sent.

gNB:

-	Broadcast a new indication to show that the emergency SMS is supported for unregistered UEs.

AMF:

-	Upon receiving an indication to show that the UE is capable of confidential emergency SMS, send to the UE the PLMN public key.

-	Receive a confidential emergency SMS and request UDM/SIDF to decrypt it.

UDM/SIDF

-	Decrypt the confidential emergency SMS.

********** End of Changes **********