3GPP TSG-WG SA2 Meeting #171	S2-2510672

Wuhan, China, Oct 13 - 17, 2025                          (revision of S2-2509100)

Source:	Samsung

Title:	Proposal on Interim agreements for KI#6

Document for:	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes Interim agreements for KI#6 for the FS_Sensing_ARC.

# 1.1	Discussion

There are 39 solutions present in the TR 23.700-14 V1.0.0, out of which 14 solutions address KI#6.

Solution #3, #15, #20, #27, #34, #35 and #39 is addressing KI#6 based on TR 23.700-14 V1.0.0.

Key functionalities are summarized as below:

- The configuration parameters are received from Sensing service consumer or sensing operation server

Sensing Function receives the configuration parameter from AF, NEF, some server, some parameter is additionally generated by SF itself

#3, #15, #20, #27, #34, #35, #39

- The configuration parameters are sent directly to Sensing Entity

Sensing Function sends the configuration parameters to sensing entity directly.

#3, #15, #20, #27, #34, #35

- Configuration parameters are sent via AMF to Sensing Entity

Sensing Function sent the configuration parameters via AMF through sensing service container

#35, #39

- The content of configuration parameters

Sensing service type, target sensing area, sensing duration, sensing KPI/QoS, Sensing object type, RCS (Radar Cross Section) range list, object velocity range, sensing accuracy, sensing resolution, frequency of collection of sensing data

#27, #34, #35, #39

Observation 1: Most of the solutions are in principle proposing to SF to configure the parameters to sensing entity with the exception that sol 35 suggest to have another option of sending via AMF as well

Proposal 1: Document all the common proposals as agreed principles in clause 7.1 and keep the remaining item in clause 7.2 for further discussion

# 1.2	Interim Agreement Merged details

| tdoc no | Interim agreement on KI#6 | Company |
| --- | --- | --- |
| S2-2509100 | 23.700-14: Proposal on Interim agreements for KI#6. | Samsung |
| S2-2508241 | 23.700-14: KI#6: Conclusion | Huawei, HiSilicon |
| S2-2508287 | 23.700-14: KI#6: Interim agreements. | LG Electronics |
| S2-2508365 | 23.700-14: Proposed agreement of principles for Key Issues 6. | Nokia |
| S2-2508833 | 23.700-14: KI#6 Configuration of Parameters for Sensing Entities, Interim conclusion proposal. | ZTE |
| S2-2508859 | 23.700-14: Interim agreement for KI#6. | Ericsson |
| S2-2509219 | 23.700-14: Proposal on Interim agreements for KI#6. | Oppo |
| S2-2508604 | Interim Agreement Proposals for FS_Sensing_ARC | Apple |

S2-2508859 (Ericsson): This paper suggest that KI#4 already includes the conclusion for KI#6 and hence there is no need for explicit conclusion KI#6. Need discussion this point during meeting.

# 2	Proposal

It is proposed to include the following changes in TR 23.700-14.

* * * * Start of Changes * * * *

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.Y	Agreed Principles for KI#Y

Based on the explanation in Discussion clause the following principles are proposed for interim agreement

-	Configuration of Parameters for sensing entities:

-	Configuration parameters for sensing service can be sent by AF or any other provisioning server and received by sensing function.

-	Sensing function will derive parameters received from AF or provisioning server. Also, service function can generate some parameters applicable for sensing service based on internal logic.

-	Sensing function will send directly the parameters to sensing entity to configure for the sensing service or via AMF in a container. In the case of via AMF, it will facilitate the communication by sharing each other’s address (e.g.SF ID given to sensing entity and SE ID will be given to sensing function)

-	Sensing parameters include Sensing service type, target sensing area, sensing duration, sensing KPI/QoS, Sensing object type, RCS (Radar Cross Section) range list, object velocity range, sensing contextual information, location and speed accuracy, address for receiving sensing data.

NOTE:	The detailed information provided by the Sensing Control Function to the gNB(s) need further discussion and feedback from RAN WG3 and specified during the normative phase.

## 7.2	Topics for further consideration

### 7.2.Z	Topics for further consideration for KI#Z

Editor's note:	The parameters send to sensing entity (i.e. gNB) needs co-ordination with RAN3.

Editor's note:	Whether and how sensing service requirements are mapped into sensing configuration parameters.

Editor's note:	Sensing function sending parameters via AMF to sensing entity depends upon the out come of KI#1 architecture principles.

* * * * Next Change * * * *

The following conclusions are agreed for KI#6:

Configuration of Parameters for sensing entities:

-	Configuration parameters for sensing service can be sent by AF or any other provisioning server and received by sensing function.

-	Sensing function will derive parameters received from AF or provisioning server. Also, service function can generate some parameters applicable for sensing service based on internal logic.

-	Sensing function will send directly the parameters to sensing entity to configure for the sensing service or via AMF in a container. In the case of via AMF, it will facilitate the communication by sharing each other’s address (e.g.SF ID given to sensing entity and SE ID will be given to sensing function)

-	Sensing parameters include Sensing service type, target sensing area, sensing duration, sensing KPI/QoS, Sensing object type, RCS (Radar Cross Section) range list, object velocity range, sensing contextual information, location and speed accuracy, address for receiving sensing data.

NOTE:	The detailed information provided by the Sensing Control Function to the gNB(s) need further discussion and feedback from RAN WG3 and specified during the normative phase.

* * * * End of Changes * * * *