3GPP TSG-SA2 Meeting #172	 S2-2510972

Dallas, TX, USA, November 17-21 2025	(revision of  S2-2510160)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5613 | Rev | 1 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on parameters for subscription to exposure of energy consumption |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When an AF requests energy exposure it is not specified what time basis to use for reporting or whether the reporting is threshold-based in the Nnef_EventExposure_Subscribe service operation. The EIF exposure subscription API also needs some alignment to the CR agreed at SA2#171 in S2-2509466 |  |  |  |  |  |  |  |  |
| Summary of change: |  | reporting threshold in association with a reporting period, time window or reporting period as described in clause 5.51.2.2.4 of TS 23.501 [2]. are added as optional inputs in clause 5.2.6.2.2. Additionally, it is clarified that the threshold is in association with a reporting period in clause 5.2.28.2.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete and incorrect specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.6.2.2, 5.2.28.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

##### 5.2.6.2.2	Nnef_EventExposure_Subscribe service operation

Service operation name: Nnef_EventExposure_Subscribe

Description: the consumer subscribes to receive an event, or if the event is already defined in NEF, then the subscription is updated.

Inputs, Required: (Set of) Event ID(s) as specified in clause 4.15.3.1 or Npcf_PolicyAuthorization_Notify and Naf_EventExposure_Subscribe service operation, Target of Event Reporting (GPSI, SUPI, UE IPv4 address(es), UE IPv6 prefix(es), External Group Identifier, S-NSSAI, Internal Group Identifier, UE addressing information (IP or MAC address), or indication that any UE is targeted), Event Reporting Information defined in Table 4.15.1-1, Notification Target Address (+ Notification Correlation ID), MTC Provider Information.

Inputs, Optional: Event Filter, (set of) External Application Identifier(s), Subscription Correlation ID (in the case of modification of the event subscription), Expiry time, list of group member UE(s) whose subscription to event notification(s) are removed or added for a group-based event notification subscription, operation indication (cancellation or addition), DNN, S-NSSAI, Idle Status Indication request (if UE reachability or Availability after DDN failure reporting is requested), and, for subscription to energy-related information provided by the EIF, (reporting threshold, reporting period) pair, time window, reporting period as described in clause 5.51.2.4 of TS 23.501 [2].

Outputs, Required: When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: First corresponding event report is included, if available (see clause 4.15.1). External Identifier (representing an AF specific UE Identifier).

More changes

##### 5.2.28.2.2	Neif_EventExposure_Subscribe service operation

Service operation name: Neif_EventExposure _Subscribe

Description: The consumer subscribes to receive the energy consumption information from the network, or the subscription is updated if the same subscription is already existing in EIF.

Inputs, Required: NF ID, Target of Event Reporting as defined in clause 5.2.28.2.1, Event ID defined in clause 5.2.28.2.1, Notification Target Address (+ Notification Correlation ID), Event Reporting Information defined in Table 4.15.1-1.

Inputs, Optional: Event Filter (s) associated with each Event ID; Event Filter(s) are defined in clause 5.2.28.2.1, and, for subscription to energy-related information provided by the EIF, (reporting threshold, reporting period) pair, time window, reporting period as described in clause 5.51.2.4 of TS 23.501 [2]].

Outputs, Required: When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: First corresponding energy related information report is included, if available.

End of changes