3GPP TSG-WG SA2 Meeting #172	S2-2511124

17 – 21 November, 2025, Dallas, USA	(revision of S2-2510049)

Source:	Lenovo, Nokia, NEC, ZTE, T-Mobile US, Deutsche Telekom, AT&T, Qualcomm, Google, MediaTek

Title:	[WT#1.2, Slicing] Proposal for a new KI on Network Slicing in 6G

Document for:	Discussions and Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes a new KI description for the Network Slicing in 6G.

# 1. Discussion

## 1.1	Summary of input documents

This document merges the content of the proposals submitted to SA2#172 meeting on the Key Issue on Network Slicing in 6G.

All considered papers are listed in the table:

| TD# | Network Slicing in 6G | Source |
| --- | --- | --- |
| S2-2510049 | 23.801-01: [WT#1.2, Slicing] Proposal for a new KI on Network Slicing in 6G | Lenovo, Nokia, NEC, ZTE, T-Mobile US, Deutsche Telekom, AT&T, Qualcomm, Google, MediaTek |
| S2-2510106 | Discussion on Network Slicing for 6G | KDDI CORPORATION, Vodafone, China Mobile, Deutsche Telekom |
| S2-2510693 | 23.801-01: [WT#1.2, Slicing] Proposal to revise the scope on Network Slicing in 6G | KDDI CORPORATION, Vodafone, China Mobile, Deutsche Telekom |
| S2-2510117 | 23.801-01: [WT#1.2, Network Slicing] Simplified Network Slicing | vivo |
| S2-2510308 | 23.801-01: 23.801-01: [WT#1.2, Slicing] New KI for Network Slicing in 6G | InterDigital |
| S2-2510490 | 23.801-01: [WT#1.2, Slicing] KI for 6G System Network Slicing | Huawei, Hisilicon |
| S2-2510492 | 23.801-01: [WT#1.2, Slicing] Proposal for KI Description on Network Slicing | NTT DOCOMO |
| S2-2510568 | 23.801-01: [WT#1.2, Network Slicing] KI proposals for Network Slicing in 6G | Samsung |
| S2-2510595 | 23.801-01: [WT#1.2, Slicing] Update of WT#1.2 | OPPO |
| S2-2510734 | 23.801-01: [WT#1.2, Slicing] KI for Network Slicing in 6G | Ofinno |

## 1.2	Open issues for discussion

The pen-holder proposes to discuss the following open issues resulting from the submitted contributions:

- Whether to take Option 1 or Option 2 for the basis of the KI description?

1a, Option 1:	to use the WT scope text as baseline for KI description. This is proposed by the majority of contributing companies; or

1b, Option 2:	 use new KI description text independent of the WT scope, e.g. as proposed in S2-2510568 (by Samsung) or S2-2510734 (by Ofinno).

- Whether to further expand the clarification of the bullets 1a and 1b?

  - If yes, what clarifications are agreeable?

- Whether to include a new bullet 1c as proposed in S2-2510693 (by KDDI CORPORATION, Vodafone, China Mobile, Deutsche Telekom)?

# 2. Proposal for the TR

It is proposed to include the following text in the TR 23.801-01-020.

**** First Change (all new text) ****

## 5.X	Key Issue #X: Support of Network Slicing in the 6G system

This Key Issue aims to study the support of Network Slicing in the 6G system, including the following aspects:

1.	Study the overall design and functionalities of the network slicing in 6G assuming the network slicing in 5GS as starting point for discussions including the following:

a.	Identify and address any areas of possible improvement and simplification of network slicing design in 5GS.

i.	Identify which functionality of the network slicing in 5G is necessary to be adopted in 6G;

ii.	Identify and address any areas of possible improvement and simplification for the network slicing in 5G which is to be adopted in 6G.

iii.	Minimize the option(s) of the same network slicing feature.

iv.	Minimize the number of impacted NFs for slicing features.

b.	Identify and address any improvements of the application traffic mapping to network slice(s)/user plane connection(s).

i.	to ensure efficient network slice selection/reselection mechanisms.

ii.	to ensure efficient application traffic mapping to user plane connection(s).

iv.	to harmonize non-roaming and roaming scenario.

c.	Identify and address any feedback from markets where network slicing technologies are already being commercially deployed, to facilitate continuous improvement and ensure ongoing relevance.

NOTE 1:	Coordination between bullet 1b and the WT 1.2.x on Policy is expected.

NOTE X:	Coordination between bullet 1b and aspects of key issues derived from WT3.2 is expected.

2.	Study impacts of the network slicing in 6G on the interworking and migration.

NOTE 2:	Impacts to RAN are possible and coordination with RAN WGs is expected.

NOTE Y:	Coordination between (2) and aspects of key issues deriving from the general interworking and migration procedures is expected.

[Option 2]

### 5.X. Key Issue #X: Simplified Network Slicing for 6G

The key issue will study support for simplified network slicing for 6G:

- Study how to improve the application traffic mapping to network slice(s) to enable an introduction of additional network slicing features without impacting UE

- Identify a set of network slicing features that have multiple options and investigate whether such features can be re-designed with reduced number of options

### 5.X. Key Issue for WT#1.2 on Network Slicing

This key issue will study how to design the network slicing feature in 6G. The study will cover, but not be limited to, the following aspects:

- Study the overall design and functionalities of the network slicing in 6G assuming the network slicing in 5GS as starting point for discussions, including at least:

  - How to enable the mapping of application traffic to network slices/user plane connections in a flexible and scalable manner, while ensuring preservation of user privacy at the UE.

NOTE:	For bullet 1a, the potential requirements impacting the 6G policy framework design (e.g. URSP for 6G) should be provided to the WT#1.2 Policy study.

**** End of Changes ****