3GPP TSG-WG SA2 Meeting #172	S2-2511150

# Dallas, USA, 17 - 21 November 2025										 (revision of S2-2510211)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6301 | rev | 5 | Current version: | 18.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE radio capability for paging handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, 5GS_Ph1, TEI17 |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | #### For UE radio capability for paging handling, SA2 has provided views to RAN3/RAN2 in S2-2506082 and S2-2506087. However, following points need further clarification: Expected NG-RAN handling on UE radio capability. Clarify the uploaded UE radio capability for paging parameter |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding NOTE to clarify the expected handling of NG-RAN node to support certain functions with reference to RAN spec. Clarify the uploading of the UE radio capability for paging to CN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear description on the UE radio capability for paging which can impact the functions/features that relys on proper handling of UE radio capability for paging. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.4.4.3	Paging assistance information

The paging assistance information contains UE radio related information that assists the RAN for efficient paging. The Paging assistance information contains:

a)	UE radio capability for paging information:

-	The UE Radio Capability for Paging contains information derived by the NG-RAN node (e.g. band support information) from the UE Radio Capability information. The NG-RAN provides a single UE Radio Capability for Paging (including different NG-RAN RATs) to the AMF as specified in TS 38.413 [34] and the AMF stores this information without needing to understand its contents.

NB-IoT specific UE Radio Capability information, including paging relation information, is stored separately in AMF and only communicated to NG-RAN when UE is camping on NB-IoT as described in clause 5.4.4.1.

As the AMF only infrequently (e.g. at Initial Registration) prompts the NG-RAN to retrieve and upload the UE radio capabilities i.e. UE Radio Capability information to the AMF and the AMF may be connected to more than one wideband NG-RAN RAT, it is the responsibility of the NG-RAN to ensure that UE Radio Capability for Paging (which is derived by the NG-RAN node) contains information on NG-RAN RATs and relevant functionality that the UE support as specified in TS 38.413 [34]. To assist the NG-RAN in this task, the AMF provides its stored UE Radio Capability for Paging in every NG-AP INITIAL CONTEXT SETUP REQUEST message sent to the NG-RAN. If the NG-RAN detects an omission, then the NG-RAN should ensure that an update of the UE Radio Capability for Paging is sent to the AMF.

NOTE:	For functions (e.g. 2 Receivers for XR devices) relying on UE Radio Capability for Paging to work properly, it is expected that NG-RAN nodes provide UE Radio Capability for Paging specific to these functions when supported by NG-RAN as specified in TS 38.413 [34].

-	The UE Radio Capability for Paging is maintained in the core network, even during AMF reselection and is stored in the UCMF alongside the UE Radio Capability information associated to a UE Radio Capability ID.

b)	Information On Recommended Cells and RAN nodes For Paging:

-	Information sent by the NG-RAN and used by the AMF when paging the UE to help determining the NG RAN nodes to be paged as well as to provide the information on recommended cells to each of these RAN nodes, in order to optimize the probability of successful paging while minimizing the signalling load on the radio path.

-	The RAN provides this information during N2 release.

# * * * End of changes * * * *