3GPP TSG-SA2 Meeting #172	S2-2511176

Dallas, USA, 17th Nov 2025 – 21st Nov 2025	(revision of S2-2510397, 11116)

Source:	LG Electronics, InterDigital, Nokia

Title:	[General aspects] Architecture Requirements for 6G

Document for:	Approval

Agenda Item:	20.6.0

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract of the contribution: This paper proposes Architectural Requirements.

Discussion

Following pCRs are merged

| 20.6.0 | S2-2510121 | P-CR | Approval | 23.801-01: [General aspects] Architectural Requirements | vivo |
| --- | --- | --- | --- | --- | --- |
| 20.6.0 | S2-2510397 | P-CR | Approval | 23.801-01: [General aspects] Architecture Requirements for 6G. | LG Electronics, InterDigital |
| 20.6.0 | S2-2510472 | P-CR | Approval | 23.801-01: [General aspects] Architecture Requirements for 6G | CSCN |
| 20.6.0 | S2-2510593 | P-CR | Approval | 23.801-01: [General aspects] Update on Architecture Requirements for 6G | OPPO |
| 20.6.0 | S2-2510630 | P-CR | Approval | 23.801-01: [General aspects] Architecture Requirements for 6G | ZTE |
| 20.6.0 | S2-2510802 | P-CR | Approval | 23.801-01: [General Aspects] Architectural requirement related to user identities | Philips International B.V., Interdigital Inc. |
| 20.6.3 | S2-2510293 | P-CR | Approval | 23.801-01: [WT3.1, AI for 6G] Predictable 6G Architecture when using AI4NET. | Boost Mobile Network, EchoStar, AT&T, China Mobile, Cisco Systems, Deutsche Telekom, SK Telecom, Verizon |

Way forward discussion

1.	Whether to keep following requirements/bullets in the Architectural Requirement or capture them in Architectural Principle without using "shall" or "should"?

2.	6G System should consider the follow design principles such as: cloud nativeness, improve sustainability and energy efficiency, robustness and resiliency.

Move to Architectural Principle?

4.	6G System should minimize inter dependencies between NFs.

Move to Architectural Principle?

12.	6G System shall support to deliver predictable outcome for all services offered by 6G System (e.g. when a user wants internet connection, the user gets internet connection).

Move to Architectural Principle?

2. Whether to add "dynamic NF deployment" to bullet 2?

2.	Consider the following aspects design principles such as: cloud nativeness, improve sustainability and energy efficiency, dynamic NF deployment, robustness and resiliency.

3.	Whether to add following new requirements?

13.	Support transmission of IP packets, non-IP PDUs and Ethernet frames.

Remove considering that this is part of 5G feature table?

14.	Support service and session continuity.

Remove considering that this is part of 5G feature table?

15.	Supports adapting 6G services to individual users (i.e. humans, applications) in addition to adaptation per UE or per subscription.

How to handle this? This was never discussed as a part of architecture requirements.

4.	Whether to add the following NOTE in Architectural Requirements?

NOTE:	The system level aspects in support of the features defined in RAN WGs (e.g., energy related features) should be considered, if any.

Proposal

It is proposed to agree the following changes into TR 23.801-01.

# * * * * Start of 1st Change (All New Texts) * * * *

4.2	Architectural Requirements

The followings are architectural requirements of 6G System:

The 6G System shall

1.	Support roaming.

3.	Support multi-vendor interoperable interfaces.

The 6G System should

2.	Follow design principles such as: cloud nativeness, improve sustainability and energy efficiency, robustness and resiliency.

4.	Minimize inter dependencies between NFs when introducing a new feature or procedure for an NF.

The architectural requirements apply to all WTs and corresponding key issues in this study.

NOTE:	The system level aspects in support of the features defined in other WGs can be considered, if any.

# * * * * End of Changes * * * *