3GPP TSG-WG SA2 Meeting #172	S2-2511302

Dallas, TX, USA, 17 – 21 November 2025            (revision of S2-2510948, S2-2510907, S2-2510877, S2-2510813)

Source:	Apple, OPPO

# Title:	KI#1: Interim Conclusions

# Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract of the contribution: This contribution proposes interim conclusions for KI#1.

# 1. Discussion

This contribution proposes interim conclusions for KI#1: Identification of SMS to Emergency Response Centre and Emergency Service Type.

# 2. Text Proposal

It is proposed to add this solution to TR 23.700-65.

# * * * * First change * * * *

# 4	Architectural Assumptions and Requirements

## 4.1	Architectural Assumptions

Editor's note:	This clause documents the common architecture assumptions identified for the study.

-	The architectures for SMS over NAS (see TS 23.272 [7], TS 23.401 [5], TS 23.501 [2], TS 23.502 [3]) and SMS over IP (see TS 23.204 [8]), and the architectural principles of Emergency Services (see TS 23.167 [9]) will be used as a basis.

-	SMS over IP can be applied for both 3GPP access and non-3GPP access (either over EPS or 5GS).

-	The Short Message Service to Emergency Response Centre shall be supported also for roaming UEs, where roaming subscribers’ SMS is routed to a local PSAP, according to the local regulations of the UE’s location.

-	For SMS over NAS over EPS, 3GPP access (including TN and NTN) is in scope.

-	For SMS over NAS over 5GS, 3GPP access (including TN and NTN) and non-3GPP access are in scope.

-	UEs in limited service state are not in scope of this document.

-	Short Messages initiated by the Emergency Response Centre in response to a Short Message to the Emergency Response Centre sent from the UE are treated as regular Short Messages.

-	Short Messages can be routed to different Emergency Response Centres, depending on the type of emergency service.

-	The identification of the type of emergency service (police, ambulance, fire brigade, etc.) is based on the same mechanism used for emergency calls (see TS 23.167 [9]).

-	This study builds upon requirements specified in clause 36 of TS 22.101 [6]. For SMS over IP emergency routing, existing IMS emergency services architecture in TS 23.167 [9] shall be leveraged.

-	The payload structure of the SMS to Emergency Response Centre should be consistent across SMS over NAS and SMS over IP transport mechanisms to ensure uniform PSAP processing capabilities.

Editor’s note:	Whether the delivery of SMS to Emergency Response Centre shall be exclusively via an SMSC is FFS.

-	Solutions need not cater to solve the problem of UE non-detectable SMS to Emergency Response Centre.

# * * * * Second change * * * *

# 8	Conclusions

Editor's note: This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.x	Interim conclusions for KI#1

Editor's note: This is not an exhuastive list containing all the conclusions necessary to conclude KI#1.

1.	The serving PLMN provides the emergency number list and associated emergency service types to the UE during Registration/Attach or IMS registration procedures.

2.	The UE detects the SM to Emergency Response Centre based on the used emergency number matching a locally-configured number in the UE or a number from the network-provided emergency number list.

3.	For sending a SM to Emergency Response Centre, the UE provides:

a)	the local emergency number.

# * * * * End of change * * * *