SA WG2 Meeting #172	S2-2510012

17 – 21 November, 2025, Dallas, US	(Revision of S2-2509819)

Source:	Ofinno

Title:	[WT#1.1, all topics] Work task #1.1 Support of 6G Control signaling

Document for:	Approval

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper proposes new KIs associated with WT#1.1 to support control signalling for 6G.

# 0. Background

At SA2 pre-meeting CC #3, SA2 discussed proposals on updating WT#1.1.

# 1. Discussion on Definition

In 3GPP, the term ‘NAS’, ‘user plane’ and ‘control plane’ have been used frequently for a long time. When these words are used in 3GPP discussion, the meaning of these words are well understood, based on the agreed architecture and procedures. However, for discussion for 6G, as various proposals are submitted for enhancement of NAS. We need to make sure whether the meaning is clear for 6G.

Until the work CUPS at the later phase of 4G EPS, a node (e.g., S-GW, P-GW) supported both control plane signalling (e.g., PCO with UE) and user data delivery.

During SA2 CC, companies mentioned following in TS 24.501.

## 4.1 Overview The non-access stratum (NAS) described in the present document forms the highest stratum of the control plane between the UE and the AMF (reference point "N1" see 3GPP TS 23.501 [8]) for both 3GPP and non-3GPP access.

NAS is described in terms of node (“between UE and AMF”) and interface (“N1”). Node/Interface is yet to be defined for 6G.

Observation 1: “NAS” description in TS 24.501 is dependent on NF and Interface, which will be defined during solution phase.

In 5G, following cases make it difficult:

- So-call ‘C-plane’ can be used for delivery IoT data. “user data” delivery itself can not be used as a criterion of CP vs UP.

- “RQI” is transported between UPF and UE. This is used for session management.

- Configuration of LCS UP delivery is done with signalling with SMF, and data for LCS is delivered via UPF. The borderline is not clear, because all are signalling and control information.

- “PMP” protocol is used between UE and UPF. PMP protocol is used for session management and policy update.

- Overlay/Underlay network architecture. E.g., in this case, a NAS message is transported via UP of another network. In this case, a NAS message is transported over IP protocol, over some access legs.

- For secondary authentication, PDU session messages are exchanged to transport authentication data. This data further uses N4 signalling, and UPF-AF path. These authentication data is for session management (authentication) itself. Similar example may be UAS authentication.

Observation 2: It is difficult to draw a clear boundary between CP and UP.

There are two approaches for definition of CP and UP:

- Alt.1: Which path is used for transport of information. Using 5G as an example:

  - if an information (e.g., ‘CIOT’) goes through via AMF, CP is used.

  - if an information (e.g., ‘RQI’, “PMP unavailability report”) goes through via UPF, UP is used.

- Alt.2: What is transported:

  - if an information (e.g., ‘CP CIOT data’) is ‘application’ data, the path/method used for delivery is UP.

  - if an information (e.g., ‘RQI’, “PMP message”) is not associated with ‘application’ data, the path/method used for delivery is CP.

Observation 3: CP/UP can be categorized either using ‘what information’ or ‘via which node’.

There are proposals to introduce ‘new plane’. Then, it is necessary to make clear what is meant by ‘control plane’, ‘user plane, and ‘new plane’.

At least, before the end of 6G study, definition of ‘NAS’, ‘control plane’ and ‘user plane’ needs to be re-checked.

Proposal 1: Clarify ‘user plane’, ‘control plane’ and ‘new plane’, during solution phase of 6G study.

# 2. Discussion on WT

For operator services, there are different types of control information.

For example, in IMS call/SMS, control signalling for these services is not solely transport via a PDU session.

- First phase control information: Signalling (e.g., Voice of PS domain support indication, SMS registration indication) is delivered during registration procedure (e.g., from AMF to UE). For SM signalling, P-CSCF address is delivered via a PDU session message.

- Second phase control information: Once PDU session is established, controlling signalling for IMS registration (e.g., Registration, INVITE) is delivered inside the PDU session.

Another example is LCS-UPP. Configuration of LCS-UPP (e.g., address of LCS UP delivery) is send via CP.

Thus, there is no need to limit transport of controlling signalling for operator service, to only one of CP, UP, or new plane.

Proposal 2: Control signalling for operator service can use both CP and/or UP and/or new plane.

# 2. Proposal

It is proposed to agree on:

Proposal 1: Clarify ‘user plane’, ‘control plane’ and ‘new plane’, during solution phase of 6G study.

Proposal 2: Control signalling for operator service can use both CP and UP or new plane.

**** First Change (relative to version of CC#3) ****

# Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Non-Access Stratum: The Non-Access Stratum (NAS) forms the highest stratum of the control plane between the UE and the 6G CN.

# Annex A.X. WT#1.1 Study the support for control signalling for 6G System

1.	Study the support for control signalling for 6G System, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality with minimal or no impact to other non-access stratum functionalities.

b.	Void

NOTE 1: 	It is assumed that this WT covers 6G System procedures including functionalities such as mobility management, session management, NAS transport and UE NAS identifiers.

NOTE 2: For the above bullet a, target would be not to impact other non-access stratum functionalities when introducing new non-access stratum functionalities. If this is not possible, objective is to minimize impact.

2. 	Study the support for control signalling for 6G System, including at least the following:

a.	Whether and how to support generic mechanisms (e.g. service discovery, service authorization, transport mechanism) for UE to Core Network interaction to support operator services.

NOTE 3: This WT1.1 bullet 2a can include any transport mechanism such as using NAS, UP, and/or new plane. WT1.1 bullet 2a can have dependency on other work tasks e.g. WT3, WT4, WT5, WT6, WT1.1 bullet 1a.

NOTE 4: Definition and difference of CP, UP and new plane will be clarified during the study.

# Key Issue #1.1 Study the support for control signalling for 6G System

Key Issue #1.1.1

1.	Study the support for control signalling for 6G System, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality with minimal or no impact to other non-access stratum functionalities.

NOTE 1: 	It is assumed that this key issue covers 6G System procedures including functionalities such as mobility management, session management, NAS transport and UE NAS identifiers.

NOTE 2: For the above bullet a, target would be not to impact other non-access stratum functionalities when introducing new non-access stratum functionalities. If this is not possible, objective is to minimize impact.

Key Issue #1.1.2

2. 	Study the support for control signalling for 6G System, including at least the following:

a.	Whether and how to support generic mechanisms (e.g. service discovery, service authorization, transport mechanism) for UE to Core Network interaction to support operator services.

NOTE 3: This KI1.1 bullet 2a can include any transport mechanism such as using NAS, UP and/or new plane. KI1.1 bullet 2a can have dependency on other key issues defined for e.g. WT3, WT4, WT5, WT6, WT1.1 bullet 1a.

NOTE 4:	Interdependency of NFs within 6G CN in the procedure is to be minimized.

NOTE 5: 	System Procedures will include interaction between RAN-CN, interaction within CN NFs. The signaling interaction between RAN-CN need coordination and alignment with RAN3.

NOTE 6: Interworking and migration aspects, where applicable, are covered by key issues for WT#2. Impact to IWK due to features introduced by this key issue will be covered by this key issue.

**** End of Changes ****