SA WG2 Meeting #172	S2-2510044

17 – 21 Nov, 2025, Dallas, US	(revision of S2-2509836)

Source:	LG Electronics

Title:	[WT#3.1, AI for 6G Network] WT#3.1 update & new key issue proposal

Document for:	Approval

Agenda Item:	20.6.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper proposes two updates to the WT#3.1 on top of S2-2509836. Especially, it proposes to add two new bullets to cover bi-directional AI-driven collaboration between the 6G network and the UE.

1. Justifications

1.1. Justification for new bullets

The baseline for WT#3.1 (AI for 6G architecture) in S2-2509836 outlines the scope for studying the use of AI within the 6G network. This scope covers how the 6G CN can fulfil requests from UEs/AFs and access external capabilities (e.g., from an AF).

However, SA1 TR 22.870 (Study on 6G Use Cases and Service Requirements) has identified use cases that require enhanced AI-driven collaboration, which is based on bi-directional information directions between the 6G network and the UE.

- 6.4		Use case on system performance optimisation using AI

End users’ behavioural context may be gathered both at the device and at the network and used in a complementary fashion to optimize communication. For example, the network may use historical information from location updates and location measurements to infer the user’s current position and environmental conditions. Additionally, by leveraging AI capabilities in the UE, the UE may identify scenarios where current and future location information are available and useful and may provide this information to the network.

<< Some parts omitted>>

5. In certain cases, the UE may not be able to share user location information with the network due to privacy considerations or rapid changes in location. However, the UE can still collaborate with the network in setting location-related parameters. This can be done via the UE recommending parameter settings to the network, or the network may provide a range of parameters to the UE and based on its AI capabilities and user context, the UE may select a desired option. Ultimately, the network and UE would collaborate to select the appropriate location parameters that support the communication service efficiently.

- 6.32	Use case on disaster rescue planning enabled by network AI Agents

Operator A has deployed network AI Agents in its 6G network and is able to provide intent based services enabled by network AI Agents as well as other related 3GPP services (sensing, AI inference, model training, computing, connection, etc.) to users.

The rescue team in region Z, which owns multiple rescue robots, has subscribed to the services enabled by network AI Agents provided by Operator A.

<< Some parts omitted >>

7. The 6G network sends the information (e.g. sensing results, AI inference results) to the rescue robots, which then conducts rescue operations based on the received information.

8. The rescue robot provides feedback on actual rescue situations (such as successful rescues, failed rescues, and the reasons for failure) to the network AI Agents.

9. The network AI agent quickly performs self-reflection based on the feedback (e.g. collecting new data, fine-tuning the model, etc.).

These SA1 scenarios represent a bi-directional collaboration model:

1. Network-to-UE: In the context of AI for 6G Network, SA1 scenarios establish the architectural need for a Network-to-UE information direction, which provides the basis for collaboration. We propose to study how AI capable entities in the 6G CN can provide specific AI model related information (e.g., network-generated AI recommendation, AI model inference results) to the UE. This Network-to-UE information direction is not explicitly covered in the current WT#3.1 scope.

2. UE-to-Network: For the AI for 6G Network to optimize its operations (e.g., network behaviour for supporting mobility, QoS), the required collaboration(or working together) between CN and UE implies the network AI can obtain capabilities and/or information from the UE, such as UE-provided context (e.g., the UE’s capabilities, model status, AI model inference result). The current WT#3.1 scope (bullet 4) only addresses accessing capabilities from an AF.

These two directions (Network to UE and UE to Network) are distinct requirements for bi-directional collaboration. In addition, they form a complete feedback loop, allowing for a much more dynamic and efficient AI-driven network optimization.

To provide a complete architectural study for this UE-Network AI collaboration, the WT#3.1 scope should be updated to cover both directions of this interaction.

1.2. Justification for re-numbering bullet 1

Additionally, this contribution proposes a minor correction to the numbering of item 1 for logical clarity. In the current baseline, bullet 1.1 deals with the requests from the UEs and AFs include intent, while bullet 1.2 deals with the constraints on the use and expression of intents. Therefore, it is logically appropriate for bullet 1.2 to be a sub-bullet of 1.1. This structural change will improve the readability and logical flow of the document. It is proposed to re-number 1.2 as 1.1.1 and adjust the subsequent bullets (1.3 and 1.4) accordingly.

2. Proposals

This contribution proposes some updates to WT#3.1 to fully capture this bi-directional scope:

1. Add a new bullet 10 to study how 6G CN AI entities provide network capabilities and/or information to the UE.

2. Add a new bullet 11 to study how 6G CN AI entities obtain capabilities and/or information by the UE.

3. Re-number 1.2 as 1.1.1 and adjust the subsequent bullets (1.3 and 1.4) accordingly.

* * * * Start of 1st Change * * * *

on top of “S2-2509836”

Annex A.3. WT#3 Scope

WT#3: Study how to support and enable use of AI in 6G (e.g. AI agent, AI framework).

NOTE 1: 	The term AI agent refers to the general concept of agents autonomously performing tasks on behalf of users, systems, and/or applications. As the SA1 work is still in progress, adapting the definition of AI agent from SA1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

WT3.1:	AI for 6G architecture

The AI for the 6G architecture shall be multi-vendor interoperable, reliable and sustainable.

Study whether and how to provide an architecture for AI to fulfil the following:

1.	enable the 6G CN to fulfil requests from UEs and AFs leveraging AI technologies in the 6G CN (e.g. AI agent), using 6G CN functionality available in the network:

1.1	when the requests from the UEs and AFs include intent

NOTE 1:	An intent refers to expectations including requirements, goals, conditions, and constraints, without specifying how to achieve them. Intent cannot remain unspecified. What intent means in SA2 specifications will be determined by the study.

1.1.1	determine the constraints on the use and expression of intents to be processed and interpreted unambiguously by the AI capable entities in the 6G CN and define the mechanisms required to support these constraints, if any.

NOTE 2:	It is not required to support intents in a network.

NOTE 3:	It is assumed it is not required for the MT stack of UE to produce nor understand the intent.

1.2 when the requests from the UEs and AFs do not include intent (e.g. like registration request in previous generations)

NOTE 4:	The MT stack of UE is assumed to be agnostic to whether the network uses AI capable entities (e.g. AI agent, AI-enabled NFs) to address UE requests not including intent.

1.3 design 6G system procedures in 6G CN so that parts of these procedures can be composed dynamically using AI capable entities.

NOTE 5:	This should not result in duplicated procedures in 3GPP specifications.

NOTE 6:	It is assumed that the 6G study will still define 6G system procedures for all 3GPP features, without requiring AI capabilities

2.	enable closed-loop operations and learning techniques such as reinforcement learning, in 6G CN

3.	enable entities in 6G CN to access network AI capabilities provided by 6G NFs

4.	enable AI capable entities in 6G CN to access trusted external capabilities provided by AF

5.	enable the monitoring of the performance of AI capable entities in 6G CN

6.	enable the operator to dynamically control the use of AI capabilities in its 6G CN, i.e. support different operator-configurable levels of autonomy based on operational needs, including the option to not use any AI capabilities.

7.	support roaming scenarios, e.g. how visited networks using or not AI technologies can interwork with home network using or not AI technologies.

8.	enable NFs of the 6G CN to have AI/ML capabilities, e.g. ML model provisioning, inferencing, training and monitoring

9.	enable the 6G CN AI architecture to ensure interoperability with 5G CN AI architecture if needed, for the purpose of maintaining a consistent service for UEs across 5G and 6G

10. enable AI capable entities in 6G CN to provide network capabilities and/or information (e.g., AI model related information, such as AI inference results) to the UE

11. enable AI capable entities in 6G CN to obtain capabilities and/or information (e.g., AI model related information, such as the UE’s AI capabilities, feedback for model fine-tuning) provided by UE

NOTE 7: It is assumed that operators can decide whether to deploy AI capabilities in their network.

NOTE 8:	Aspects related to SA3, SA5, SA6 will be coordinated with each group respectively during the study.

NOTE 9:	Further alignments with SA1 consolidated requirements may be needed.

NOTE 10:	This sub work task does not cover AI agents in UE.

* * * * Start of 2nd Change * * * *

(All new text)

5.X. Key Issue #X: AI for 6G Network

Copy from WT#3.1 above

* * * * End of Change * * * *