3GPP TSG-WG SA2 Meeting #172	S2-2510236

November 17 – 21, 2025, Dallas, TX, USA

Source:	Ericsson

Title:	KI#1 conclusion update: Voice media transport

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract: This contribution proposes an updated interim conclusion for KI#1 in TR 23.700-19, v1.1.0, to use one PDN connection with 2 EPS bearers for IMS and also to consider SA1/SA4 answers to evaluate whether IP or non-IP is selected.

# Discussion

In SA2#171, it was agreed to transport the voice media via user plane (DRB+S1-U).

With existing specification 23.401 clause 4.10, there shall not be a mix of CP and UP for SGi PDN connections.

All the SGi PDN connections of a UE shall either use S11-U or S1-U at any point in time.

Though the above limitation applies on PDN connection level, the reasoning discussed in S2-162979/C4-163186 applies on bearer level, quoted below:

Besides, support of concurrent S1-U and S11-U bearers would cause unnecessary complexity to the SGW by moving away from the existing EPS design principle that an SGW sees either all the bearers established or none for an LTE user. With concurrent S1-U and S11-U bearers, scenarios would exist where the SGW would start seeing a UE in LTE with only a subset of its bearers “user plane connected”, e.g. when a UE in idle mode starts sending data over NAS for the SGi PDN connection that would be pinned to the CP, or upon switching some SGi PDN connections from CP to UP (during which the S11-U bearers of the PDN connection pinned to the CP would need to remain established), or e.g. upon receipt of a GTP Error Indication from the eNB which would lead to tear down the S1-U bearers only.

Support of concurrent S1-U and S11-U bearers would also require additional GTP protocol extensions, as the current S11 design does not consider such support, and further analysis on potentially other CT4 impacts (e.g. path management and restoration procedures).

So for the above reasons, CT4 agreed to NOT support concurrent S1-U and S11-U bearers. As a result, if the UE has multiple SGi PDN connections, all these SGi PDN connections shall either be "pinned" or "not pinned" to the CP.

Since the architecture assumption of this study assumes that only SGi PDN connection is considered, it is appropriate to transport the SIP signaling via user plane (DRB+S1-U) as well, to avoid the unnecessary complexity of concurrent S1-U and S11-U bearers.

Proposal 1: It is proposed to transport SIP signaling via user plane (DRB+S1-U).

When CIoT introduced in Rel-13, it was decided to not support dedicated EPS bearer. Copy the reasoning from the CR S2-176669 below.

| Reason for change: | NB-IoT provides access to network services using physical layer optimized for very low power consumption and (very) low data rates. In case the UE supports S1-u data transfer, NB-IoT RAT can support maximum two DRBs. In §4.3.5.9 it is stated that “inter-RAT mobility to/from the NB-IoT RAT is not supported, and GBR bearers are not supported in the NB-IoT RAT, hence the UE should not include the "Voice domain preference and UE's usage setting" IE at Attach Request or Tracking Area Update Request on the NB-IoT RAT.” Said otherwise, NB-IoT provides no support for IMS voice. In addition, given data rate limitation and the restricted number of DRBs it is debatable whether for a given UE QoS differentation would be useful for non-GBR bearers. Furthermore, the use of dedicated bearer (IP), using all available DRBs, will inevitably yield problematic situations such as preventing transfer of other data when a 2nd PDN connection would need UP data transfer, deactivating ongoing data transfer when the SIM requires a BIP connection, etc. |
| --- | --- |

The above listed reasons are no longer applicable for new use cases, such as IMS voice over NB-IoT GEO satellite access:

- Neither SIP signaling, nor IMS voice media has the same characteristics as small and low-rate user data. (see traffic model for Cellular IoT in TR 23.720 clause 4.3). Transporting small/low-rate user data over dedicated bearer adds substantial signaling overhead relative to the tiny, infrequent user payloads. But this is not the case for IMS voice.

- According to the RAN2 LS reply R2-2507784, it is technically feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20. The problem of consuming all the available DRBs when enabling dedicated bearer will thus be gone.

For IMS voice, SIP signaling and voice media demand different treatment in the data radio bearer. In the existing VoLTE solution, SIP signaling in default bearer utilizes RLC Acknowledged mode (AM) because it prioritizes reliability over latency, while the voice media in dedicated bearer is using RLC Unacknowledged mode (UM) because real-time voice traffic can tolerate some packet loss but not retransmission delays. Considering the GEO satellite long propagation delays, it is important to retain the principle of using different radio bearers (EPS bearers) for SIP signaling and voice media.

Mixing SIP signaling and voice media in one EPS bearer may bring additional challenges and impact with regards to Lawful Intercept, since the existing solution assumes that SIP signaling and voice media is carried via different EPS bearers. The SGW in VPLMN performs the inspection for roaming subscribers accessing IMS voice service via S8HR. The SIP signaling in the default bearer of all inbound roaming subscribers is duplicated by SGW and sent to LMISF. LMISF checks if the SIP signaling involves a user under LI target. If it is, LMISF instructs SGW to duplicate also the dedicated bearer data (i.e. voice media) towards LMISF. If SIP signalling and voice media are carried on a single EPS bearer a new solution for LI would need to be devised.

Given the above considerations, the restriction on using dedicated bearer does not apply to IMS voice service. IMS voice over GEO NB-IoT requires two EPS bearers, and it is therefore practical to reuse the existing framework that supports multiple EPS bearers within a single PDN connection. Otherwise, coordinating two separate PDN connections for one service would introduce unnecessary complexity and additional signalling.

Proposal 2: It is proposed to use 2 EPS bearers of one PDN connection for transporting SIP signaling and voice media.

There is outstanding LS from SA2 to SA1 and SA4 S2-2507636. See below.

- Question 4 (To SA4): What are the essential RTP header fields for the minimum information that needs to be provided in the RTP header for IMS voice over NB-IoT NTN?

- Question 5 (To SA4): Does SA4 have any views on how to reduce the RTP header overhead for IMS voice over NB-IoT NTN?

- Question 11 (To SA1): Can SA1 confirm, whether support for more than one IMS voice calls and support for DTMF is required for IMS voice over NB-IoT NTN?

The original motivation of non-IP based solution to transport voice media is to reduce protocol overhead, while the answer to these questions above plays an important role to determine how large the overhead will be for non-IP based solution.

Also, based on the discussion paper S2-2510237, SA4’s progress on voice codec and the SID behaviour should be taken into account when selecting IP and non-IP solutions, as it determines whether large ROHC packet is needed or not during a normal talk/silent transition.

Proposal 3: It is proposed to add an editor’s note in the conclusion to indicate that the selection of IP and non-IP based solution for voice media should consider the SA1 and SA4’s input.

* * * Start of change * * * *

## 8.1	Interim conclusions for Key Issue #1

Editor's note:	Other interim conclusions for Key Issue#1 is FFS.

The following interim conclusions for Key Issue #1 Support of IMS voice call over NB-IoT via GEO satellite connecting to EPC are made:

-	The voice packets shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U.

-	The voice packets shall be transported in the same PDN connection as SIP signalling, but in a different EPS bearer.

Editor’s note: The selection between IP and non-IP based solution for transporting voice media should consider SA1 requirements (e.g. multi-call support, DTMF) and SA4’s progress on Ultra Low Bit Rate Speech Codec.

* * * End of change * * * *