SA WG2 Meeting #172	S2-2510262

13 – 17 November, 2025, Dallas, USA

Source:	Ericsson

Title:	Key Issue #2: Enhancements to capability exposure framework for eMMTel services

Document for:	Approval

Agenda Item:	20.7.1

Work Item / Release:	FS_NG_RTC_Ph3_ARC / Rel-20

Abstract of the contribution: This contribution proposes a new solution for FS_NG_RTC_Ph3. The solution addresses the expose of IMS capability for eMMTel services (voice/video).

# 1. Discussion

This pCR proposes a solution to the Key Issue #2:”….how to enhance IMS capability exposure framework to expose IMS capability for eMMTel services (voice/video) and related events besides Data Channel, supporting IMS session establishment, modification and release to enable eMMTel services that may require enhancing audio/video services with DC capabilities” by specifying the addition of MMTel media (e.g. audio, video) to the relevant existing service operations to align with the current specification.

# 2. Text Proposal

It is proposed to capture the following changes into TR 23.700-56.

# * * * * all new change * * * *

# 6	Solutions

## 6.X	Solution #X: KI#2 - Enhancement of IMS capability exposure framework

### 6.X.1	High level Solution principles

This solution addresses KI#2.

The main principles of the solution are to align with and expand on existing specified functionality:

- to add exposure capabilities to introduce support for establishment, modification and release of MMTel media (e.g., audio & video).

- addition of MMTel media (e.g., audio & video) exposure capabilities is in the context of data channel (e.g. enhancing audio/video services with DC capabilities or enhancing DC services with audio/video services capabilities).

### 6.X.2	Description

The solution specifies enhancements to the IMS exposure framework ensuring that exposure capabilities for session establishment, modification and release are media agnostic, i.e., other MMTel media (e.g., audio, video) are also supported in addition to the existing specified DC media.

The solution caters for the addition of MMTel media (e.g., audio, video) to the existing specified relevant service operations. The proposal builds on aligning with already specified functionality where the establishment, modification and release of MMTel media (e.g., audio, video) is supported in the following services:

- Nimsas_MediaControl_MediaInstruction service operation (see TS 23.228 AA.2.4.3.2)

- Nmf_MRM_Create service operation (see TS 23.228 AA.2.5.2.2).

3GPP stage 3 has also defined support for audio / video (see TS 29.175, for example chapter 6.1.6.3.4).

The extended functionality will for example enable a DC AS to add, modify or release MMTel media (e.g. audio/video) to / from a standalone ADC or enable a person / user to add, modify or release MMTel media (e.g. audio/video) towards DC AS (for DC enabled services).

The IMS AS services that are amended to indicate support of MMTel media (e.g. audio / video) in addition to the existing DC media are:

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nimsas_ImsSessionManagement | Create | Request/Response | NEF, Trusted AF |
|  | Update | Request/Response | NEF, Trusted AF |
|  | Delete | Request/Response | NEF, Trusted AF |
|  | Notify | Subscribe/Notify | NEF, Trusted AF |

Support for MMTel media (e.g. audio/video) shall also be included in the yet undefined services on DC3 & DC4 reference point.

### 6.X.3	Procedure

#### 6.X.3.1  	DC AS initiated IMS audio session addition with a standalone A2P application data channel

Figure 6.X.3.1-1 depicts a signalling flow diagram initiated by DC AS for adding MMTel media (e.g., audio) to a standalone Application Data Channel A2P application data channel with UE A. This signalling flow extends the flow described in TS 23.228 AG.2.2.2

Figure 6.X.3.1-1: Establishment procedure of an adding an IMS audio session with a standalone A2P application data channel

0.  The DC AS decides to add an IMS audio session with an already established standalone A2P application data channel with UE A.

1.	The DC AS sends a Nnef_ImsSessionManagement_Update request to the NEF requiring to add an IMS audio session as addition to an already created standalone A2P application DC with UE A.

The DC AS sends the session ID (of previously created ADC) and UE A’s IMPU in addition to the media operation set including the parameters for the media type set as “Audio” to NEF. The DC AS may include a Notification Target Address and Correlation ID to the Nnef_ImsSessionManagement_Update request to be notified for the progress of the session creation.

2.	If the NEF authorizes the request, the NEF queries the HSS with Nhss_ImsUECM_AsInfoGet service operation to retrieve the IMS AS instance serving UE A.

3.	The NEF sends a Nimsas_ImsSessionManagement_Update request to the IMS AS-1 requiring to add an IMS audio session. The request includes the session ID (of previously created ADC) and UE A’s IMPU in addition to the media operation set. The media operation set includes the parameters for the media type set as “Audio”, binding info & the MDC2 media endpoint. The NEF may include a Notification Target Address and Correlation ID to the Nimsas_ImsSessionManagement_Update request to be notified of the session create progress.

4.	The IMS AS-1 validates the user subscription data and checks the IMS audio & DC capability of UE A to determine whether the request should be notified to DCSF-1. If UE A does not have subscription of IMS data channel and MMTel media (e.g., audio), then the request shall be rejected with appropriate cause and the subsequent steps are skipped. If the IMS AS-1 allows the request to proceed, it returns a successful Nimsas_ImsSessionManagement_Update response to the NEF.

5.	The NEF returns a successful Nnef_ImsSessionManagement_Update response generated by the IMS AS-1 to the DC AS.

6.	The IMS AS-1 sends a Nimsas_SessionEventControl_Notify request to the DCSF-1 with event set to ExternalSessionCreateEvent. The notification includes the necessary parameters based on the information extracted from the media operation set (including MediaInfoList: MediaType - “Audio”) received in step 3 and other stored parameters in the IMS AS-1 if applicable.

7.	After receiving the session event control notification, the DCSF-1 determines the policy how to process the A2P media audio establishment request based on the related parameters in the notification and/or DCSF-1 service specific policy.

8.	The DCSF-1 determines that the A2P media audio requires the DCAS audio to be anchored in MF-1.

9.	The DCSF-1 instructs the IMS AS-1 to set up the A2P audio by sending a Nimsas_MediaControl_MediaInstruction request.

10.	Based on the instruction from DCSF-1, the IMS AS-1 interacts with the MF-1 to allocate audio media resource towards UE A and MDC2 media resource towards the DC AS.

11.	The IMS AS-1 returns the Nimsas_MediaControl_MediaInstruction response to the DCSF-1.

12. DCSF send a connection (audio) establishment request offer to DCAS including the MDC2 media resource.

13. DCAS accepts the request and provides MDC2 media resource (audio).

14. The DCSF-1 instructs the IMS AS-1 to set up the audio by sending a Nimsas_MediaControl_MediaInstruction request.

15.	Based on the instruction from DCSF-1, the IMS AS-1 interacts with the MF-1 to request update of DCAS side media resource information.

16. The IMS AS-1 returns the Nimsas_MediaControl_MediaInstruction response to the DCSF-1.

17. The DCSF-1 returns the Nimsas_SessionEventControl_Notify response to the IMS AS-1.

18.	The IMS AS-1 generates a re-INVITE request in which the SDP offer contains the media information of the audio A2P application required by the DC AS, binding information and the session ID.

The IMS AS-1 sends the re-INVITE request to UE A.

19.	DC media negotiation may be completed through 18X/PRACK/200 OK(PRACK)/200 OK(INVITE)/ACK procedure.

20.	The IMS AS-1 requests MF-1 to update UE-A side media (Audio) resource information.

21.	UE A established audio with DC AS.

22. If the NEF included a Notification Target Address in the Nimsas_ImsSessionManagement_Update request in step 3, the IMS AS-1 notifies the NEF for the progress of the audio session creation with the Nimsas_ImsSessionManagement_Notify request.

23If the DC AS included a Notification Target Address in the Nnef_ImsSessionManagement_Update request in step 1, the NEF notifies the DC AS for the progress of the session creation with the Nnef_ImsSessionManagement_Notify request.

24. The DC AS returns a Nnef_ImsSessionManagement_Notify response to the NEF.

25. The NEF returns a Nimsas_ImsSessionMangement_Notify response to the IMS AS-1.

#### 6.X.3.2	Person-to-Application (P2A) MMTel media (audio) and Application Data Channel Setup

Figure 6.X.3.1-2 depicts a signalling flow diagram for adding an IMS audio session in a in a person to application use case. This signalling flow extends the flow described in TS 23.228 AC.7.2.2-1 where a P2A ADC is setup.

Figure 6.X.3.2-1: Person-to-Application (P2A) audio & application Data Channel set up Signalling Procedure

The steps in the call flow are as follows:

0-3.	As in the base signalling procedure (AC.7.2.2-1), steps 0-3 of clause AC.7.2.1 applies. As outlined in step 0 (red text) it is assumed that DC AS has provisioned triggers in IMS AS to be informed via DCSF of an audio (setup) media request.

NOTE 1:	The SIP re-INVITE (step 1) is initiated by the served users UE (in this case UE#1).

4.	After receiving the session event notification, the DCSF determines the policy about how to process the application data channel / audio establishment request based on the related parameters (i.e. associated DC application binding information) in the notification and/or DCSF service specific policy.

5.	The DCSF determines that the added Application Data Channel / audio media of the offer takes DC Application Server as target endpoint and requires to anchor in the MF.

6.	The DCSF invokes Nimsas_MediaControl service operation to instruct IMS AS to terminate the media flow of the originating UE to MF. The instruction also includes information to be consumed by the MF that the audio media shall be relayed via the MDC2 interface.

7.	The IMS AS invokes Nmf_MRM_Create(List of Media Termination Descriptors) service operation to instruct MF on application data channel / audio establishment and data channel / audio media resource reservation based on the DC / audio media information received from DCSF.

8.	The IMS AS notifies the MediaControl instruction control response to DCSF.

9.	The DCSF stores the media resource information and sends a P2A (audio / DC) application data channel establishment request (including the MDC2 media resource received from MF) to the DC Application Server via DC3/DC4.

10.	DC Application Server accepts the P2A application data channel establishment request, returning an MDC2 reserved media resource as answer and is prepared for UE#1 traffic through MDC2.

NOTE 2:	Details on how DCSF communicates with the DC Application Server is for study as part of KI#3.

11.	DCSF requests IMS AS to update the MF resource with MDC2 media endpoint information of DC Application Server.

12.	IMS AS updates the MF resource.

13.	IMS AS notifies the MediaControl instruction control response to DCSF.

14.	DCSF replies to the notification received in step 3.

15-16.	IMS AS sends the re-INVITE to remote network side and UE#2, via the originating S-CSCF, which does not include application data channel request in the SDP for the application data channel.

17-19:	UE#2 and terminating network returns a 200 OK response with SDP answer for audio/video.

20.	IMS AS notifies the DCSF about the successful result of the MediaChangeRequest event.

21.	DCSF replies to the notification.

22-23.	The IMS AS includes SDP answer for application data channels / audio to UE#1 in 200 OK response and sends 200 OK response to S-CSCF and P-CSCF.

24.	The originating network P-CSCF executes QoS procedure for application data channel / audio media based on the SDP answer information from the 200 OK response.

25.	CSCF returns the 200 OK response to UE#1.

26.	UE#1 send ACK to the terminating network.

27.	The application data channel / audio between UE#1 and DC Application Server is established via MF. MF forwards data channel / audio traffic between UE#1 and DC Application Server based on MDC2 media point information received in step 9 and 12.

### 6.X.4	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

DC AS:

- sending establishment, modification and release requests for MMTel Audio / Video.

- sending DC AS-side information for MDC2 configuration for Audio / Video

- receiving MF-side (audio/video) information for MDC2 configuration.

DCSF:

- receiving subscription of the event ExternalSessionCreateEvent (with MediaType=Audio/Video)

- requesting IMS AS for MDC2 (audio/Video) configuration.

- sending MF-side (audio/video) information for MDC2 configuration to DC AS,

- sending request to create DC AS-side for the MDC2 (Audio/Video).

- receiving DC AS-side information for MDC2 (Audio/Video) configuration

IMS AS:

-	receiving MDC2 (audio/Video) configuration request from DCSF,

-	instructing MF for MDC2 (audio/Video) configuration.

MF:

-	receiving MDC2 (audio/Video) configuration instructions from IMS AS,

-	performing MDC2 (audio/Video) configuration for DC AS service like e.g. translation & transcription services.

# * * * * End of changes * * *