3GPP TSG-SA WG2 Meeting #172	S2-2510347

# November 17-21, 2025 Dallas, USA	(a revision of S2-2508288)

Source:	Skylo Technologies, ViaSat, Boost Mobile Network, EchoStar

Title:	Interim Conclusion for KI#1

Agenda item:	20.1.1

Document for:	Agreement

Work Item / Release: FS_5GSAT_Ph4_ARC /Rel-20

Abstract of the contribution: This paper proposes Interim conclusions for KI#1

# 1. Discussion

TS 23.700-19 v1.1.0 has documented 23 solutions relevant to KI#1 and KI#2.

Regarding KI#1, while evaluating these solutions, the following observations and proposals are identified for system enhancements to support IMS voice service over NB-IoT via a GEO satellite connection to the EPC.

- NB-IoT Non-IP Data Delivery over User Plane (UP), as specified in TS 23.401, provides the lowest protocol overhead among other UP/CP and IP/Non-IP combinations, making it especially suitable for constrained and high-latency environments such as GEO-based NTN. It is proposed to use UP NIDD to support both media packets and voice over GEO signalling delivery for voice over GEO service to accommodate more simultaneous calls using limited IoT NTN (GEO) transmission bandwidth.

- Two EPS bearers shall be used, one for signalling and one for voice media packets, due to their different transport requirements. This separation is consistent with IMS handling of control and media and enables better traffic management and differentiated satellite resource allocation.

Taking into the account of the architecture assumptions specified in TS 23.700-19, where enhancements to NB-IoT (GEO) shall be minimized and considering that NB-IoT does not support dedicated bearers, default bearers shall be used. These two default bearers shall be established within two separate non-IP PDN connections, as one PDN connection can only contain a single default bearer.

In summary, two non-IP PDN connections are proposed, each carrying one default bearer, with one for signalling and one for media. This approach also removes the dependency on the RAN.

- IP with RoHC adds additional complexity and CPU processing for both the UE and the network. This makes it less efficient for NB-IoT, where devices are constrained by limited processing capability, power consumption, and memory footprint. ROHC is not a fit for voice over GEO ROHC’s two-party negotiation process adds several seconds to call setup and increases latency, a significant issue in GEO. ROHC's initial state introduces a large IP overhead, which isn't suitable for the narrowband channels used in IoT NTN.

# 2 Proposal

It is proposed to include the following changes in TR 23.700-19 V1.1.0

* * * First Change * * * *

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.1	Interim conclusions for Key Issue #1

Editor's note:	Other interim conclusions for Key Issue#1 is FFS.

To support IMS voice call over NB-IoT (GEO), while minimizing enhancements to NB-IoT (GEO), EPC and IMS, the following interim conclusions for Key Issue #1 Support of IMS voice call over NB-IoT via GEO satellite connecting to EPC are made:

-	The voice packets shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U.

- Separate EPS bearers shall be established for voice-over-GEO signalling and voice media traffic.

- A default bearer shall be used for voice-over-GEO signalling. Another default bearer, established within a separate PDN connection, shall be used for the delivery of voice packets.

- Voice data packets shall be delivered using the Non-IP Data Delivery (NIDD) mechanism within a Non-IP PDN connection.

- Voice-over-GEO signalling shall be delivered using the NIDD mechanism within a Non-IP PDN connection.

- In the case of Non-IP Data Delivery, point-to-point tunnelling by UDP/IP encapsulation shall be used as described in TS 23.401 sub-clause 4.3.17.8.3.3.2.

* * * End of Changes * * * *