3GPP TSG-WG SA2 Meeting #172	S2-2510566

Dallas, US, 17th Nov – 21st Nov, 2025	(revision of S2-2509796)

Source:	Huawei, HiSilicon, TNO

Title:	[WT#7, NTN] NTN specific WTs and KIs for 6G support of NTN

Document for:	Approval

Agenda Item:	20.6.7

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract: This contribution proposes the scope of WT#7 and corresponding Key Issue(s) for SA2 6G SID.

# 1.	Introduction

During SA2#171 S2-2509796 “[WT#7]: Work task and key issue for 6G satellite access” was marked as postponed and represents the latest version of the WT & KI proposals after discussions in the meeting.

The contents of S2-2509796 are used as the basis of this update, which clarifies several aspects.

# 2.	Justifications

## 2.1	Backhaul Impacts

There was discussion at SA2#171 about impacts that use of satellite backhaul can create.

During the satellite studies in 5G, the satellite backhaul connecting RAN and CN is established via feeder links and inter-satellite links. Satellite backhauling recognizes the following generic cases:

1.	Usage of transparent satellites to connect RAN and CN that are located (on the distinct points) on the ground, In this case feeder and service links used for backhauling exploit legacy satellite links [1].

2.	Usage of regenerative satellite payload that facilitates deployment of RAN on board satellites. Regenerative satellites support the usage of inter-satellite links (ISL). CN can be deployed on the ground allowing deployment of some Network Functions on board satellites (e.g., UPF, Edge Computing) [1].

Satellite backhaul via feeder links and/or inter-satellite links has the following characteristics:

-	The Feeder links can switch quickly due to fast movement of the satellites.

-	The inter-satellite links are less stable when compared with e.g., fibre links, especially for cross-orbit inter-satellite links, due to harsh environment and relative movement of satellites. In addition, modern satellite systems could use both radio frequency (RF) and free space optical (FSO) communication for inter-satellite links. Differences in FSO and RF connectivity might introduce additional challenges in the backhaul deployments.

The above characteristics may cause problems including:

-	High jitter, high packet loss rate, and packet disordering, which in turn causes the drop of TCP throughput. According to “LEO Satellite vs. Cellular Networks: Exploring the Potential for Synergistic Integration” [2], TCP severely suffers from such a high packet loss of Starlink, leading to only 1/5 of the throughput achieved by UDP over Starlink.

-	Additionally, it leads to long packet delay due to retransmission and slow start of TCP.

The assumption of high reliability in terrestrial transport networks does not apply in the NTN scenario. NTNs need to address challenges such as higher packet loss, higher jitter, variation in throughput and higher packet reordering to prevent degradation of user experience and maintain system performance.

Considering the above observations and considerations, usage of NTN backhauling introduces additional challenges compared to TNs. In line with this, it is proposed to address NTN backhauling aspects with WT#7 of SA2 6G SID.

# 3. Text Proposal

It is proposed to capture the following changes vs. TR 23.801-01.

# * * * * First change * * * *(All new)

# A.X	Work Task 7

WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects, including:

1.	Study whether and how to mitigate effects from the use of dynamic satellite backhaul and/or feeder links on the user plane traffic between a UE and the CN and maintain system performance.

NOTE 1:	The detailed scope for WT#7 will be coordinated and aligned with RAN, including multi-orbit NTN-NTN mobility handling, in Q1 2026. A checkpoint is set to Q1 2026 to revisit WT/KI descriptions if needed

NOTE 2:	This WT has IWK aspects that are scoped under WT#2.

NOTE 3:	This WT has mobility aspects that are scoped under WT#1.1.

# * * * * Second change * * * *(All new)

## 5.X	Key Issue #X: Support 6G RAT for NTN

WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects, including:

1.	Study whether and how to mitigate effects from the use of dynamic satellite backhaul and/or feeder links on the user plane traffic between a UE and the CN and maintain system performance.

NOTE 1:	The detailed scope for WT#7 will be coordinated and aligned with RAN, including multi-orbit NTN-NTN mobility handling, in Q1 2026. A checkpoint is set to Q1 2026 to revisit WT/KI descriptions if needed

NOTE 2:	This WT has IWK aspects that are scoped under WT#2.

NOTE 3:	This WT has mobility aspects that are scoped under WT#1.1.

# * * * * End of changes * * * *

# 4. References

[1] 3GPP TR 23.700-27: “Study on 5G System with Satellite Backhaul”, Release-18

[2] Bin Hu, Xumiao Zhang, Qixin Zhang, Nitin Varyani, Z. Morley Mao, Feng Qian, and Zhi-Li Zhang. 2023. LEO Satellite vs. Cellular Networks: Exploring the Potential for Synergistic Integration. In Companion of the 19th International Conference on Emerging Networking EXperiments and Technologies (Paris, France) (CoNEXT 2023). Association for Computing Machinery, New York, NY, USA, 45–51. doi:10.1145/3624354.3630588.