3GPP TSG-SA WG2#172	S2-2510923

Dallas, USA, 17-21 November 2025	(was S2-2510265, S2-2510859)

Source:	Qualcomm Incorporated, Deutsche Telekom, vivo

Title:	Compromise proposal for KI#1 and KI#2

Document for:	Discussion/Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC/Rel.20

Abstract of the contribution: Proposes a compromise proposal for KI#1 and KI#2 that can accommodate most prominent solution options.

# 1.

# 1.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-19.

# >>>>BEGINNING OF CHANGES<<<<

## 8.1	Interim conclusions for Key Issue #1

Editor's note:	Other interim conclusions for Key Issue#1 is FFS.

The following interim conclusions for Key Issue #1 Support of IMS voice call over NB-IoT via GEO satellite connecting to EPC are made:

-	The voice packets shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U.

…

-	The PDN connection type for support of IMS voice over NB-IoT NTN is always IP.

-	The UE IP address is allocated as part of the PDN connection establishment to the APN used for IMS voice, and is used by the UE towards IMS during IMS registration.

-	The destination IP address and UDP port to be used to send DL voice traffic to the UE is provided by the UE to the IMS at IMS session-related signaling.

-	For "non-IP" mechanism to transport voice packets, the following header manipulation applies:

For downlink traffic:

-	the appropriate removal/compression of certain fields from the RTP headers is performed in Access GW that is part of IMS. This modified RTP header with voice data is carried between Access GW and PDN GW using UDP/IP transport. The destination UDP/IP is the same as UE provided IP and UDP port for the call session;

- 	UDP/IP headers of voice traffic are removed in PDN GW;

-	UE restores the earlier removed/compressed fields of RTP/ UDP/IP headers using UE internal logic.

For uplink traffic:

-	UE removes/compresses the appropriate fields from the RTP header and removes UDP/IP headers using UE internal logic;

- 	The UDP/IP headers of voice traffic are restored in PDN GW. This restoration is based on the UDP/IP address of the AGW resource and the UDP/IP address of the UE negotiated allocated during call setup.

-	The earlier removed/compressed fields in RTP header are restored in the Access GW that is part of IMS.

-	These conclusions assume no support for multi-party calls and a single UDP flow between PDN GW and Access GW per UE.

Editor's Note: Whether more than one IMS voice calls and support for DTMF is required for IMS voice over NB-IoT NTN is FFS (see S2-2507577).

-	Each non-IP datagram exchanged over the PDN connection corresponds to one and only one IP packet on the data network.

NOTE x: Which RTP headers are removed/compressed will be decided by SA4 but it needs to adapt to the above non-IP mechanism defined by SA2.

Editor's Note: How the support and use of "non-IP" mechanism is negotiated between the UE and network is FFS.

# >>>>END OF CHANGES<<<<