3GPP TSG SA WG2 Meeting #79E (Electronic)	TD S2-103148

06 - 13 July 2010, Elbonia

Source:	NTT DOCOMO

Title:	Priority handling on IMS application level

Document for:	Discussion / Approval

Agenda Item:	3.3

Work Item / Release:	eMPS

Abstract of the contribution: This contribution clarifies the priority handling of SIP message at IMS application level.

- Discussion

Priority call handling for IMS MO call has been discussed and the EPS Bearers for default and IMS/SIP signaling get prioritized handling, while the priority treatment of IMS/SIP signaling itself for the IMS MO call is not yet clarified.

In Rel9 IMS specification, “Resource-Priority header” has already specified that is used for priority services and it is also described how the P-CSCF behaves when it receives this information in SIP message.

Below is the description in TS24.229.

-----------------------------------------------------------------------

## 5.2	Procedures at the P-CSCF

### 5.2.1	General

Where the P CSCF provides emergency call support, the procedures of subclause 5.2.10 shall be applied first.

Subclause 5.2.2 through subclause 5.2.9 define P-CSCF procedures for SIP that do not relate to emergency. All SIP requests are first screened according to the procedures of subclause 5.2.10 to see if they do relate to an emergency.

For all SIP transactions identified:

-	as relating to an emergency; or

-	if priority is supported, as containing an authorised Resource-Priority header field, or, if such an option is supported, relating to a dialog which previously contained an authorised Resource-Priority header field;

the P-CSCF shall give priority over other transactions or dialogs. This allows special treatment of such transactions or dialogs. If the P-CSCF recognises the need for priority processing to a request or if the P-CSCF recognises the need to provide different priority processing than the one indicated by the originating UE, based on the information stored during registration, the P-CSCF may insert or modify Resource-Priority header in accordance with RFC 4412 [116].

NOTE 1: 	The special treatment can include filtering, higher priority processing, routeing, call gapping. The exact meaning of priority is not defined further in this document, but is left to national regulation and network configuration.

The P-CSCF shall support the Path and Service-Route header fields.

NOTE 2:	The Path header field is only applicable to the REGISTER request and its 200 (OK) response. The Service-Route header field is only applicable to the 200 (OK) response of REGISTER request.

NOTE 3:	In subsequent procedures, the P-CSCF can address the needs of individual users (e.g. in support of attached enterprise networks or in support of priority mechanisms, from information saved during registration. In this release of the specification, no information is specified in the registration procedures to perform this, and therefore this information has to either be associated with the user at time of registration from configured information, or by a mechanism outside the scope of this release of the specification.

-----------------------------------------------------------------------

With the above description, it seems it is not yet specified how the P-CSCF gets the priority information so that there seems no standardized way to achieve SIP signalling priority handling.

- Proposal

In this paper we propose the following aspects.

- The procedure for P-CSCF to obtain the priority information for the MPS-subscribed UE.

- SIP message handling for IMS MO call initiated by the MPS-subscribed UE.

******************* Start of the 1st Change *********************

#Changes made after 24th conference call are with revision mark.

# 6	Key Issues for MPS

Editor’s Note: This subclause will describe the proposed solutions to meet the architecture requirements in section5.

## 6.1	IMS Multimedia Priority Services

Editor’s Note: This subclause will describes the key issues for IMS based multimedia priority service including the aspect of interworking between IMS and EPS bearer.

### 6.1.x		Key Issue x – Priority information download procedure

#### 6.1.x.1	Description

To recognize the MPS service on IMS application level, the P-CSCF needs subscriber’s profile of service priority level.

#### 6.1.x.2	Solution

The figure 6.1.x shows the an example priority information download procedure

Note: Figure 6.1.x is an example flow to realize the priority information download to the P-CSCF. Other method, such as SUBSCRIBE-NOTIFY method, may be used and detail will be specified in Stage3. Priority information download to the P-CSCF shall be performed before a MPS-subscribed UE makes the IMS mobile originating call after IMS registration has completed.

Figure 6.1.x: IMS Registration Flow

1-6. These steps do not change from the corresponding IMS Registration procedures specified in 3GPP TS.23.228 [x].

7.	The S-CSCF receives the priority information, i.e. Service Priority Level, for the MPS-subscribed UE from HSS, and maintains it as a part of user profile. (See 3GPP TS 24.229[Y] and 3GPP TS 29.228[Z] for details)

8-

14.	 The priority information is sent to P-CSCF. The P-CSCF stores this information for the MPS-subscribed UE.

Note: CT WG1 is discussing various mechanisms on how to transfer the priority status between S-CSCF and P-CSCF.

The figure 6.1.y shows the SIP message handling for IMS MO call initiated by the MPS-subscribed UE.

Figure 6.1.y: IMS MO call procedure initiated by the MPS-subscribed UE

Step1 does not change from the corresponding IMS MO call procedures specified in TS.23.228 [x].

2.	P-CSCF judges whether the INVITE message requires the priority handling based on user profile stored in the registration procedure, the priority requested by the user and/or MPS code/identifier provided by INVITE message..If the session is determined to have priority handling, then P-CSCF inserts/replaces the priority indication and forwards it to the S--CSCF.

Step3 doses not change from the corresponding IMS MO call procedures specified in TS.23.228 [x].

4.	S-CSCF will treat the INVITE request based on the priority indication received from the P-CSCF. S-CSCF then forwards the INVITE to the next hope with this priority indication.

Following Steps do not change from the corresponding IMS MO call procedures specified in TS.23.228 [x].

******************* End of the 1st Change *********************

******************* Start of the 2nd Change *********************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[X]	3GPP TS 23.228 IP Multimedia Subsystem (IMS); Stage 2

[Y]	3GPP TS 24.229 IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3

[Z]	3GPP TS 29.228 IP Multimedia (IM) Subsystem Cx and Dx interfaces; Signalling flows and message contents

******************* End of the 2nd Change *********************