3GPP TSG SA WG2 Meeting #84	TD S2-111356

11 – 15 April 2011, Bratislava, Slovac

Source:	China Telecom, China Unicom, KDDI, BT, Telecom Italia, ZTE, Allot Communications, Tekelec, Bridgewater, GENBAND, Hitachi, Huawei

Title:	WID for usage monitoring control enhancement

Document for:	Approval

Agenda Item:	10.1

Work Item / Release:	Rel-11

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.

# Title * : Usage Monitoring Control Enhancement

## Acronym * : ()

## Unique identifier *

## 1	3GPP Work Area *

|  | Radio Access |
| --- | --- |
| X | Core Network |
|  | Services |

## 2	Classification of WI and linked work items

### 2.0	Primary classification *

This work item is a … *

|  | Study Item (go to 2.1) |
| --- | --- |
| X | Feature (go to 2.2) |
|  | Building Block (go to 2.3) |
|  | Work Task (go to 2.4) |

### 2.1	Study Item

| Related Work Item(s) (if any] |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Go to §3.

### 2.2	Feature

| Related Study Item or Feature (if any) * |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Go to §3.

### 2.3	Building Block

| Parent Feature (or Study Item) |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

This work item is … *

|  | Stage 1 (go to 2.3.1) |
| --- | --- |
|  | Stage 2 (go to 2.3.2) |
|  | Stage 3 (go to 2.3.3) |
|  | Test spec (go to 2.3.4) |
|  | Other (go to 2.3.5) |

#### 2.3.1		Stage 1

| Source of external requirements (if any) * |  |  |
| --- | --- | --- |
| Organization | Document | Remarks |

Go to §3.

#### 2.3.2		Stage 2  *

| Corresponding stage 1 work item |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

| Other source of stage 1 information |  |  |
| --- | --- | --- |
| TS or CR(s) | Clause | Remarks |

If no identified source of stage 1 information, justify: *

Go to §3.

#### 2.3.3		Stage 3 *

| Corresponding stage 2 work item (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

| Else, corresponding stage 1 work item |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

| Other justification |  |  |
| --- | --- | --- |
| TS or CR(s) Or external document | Clause | Remarks |

If no identified source of stage 2 information, justify: *

Go to §3.

#### 2.3.4		Test spec *

| Related Work Item(s) |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

Go to §3.

#### 2.3.5		Other *

| Related Work Item(s) |  |  |  |
| --- | --- | --- | --- |
| Unique ID | Title | Nature of relationship | TS / TR |

Go to §3.

### 2.4		Work task *

| Parent Building Block |  |  |
| --- | --- | --- |
| Unique ID | Title | TS |

## 3	Justification *

The usage monitoring control has been introduced into PCC since Rel-9 which provides the operator the capability to enforce dynamic policy decisions based on the total network usage in real-time. It was enhanced under SAPP Work Item in rel-11 to support usage monitoring for services that are detected by the TDF.  It need to be studied if the following requirements can be fulfilled or extensions to PCC framework are needed:

- One service data flow/application only can belong to one monitoring group (i.e. A PCC/ADC rule corresponding to the service data flow/application only can include one monitoring key). If the operator has the policy that one service/application is included in more than one monitoring group, it needs to be investigated how usage monitoring control needs to be enhanced.

- How a service data flow/application can be disabled from the existing usage monitoring group of services/group of applications.

- How to exclude the usage of a particular service data flow from the accumulated usage for the IP-CAN session.

- Operators may have different usage allowance for the same service data flow/application or IP-CAN/TDF session in the different condition, e.g. leisure and busy hour, roaming and non-roaming. It is useful to optimize the procedure to reduce concurrent signalling caused by allowance change due to such condition, e.g. by keeping the accumulated usage value when the usage threshold is changed, but report it only when the next report (e.g. session termination, report on demand from PCRF etc.) is done i.e. to avoid many simultaneous reports .

- How to apply usage monitoring control for a subscriber group  e.g. the members of a family or a company, or a group of devices belonging to a user. that share the same usage allowance threshold. In current PCC Architecture, different PCEFs and PCRFs may be selected for the subscribers when they are connected to the same APN at the same time, especially when the multiple PCRFs are deployed in the network.

.

## 4	Objective *

This work item aims to investigate if enhancements to the existing PCC architecture are needed. For those that are needed,  specify the enhancements to the policy control architecture to lift the possible restrictions of the usage monitoring control as mentioned in the justification part

Specifically, potential enhancement may include:

- Use cases and requirements to support usage monitoring control enhancement.

- How to exclude the usage of a particular service data flow from the accumulated usage for the IP-CAN session.

- How a service data flow/application can be disabled from the existing usage monitoring group of services/group of applications.

- How to exclude the usage of a particular service data flow from the accumulated usage for the IP-CAN session.

- Operators may have different usage allowance for the same service data flow/application or IP-CAN/TDF session in the different condition, e.g. leisure and busy hour, roaming and non-roaming. It is useful to optimize the procedure to reduce concurrent signalling caused by allowance change due to such condition, e.g. by keeping the accumulated usage value when the usage threshold is changed, but report it only when the next report (e.g. session termination, report on demand from PCRF etc.) is done i.e. to avoid many simultaneous reports .

- How to apply usage monitoring control for a subscriber group e.g. the members of a family or a company, or a group of devices belonging to a user. That shares the same usage allowance threshold. In current PCC Architecture, different PCEFs and PCRFs may be selected for the subscribers when they are connected to the same APN at the same time, especially when the multiple PCRFs are deployed in the network.

## 5	Service Aspects

The proposed work will not impact specific services but is likely to have some impact on aspects of service delivery.

## 6	MMI Aspects

N/A

## 7	Charging Aspects

N/A.

## 8	Security Aspects

N/A

## 9	Impacts *

| Affects: | UICC apps | ME | AN | CN | Others |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  |  |
| Don't know |  |  |  |  | X |

## 10	Expected Output and Time scale *

| New specifications * [If Study Item, one TR is anticipated] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Spec No. | Title |  | Prime rsp. WG | 2ndary rsp. WG(s) | Presented for information at plenary# |  | Approved at plenary# |  | Comments |
| 23.8xx | Usage Monitoring Control Enhancement |  | SA2 |  | SA#53(Sep 2011) |  | SA#54(Dec 2011) |  |  |
| Affected existing specifications * [None in the case of Study Items] |  |  |  |  |  |  |  |  |  |
| Spec No. | CR | Subject |  |  |  | Approved at plenary# |  | Comments |  |
| 23.203 |  | Policy and charging control architecture |  |  |  | SA#54(Dec 2011) |  |  |  |
| 22.278 |  | Service requirements for the Evolved Packet System (EPS) |  |  |  | SA#53(Sep 2011) |  |  |  |
| 32.296 |  | New feature description will be considered |  |  |  | SA#54(Dec 2011) |  |  |  |

## 11		Work item rapporteur(s) *

Zaifeng Zong (zong.zaifeng@zte.com.cn) (SA2)

Mian Li (Li.Mian@zte.com.cn) (SA1)

Hui Cai (Sarah.Cai@huawei.com) (SA5)

## 12		Work item leadership *

SA2  (primary)

SA1 (secondary)

SA5 (secondary)

## 13		Supporting Individual Members *

Supporting IM name

China Telecom

China Unicom

KDDI

BT

Telecom Italia

ZTE

Allot Communications

Tekelec

Bridgewater

GENBAND

Hitachi

Huawei

form change history:

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"