3GPP TSG SA WG2 Meeting #93	TD S2-123659

08 - 12 October, Sofia, Bulgaria

Source:	Huawei, Hisilicon

Title:	Optimized ISRP for OPIIS without addition of Filter Rules for inter-APN routing

Document for:	Approval

Agenda Item:	8.6

Work Item / Release:	OPIIS / Release 12

Abstract of the contribution: This paper proposes to provide a solution for achieve OPIIS functionalities by extend ISPR without creating a separated inter-APN routing container.

#### 1. Discussion

Currently there are two candidate solutions for OPIIS which all need to define a new inter-APN policy rules while using this rule differently.  Although introduction of a new separated inter-APN policy rules is a relatively neater solution, it inevitably has some drawbacks, such as

- Increasing the unnecessary overhead,  Per 24.312, existing flow distribution containers inside ISPR already have APN and its corresponding   routing and flow, although they serve for different purse.  Also the If flow filters can be shared for both APN mapping and ISPR because the final selected access interface and APN should associate with the same IP flow.   Therefore by duplicating those information within same ISPR will potentially create signalling overload in the network while provisioning to the massive users during short period time.

E.g,

The interAPN routing policy from solution 2:

And the forflowbased rules:

- Lack of supporting on dynamic APN and IP interface selection. Because the APN and IP interface association may not always statistic and can be different by the routing criteria of different routing rules (PerFlowBased, ForServiceBased and ForNonSeamlessOffload), such as, UE will choose different interface depending on the current time and location, special the access interface it choose.   But by separating APN routing rules which is statistic out from those dynamic routing rules, UE may select the wrong APN and IP interface. For example,  per operator policy overload policy, for certain region, all video service through CSG cell and WiFi will be route to APN 1 during peak time (9am – 9 pm),  but during un-peak time, only the video service through WiFi will be route to APN1. Therefore the mapping relationship between APN, IP flow and interface may need to be linked to access interface selection policies.

In order to address those above issues,  this contribution tries to explore  a potential solution which tries to re-use existing ISRP rules as much as possible and adding APN filter capability into existing  three flow distribution container of ISPR.

#### 3. Proposed addition in TR 23.853

*****************  START OF CHANGES *******************

# 5	Solutions

## 5.x	Solution x: Integrating APN routing policy into existing ISRP filter rules

### 5.x.1	Description

### This solution is integrating the IPflow and IP interface (APN) mapping capability into existing three ISPR filter rules for IFOM, MAPCON and non-seamless offload, by simple introducing APN information as possible routing result like access technology whenever a distribution rule is evaluated. Therefore, when UE handle a uplink traffic, it evaluate the active distribution container which match the IP flow, and obtain the rule result which not only contains the access interface information but also the APN and IP flow mapping information.  Therefore, the APN mapping can be static basing on the IP flow, or dynamic basing on the access interface UE select.

### Because currently for MAPCON, each Filter Rule identifies traffic based on APN, so this solution need to extend MAPCON filter rules to be able to identify traffic based on simple IP flow filter if the APN for that uplink flow is unknown before the evaluation, so UE can identify the APN first then its corresponding access interface basing with that APN.

### Since this feature is for routing of IP flows among a choice of available interfaces in both 3GPP and non-3GPP accesses, therefore, for 3GPP routing, UE will use the extended MAPCON filter rules as mentioned above to find the matched APN for certain uplink traffic. For 3GPP routing, the routing result will only contain APN but no selected access technology.   Also for 3GPP only UE, in order to use OPIIS functionality, it need to download the ANDSF policy with OPIIS capability and apply ISRP.

### For the case which the routing validity condition contains APN restriction while there is no APN associate with the uplink traffic stream, UE can first evaluate the uplink traffic with the filter rules to obtain the potential matched APNs, then using this APN information to run a second evaluation with the distribution container to  obtain the routing result with the access interface and APN.

### The general procedure is illustrated below:

### 5.x.2	Impact on existing nodes or functionality

This solution keeps the existing flow distribution contains without creating new container and rules, the main changes are introducing APN selection into the routinerules, which UE will not only obtain the selected access info but also the ipflow and IP interface mapping relationship.  The main procedure and functionality impacts are:

1, Extend routing result to include APN information

2. Extend MAPCON rules to add IP flow filter, and use MAPCON for selecting IP interface for intra-3GPP traffic .

3. Some procedure changes illustrated in the previous section.

*****************  END OF CHANGES *******************