T1P1.5/99-181r1

# TITLE:	CR Against GSM 03.32

SOURCE:			CPS Ltd.

AUTHORS:		Andy Read

CONTACT:		Andy Read

CPS Ltd.,

62-64 Hills Road,

Cambridge  CB2 1LA

United Kingdom

+44 1223 326900

Andy.Read@cursor-system.com

DATE:				March 17th, 1999

ABSTRACT:		CR for GSM 03.32 to add elliptical confidence regions.

COPYRIGHT STATEMENT:

 1999 CPS Ltd.

The contributor grants a free, irrevocable license to Committee T1 to incorporate text contained in this contribution and any modification thereof in the creation of a T1 standards publication; to copyright in T1’s name any T1 standards publication even though it may include portions of this contribution; and at T1’s sole discretion to permit others to reproduce in whole or in part the resulting T1 standards publication.

Permission is granted to Committee T1 participants to copy any portion of this document for the legitimate purposes of the T1. Copying for monetary gain or other non-T1 related purposes is prohibited.

NOTICE:	This contribution has been prepared by CPS as a basis for discussion to assist the Technical Subcommittee T1P1. The document should not be construed as a binding proposal on CPS. Specifically, CPS reserves the right to modify, amend or withdraw the contents of this contribution.

ETSI SMG#29	Tdoc SMG 99???

Miami

June 21-25, 1999

| CHANGE REQUEST No : |  |  |  |  |  | xxxx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 03.32 |  | Version |  | 5.2.0 |  |  |  |
| Submitted to SMG | 29 |  | for approval |  | X | without presentation ("non-strategic") |  |  |  |  |  |
| list plenary meeting or STC here  |  | for information |  |  |  | with presentation ("strategic") |  |  |  | X |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME |  | Network | X |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | Location Services (LCS) |
| --- | --- |

| Source: | T1P1.5 | Date: | 15 March 99 |
| --- | --- | --- | --- |

| Subject: | MAP Impacts for Location Services (LCS) |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature | X |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 | X |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | Add options for elliptical uncertainty region and altitude parameter to support LCS |
| --- | --- |

| Clauses affected: | 5, 6, 7, Annex A |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

TD <>

TS 101 109 V5.2.0 (1998-10)

Technical Specification

Digital cellular telecommunications system (Phase 2+);

Universal Geographical Area Description (GAD)

(GSM 03.32 version 5.2.0 Release 1996)

Reference

RTS/SMG-030332QR1

Keywords

Digital cellular telecommunications system, Global System for Mobile communications (GSM) Universal Geographical Area Description (GAD);

ETSI Secretariat

Postal address

F-06921 Sophia Antipolis Cedex - FRANCE

Office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - NAF 742 C

Association à but non lucratif enregistrée à la

Sous-Préfecture de Grasse (06) N° 7803/88

X.400

c= fr; a=atlas; p=etsi; s=secretariat

Internet

secretariat@etsi.fr

http://www.etsi.fr

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© European Telecommunications Standards Institute 1998.

All rights reserved.

Contents

TITLE:	CR Against GSM 03.32	1

Intellectual Property Rights	8

Foreword	8

1	Scope	9

2	Normative references	9

3	Definitions and abbreviations	9

3.1	Definitions	9

3.2	Abbreviations	9

4	Reference system	10

5	Shapes	10

5.1	Ellipsoid Point	10

5.2	Ellipsoid point with uncertainty circle	11

5.2a	Ellipsoid point with uncertainty ellipse	12

5.3	Polygon	12

6	Coding	13

6.1	Point	13

6.2	Uncertainty	14

7	General message format and information elements coding	14

7.1	Overview	14

7.2	Type of Shape	15

7.3	Shape description	15

7.3.1	Ellipsoid Point	16

7.3.2	Ellipsoid Point with uncertainty Circle	17

7.3.2a	Ellipsoid Point with uncertainty Ellipse	18

7.3.3	Polygon	19

Annex A (informative): Element description in compact notation	20

Annex B (informative): Status of GSM 03.32	21

History	22

# Intellectual Property Rights

IPRs essential or potentially essential to the present document may have been declared to ETSI. The information pertaining to these essential IPRs, if any, is publicly available for ETSI members and non-members, and can be found in SR 000 314: "Intellectual Property Rights (IPRs); Essential, or potentially Essential, IPRs notified to ETSI in respect of ETSI standards", which is available free of charge from the ETSI Secretariat. Latest updates are available on the ETSI Web server (http://www.etsi.fr/ipr or http://www.etsi.org/ipr).

Pursuant to the ETSI IPR Policy, no investigation, including IPR searches, has been carried out by ETSI. No guarantee can be given as to the existence of other IPRs not referenced in SR 000 314 (or the updates on the ETSI Web server) which are, or may be, or may become, essential to the present document.

# Foreword

This ETSI Technical Specification (TS) has been produced by the Special Mobile Group (SMG) of the European Telecommunications Standards Institute (ETSI).

This specification defines an intermediate universal Geographical Area Description within the digital cellular telecommunications system.

The contents of this specification are subject to continuing work within SMG and may change following formal SMG approval. Should SMG modify the contents of this specification it will then be republished by ETSI with an identifying change of release date and an increase in version number as follows:

Version 5.x.y

where:

5	GSM Phase 2+ Release 1996;

x	the second digit is incremented for all other types of changes, i.e. technical enhancements, corrections, updates, etc.;

y	the third digit is incremented when editorial only changes have been incorporated in the specification.

# 1	Scope

This ETSI Technical Specification (TS) defines an intermediate universal Geographical Area Description which subscriber applications or GSM services can use and the network can convert into an equivalent radio coverage map.

For GSM services which involve the use of an "area", it can be assumed that in the majority of cases the Service Requester will be forbidden access to data on the radio coverage map of a particular PLMN and that the Service Requester will not have direct access to network entities (e.g. BSC/BTS).

The interpretation by the PLMN operator of the geographical area in terms of cells actually used, cells that are partly within the given area and all other technical and quality of service aspects are out of the scope of this specification.

# 2	Normative references

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.

-	A non-specific reference to an ETS shall also be taken to refer to later versions published as an EN with the same number.

[1]	GSM 01.04: "Digital cellular telecommunications system (Phase 2+); Abbreviations and acronyms".

[2]	GSM 04.07: "Digital cellular telecommunications system (Phase 2+); Mobile radio interface signalling layer 3  General aspects".

[3]	Military Standard WGS84 Metric MIL-STD-2401 (11 January 1994): "Military Standard Department of Defence World Geodetic System (WGS)".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of this specification, the following definitions apply:

Service Requester: Entity, which uses the Geographical Area Description in any protocol to inform the network about a defined area

## 3.2	Abbreviations

In addition to those mentioned below, abbreviations used in this specification are listed in GSM 01.04.

For the purposes of this specification, the following abbreviations apply:

GAD	Geographical Area Description

GPS	Global Positioning System

WGS	World Geodetic System

# 4	Reference system

The reference system chosen for the coding of locations is the World Geodetic System 1984, (WGS 84), which is also used by the Global Positioning System, (GPS). The origin of the WGS 84 co-ordinate system is the geometric centre of the WGS 84 ellipsoid. The ellipsoid is constructed by the rotation of an ellipse around the minor axis which is oriented in the North-South direction. The rotation axis is the polar axis of the ellipsoid, and the plane orthogonal to it and including the centre of symmetry is the equatorial plane.

The relevant dimensions are as follows:

Major Axis (a) = 6378137 m

Minor Axis (b) = 6356752,314 m

First eccentricity of the ellipsoid =a2−b2b2=0,0066943800668

Co-ordinates are then expressed in terms of longitude and latitude relevant to this ellipsoid. The range of longitude is -180° to +180°, and the range of latitude is -90° to +90°. 0° longitude corresponds to the Greenwich Meridian, and positive angles are to the East, while negative angles are to the West. 0° latitude corresponds to the equator, and positive angles are to the North, while negative angles are to the South. Altitudes are defined as the distance between the ellipsoid and the point, along a line orthogonal to the ellipsoid.

# 5	Shapes

The intention is to incorporate a number of different shapes, that can be chosen according to need. In this version only a minimum number of shapes are described.

-	Ellipsoid Point;

-	Ellipsoid point with uncertainty circle;

-	Ellipsoid point with uncertainty ellipse;

-	Polygon.

Each shape is discussed individually.

## 5.1	Ellipsoid Point

The description of an ellipsoid point is that of a point on the surface of the ellipsoid, and consists of a latitude and a longitude. In practice, such a description can be used to refer to a point on Earth’s surface, or close to Earth’s surface, with the same longitude and latitude. No provision is made in this version of the standard to give the height of a point.

Figure 1 illustrates a point on the surface of the ellipsoid and its co-ordinates.

The latitude is the angle between the equatorial plane and the perpendicular to the plane tangent to the ellipsoid surface at the point. Positive latitudes correspond to the North hemisphere. The longitude is the angle between the half-plane determined by the Greenwich meridian and the half-plane defined by the point and the polar axis, measured Eastward.

Figure 1: Description of a Point as two co-ordinates

## 5.2	Ellipsoid point with uncertainty circle

The "ellipsoid point with uncertainty circle" is characterised by the co-ordinates of an ellipsoid point (the origin) and a distance r. It describes formally the set of points on the ellipsoid which are at a distance from the origin less than or equal to r, the distance being the geodesic distance over the ellipsoid, i.e., the minimum length of a path staying on the ellipsoid and joining the two points, as shown in figure 2.

As for the ellipsoid point, this can be used to indicate points on the Earth surface, or near the Earth surface, of same latitude and longitude.

The typical use of this shape is to indicate a point when its position is known only with a limited accuracy.

Figure 2: Description of an uncertainty Circle

## 5.2a	Ellipsoid point with uncertainty ellipse

The "ellipsoid point with uncertainty ellipse" is characterised by the co-ordinates of an ellipsoid point (the origin),  distances r1 and r2 and an angle A. It describes formally the set of points on the ellipsoid which fall within or on the boundary of an ellipse with semi-major axis of length r1 rotated at angle A measure clockwise from north and semi-minor axis of length r2, the distances being the geodesic distance over the ellipsoid, i.e., the minimum length of a path staying on the ellipsoid and joining the two points, as shown in figure 3.

As for the ellipsoid point, this can be used to indicate points on the Earth surface, or near the Earth surface, of same latitude and longitude.

The typical use of this shape is to indicate a point when its position is known only with a limited accuracy, but the geometrical contributions to uncertainty can be quantified.

semi-major axis, r1

North

angle, A

semi-minor axis, r2

Figure 2a: Description of an uncertainty Ellipse

## 5.3	Polygon

A polygon is an arbitrary shape described by an ordered series of points (in the example pictured in the drawing, A to E). The minimum number of points allowed is 3, and the maximum number of points allowed is 15. The points shall be connected in the order that they are given. A connecting line is defined as the line over the ellipsoid joining the two points and of minimum distance (geodesic). The last point is connected to the first. The list of points must respect a number of conditions:

-	a connecting line shall not cross another connecting line;

-	two successive points must not be diametrically opposed on the ellipsoid.

The described area is situated to the right of the lines with the downward direction being toward the Earth’s centre and the forward direction being from a point to the next.

NOTE:	This definition does not permit connecting lines greater than roughly 20 000 km. If such a need arises, the polygon can be described by adding an intermediate point.

Computation of geodesic lines is not simple. Approximations leading to a maximum distance between the computed line and the geodesic line of less than 3 metres are acceptable.

Figure 3: Description of a Polygon

# 6	Coding

## 6.1	Point

The co-ordinates of an ellipsoid point are coded with an uncertainty of less than 3 metres

The latitude is coded with 24 bits: 1 bit of sign and a number between 0 and 223-1 coded in binary on 23 bits. The relation between the coded number N and the range of (absolute) latitudes X it encodes is the following (X in degrees):

N≤22390X<N+1

except for N=223-1, for which the range is extended to include N+1.

The longitude, expressed in the range -180°, +180°, is coded as a number between -223 and 223-1, coded in 2’s complement binary on 24 bits. The relation between the coded number N and the range of longitude X it encodes is the following (X in degrees):

N≤224360X<N+1

## 6.2	Uncertainty

A method of describing the uncertainty has been sought which is both flexible (can cover wide differences in range) and efficient. The proposed solution makes use of a variation on the Binomial expansion. The uncertainty r, expressed in metres, is mapped to a number K, with the following formula:

r=C1+xK−1

with C = 10 and x = 0,1. With 0 £ K £ 127, a suitably useful range between 0 and 1800 kilometres is achieved for the uncertainty, while still being able to code down to values as small as 1 metre. The uncertainty can then be coded on 7 bits, as the binary encoding of K.

Table 1: Example values for the uncertainty Function

| Value of K | Value of uncertainty |
| --- | --- |
| 0 | 0 m |
| 1 | 1 m |
| 2 | 2.1 m |
| - | - |
| 20 | 57.3 m |
| - | - |
| 40 | 443 m |
| - | - |
| 60 | 3 km |
| - | - |
| 80 | 20 km |
| - | - |
| 100 | 138 km |
| - | - |
| 120 | 927 km |
| - | - |
| 127 | 1800 km |

# 7	General message format and information elements coding

This clause describes a coding method for geographical area descriptions. A geographical area description is coded as a finite bit string. In the figures, the bit string is described by octets from top downward, and in the octet from left to right. Number encoding strings start with the most significant bit.

## 7.1	Overview

A bit string encoding a geographical description shall consist of the following parts:

-	Type of Shape;

-	Shape Description.

Such a bit string is usually part of an information element. The structure of the information element (e.g., element identifier, length) depends on the protocol in which the message containing the description is defined, and is specified in the protocol specification.

This organisation is illustrated in the example shown in figure 4.

Figure 4: Example

## 7.2	Type of Shape

The Type of Shape information field identifies the type which is being coded in the Shape Description. The Type of Shape is coded as shown in table 2.

Table 2: Coding of Type of Shape

| Bits |  |
| --- | --- |
| 4 3 2 1 |  |
| 0 0 0 0 | Ellipsoid Point |
| 0 0 0 1 | Ellipsoid point with uncertainty Circle |
| 0 0 1 0 | Ellipsoid point with uncertainty Ellipse |
| 0 1 0 1 | Polygon |
| other values | Reserved for future use |

## 7.3	Shape description

The shape description consist of different elements.

### 7.3.1	Ellipsoid Point

The coding of a point is described in figure 5.

Figure 5: Shape description of a point

S:	Sign of latitude

Bit value 0 	North

Bit value 1 	South

Degrees of latitude

Bit 1 of octet 4 is the low order bit

Degrees of longitude

Bit 1 of octet 7 is the low order bit

### 7.3.2	Ellipsoid Point with uncertainty Circle

Figure 6: Shape description of an ellipsoid point with uncertainty circle

### 7.3.2a	Ellipsoid Point with uncertainty Ellipse

Figure 6a: Shape description of an ellipsoid point with uncertainty ellipse

Angle of major axis

8 bit 2’s compliment, allowing for necessary range [-90…+90] degrees

### 7.3.3	Polygon

Figure 7: Shape description of a polygon

The number of points field encodes in binary on 4 bits the number n of points in the description, and ranges from 3 to 15.

Annex A (informative):
Element description in compact notation

The notation is the one described in GSM 04.07.

<Geographical Area Description> ::=

<Point> |

<Point with uncertainty> |

<Polygon> ;

<Point> : :=

0000 <spare>(4)

<Point co-ordinates> ;

<point co-ordinates> ::=

<Latitude sign : bit> <Unsigned latitude : bit string(23)>

<Longitude : bit string(24)> ;

<Point with uncertainty> ::=

0001 <spare>(4)

<Point co-ordinates>

<spare bit> <Uncertainty: bit string(7)> ;

<Point with uncertainty ellipse> ::=

0010 <spare>(4)

<Point co-ordinates>

<spare bit> <Uncertainty semi-major: bit string(7)>

<spare bit> <Uncertainty semi-minor: bit string(7)>

<Angle of major axis: bit string(8)>;

<Polygon> ::=

0101 <Number of points>

<Point co-ordinates>(val(Number of points)) ;

<Number of points> ::=

0011 | 0100 | 0101 | 0110 | 0111 | 1000 | 1001 | 1010 |

1011 | 1100 | 1101 | 1110 | 1111 ;

Annex B (informative):
Status of GSM 03.32

| Status of Technical Specification GSM 03.32 |  |  |
| --- | --- | --- |
| Date | Version | Remarks |
|  |  | No Phase 1 version |
| October 1997 | 5.0.0 | Specification approved by SMG#23 |
| June 1998 | 5.1.0 | CR 03.32-A001r1 (R96-F) on Clarification in section 7.3.1 (WI: CAMEL Phase 1) approved by SMG#26 |
| October 1998 | 5.2.0 | CR 03.32-A002r1 (R96-F) on Error correction in GSM 03.32 for value of uncertainty (WI: CAMEL Phase 1) approved by SMG#27 |
| March 1999 | ?.?.? | CR 03.32-A??? Addition of Uncertainty Ellipse |
| Text and figures: WinWord 6.0 Stylesheet: etsiw_70.dot Rapporteur: |  |  |

# History

| Document history |  |  |
| --- | --- | --- |
| V5.0.0 | November 1997 | Publication |
| V5.1.0 | July 1998 | Publication |
| V5.2.0 | October 1998 | Publication |
| V ?.?.? | March 1999 | Draft |