3GPP TSG-SA WG3 #4, London, 16-18 June 1999	S3-99151

3G TS 33.103 V0.0.2 (1999-06)

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

3G Security;

Integration Guidelines

3G TS 33.103 V 0.0.2

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

Reference

DTS/TSGS-________

Keywords

Security, Authentication and Key Agreement, Security Information Stored, Location of Security Functions, Parameter Lengths

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorised by written permission.
The copyright and the foregoing restrictions extend to reproduction in all media.

© 3GPP 1999

All rights reserved.

Contents

Foreword	4

1	Scope	4

2	References	4

2.1	Normative references	4

2.2	Informative references	5

3	Definitions, symbols and abbreviations	5

3.1	Definitions	5

3.2	Symbols	5

3.3	Abbreviations	5

4	Overview of the security architecture	6

4.1	Overview of the complete 3G security architecture.	6

4.2	Overview of Authentication and Key Agreement Mechanism	8

5	Security Information Stored	9

5.1	Information common to Sequence Number and Temporary Key Mechanism	9

5.1.1	Home Environment Authentication Center  HE/AuC	10

5.1.2	Serving Node Visited Location Register SN/VLR	11

5.1.3	Radio Node Controller RNC	12

5.1.3	Mobile Equipment user Identify Module USIM	13

5.1.4	Mobile Equipment ME	14

5.2	Sequence Number Mechanism	15

5.2.1	Home Environment Authentication Center  HE/AuC	15

5.2.2	Serving Node Visited Location Register SN/VLR	16

5.2.3	Radio Node Controller  RNC	16

5.2.4	Mobile Equipment user identity Module USIM	17

5.2.5	Mobile Equipment ME	17

6	Location of Security Functions	18

6.1	 Sequence Number Mechanism	18

6.1.1	Home Environment Authentication Center HE/AuC	18

6.1.2	Serving Node Visited Location Register SN/VLR	19

6.1.3	Radio Node Controller  RNC	19

6.1.4	Mobile Equipment user identity Module USIM	19

6.1.5	Mobile Equipment ME	20

Appendix 1 Temporary Key Mechanism	21

A1	Security Information stored	22

A1.1	Home Environment Authentication Centre  HE/AuC	22

A1.2	Serving Node Visited Location Register SN/VLR	23

A1.3	Radio Node Controller RNC	23

A1.4	Mobile Equipment user identity Module USIM	24

A1.5	 Mobile Equipment	24

A2	Location of Security Functions	25

A2.1	Home Environment Authentication Centre HE/AuC	25

A2.2	Serving Node Visited Location Register SN/VLR	25

A2.3	Radio Node Controller  RNC	26

A2.4	Mobile Equipment user identity Module USIM	26

A2.5	Mobile Equipment ME	27

Appendix 2	 Document history	28

# Foreword

This document has been drafted by 3GPP TSG-SA WG 3, i.e., the Workgroup devoted to “Security” issues, within the Technical Specification Group devoted to “System Aspects”.

# 1	Scope

This technical specification defines how elements of the security architecture are to be integrated into the following entities of the system architecture.

- Home Environment Authentication Center  (HE/AuC)

- Serving Network Visited Location Register ( SN/VLR)

- Radio Node Controller ( RNC)

- Mobile station User Identity Module (UIM)

- Mobile Equipment  (ME)

This specification is derived from 3G "Security architecture".

The structure of this technical specification is as follows:

Clause 5 lists the security information to be stored in the above entities of the 3G system in terms of Static information and Dynamic information.

Clause 6 defines the external specification of the security-related algorithms in terms of input and output parameters, and the parameter lengths.

The equivalent information for the alternative Temporary Key proposal is included in an appendix to this document

# 2	References

References may be made to:

a)	specific versions of publications (identified by date of publication, edition number, version number, etc.), in which case, subsequent revisions to the referenced document do not apply; or

b)	all versions up to and including the identified version (identified by “up to and including” before the version identity); or

c)	all versions subsequent to and including the identified version (identified by “onwards” following the version identity); or

d)	publications without mention of a specific version, in which case the latest version applies.

A non-specific reference to an ETS shall also be taken to refer to later versions published as an EN with the same number.

## 2.1	Normative references

The above heading is optional - if a need to sub-divide the references clause is felt to be appropriate.

[1]	 TR S3.03 Security Architecture

## 2.2	Informative references

The above heading is optional - if a need to sub-divide the references clause is felt to be appropriate.

# 3	Definitions, symbols and abbreviations

Delete from the above heading those words which are not applicable.

## 3.1	Definitions

Clause numbering depends on applicability. Defined terms should be ordered alphabetically.

For the purposes of the present document, the following definitions apply:

Confidentiality:	The property that information is not made available or disclosed to unauthorised individuals, entities or processes.

Data integrity: 	The property that data has not been altered in an unauthorised manner.

Data origin authentication:  The corroboration that the source of data received is as claimed.

Entity authentication: The provision of assurance of the claimed identity of an entity.

Key freshness: 	A key is fresh if it can be guaranteed to be new, as opposed to an old key being reused through actions of either an adversary or authorised party.

## 3.2	Symbols

Clause numbering depends on applicability. Symbols should be ordered alphabetically.

For the purposes of the present document, the following symbols apply:

||	Concatenation

	Exclusive or

f1	Message authentication function used to compute MAC

f2	Message authentication function used to compute RES and XRES

f3	Key generating function used to compute CK

f4	Key generating function used to compute IK

f5	Key generating function used to compute AK

f6	Encryption function used to encrypt the IMUI

f7	Decryption function used to decrypt the IMUI (=f6-1)

K	Long-term secret key shared between the USIM and the AuC

## 3.3	Abbreviations

Clause numbering depends on applicability. Abbreviations should be ordered alphabetically.

For the purposes of the present document, the following abbreviations apply:

3GMS	Third Generation Mobile Communication System

AK	Anonymity key

AUTN	Authentication token

AV	Authentication vector

CK	Cipher key

CS	Circuit switched

HE	Home Environment

HLR	Home Location Register

IK	Integrity key

IMUI	International Mobile User Identity

KSI	Key Set Identifier

KSS	Key Stream Segment

LAI	Location Area Identity

MAC	Message Authentication Code

MAC	The message authentication code included in AUTN, computed using f1

MS	Mobile Station

MSC	Mobile Services Switching Centre

MT	Mobile Termination

PS	Packet switched

TE	Terminal Equipment

TMUI	Temporary Mobile User Identity

RAND	Random challenge

SEQ	Sequence number

SN	Serving network

TMUI	Temporary Mobile User Identity

UEA	UMTS Encryption Algorithm

UIA	UMTS Integrity Algorithm

UN	User Name

USIM	User Services Identity Module

VLR	Visited Location Register

XRES	Expected response

XUR	Expected User Response

# 4	Overview of the security architecture

## 4.1	Overview of the complete 3G security architecture.

Figure 1 : Overview of the security architecture

Five security feature groups are defined. Each of these feature groups meets certain threats, accomplishes certain security objectives:

- Network access security (I): the set of security features that provide users with secure access to 3G services, and which in particular protect against attacks on the (radio) access link;

- Network domain security (II): the set of security features that enable nodes in the provider domain to securely exchange signalling data, and protect against attacks on the wireline network;

- User domain security (III): the set of security features that secure access to mobile stations

- Application domain security (IV): the set of security features that enable applications in the user and in the provider domain to securely exchange messages.

Visibility and configurability of security (V): the set of features that enables the user to inform himself whether a security features is in operation or not and whether the use and provision of services should depend on the security feature.

## 4.2	Overview of Authentication and Key Agreement Mechanism

Upon receipt of a request from the SN/VLR, the HE/AuC sends an ordered array of n authentication vectors (the equivalent of a GSM “triplet”) to the SN/VLR.

When the SN/VLR initiates an authentication and key agreement, it selects the next authentication vector from the array and sends the parameters RAND and AUTN to the user. The USIM checks whether AUTN can be accepted and, if so, produces a response RES which is sent back to the SN/VLR. The USIM also computes CK and IK. The SN/VLR compares the received RES with XRES. If they match the SN/VLR considers the authentication and key agreement exchange to be successfully completed.

Figure 2: Generation of an authentication vector

Figure 3: User authentication function in the USIM

# 5	Security Information Stored

## 5.1	Information common to Sequence Number and Temporary Key Mechanism

This Clause lists the security information to be stored in the relevant entities of the 3G system in terms of Static information and Dynamic information and relates to the preferred sequence number mechanism, the alternative temporary key mechanism is detailed in the Appendix.  However, sections 5.1 contains information, which is common to both mechanisms.

Notes on tables

Parameter Lengths: The length of the parameters is given to allow estimates of the message sizes on the air interface to be determined and the maximum width for algorithm design. The effective key length used will be determined by national policy.

Lifetimes: The entry in the tables for parameter lifetime is defined in the table below

| ref. | Definition |
| --- | --- |
| a | For the life of the equipment |
| b | Changed automatically at each new registration |
| c | Changed automatically at each new call |
| d | Lifetime dependent on Home Environment policy |
| e | Lifetime dependent on Serving Node policy |
| f | Lifetime dependent on User Policy |
| g | Lifetime Dependent on Manufacturer Policy |

Identities: These are shown in the tables, but unless TSG SA WG3 have a specific security issue with the length uniqueness etc there are shown in Italics as these are defined in other TSG working groups.  For example

| IMUI | * 96 bits? |
| --- | --- |

### 5.1.1	Home Environment Authentication Center  HE/AuC

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| Long Term Secret Key | K | 128 bits | a |
| User 1 permanent identity | IMUI | * |  |
| User n permanent identity | IMUI | * |  |
| Optional Enhanced user identity confidentiality |  |  |  |
| Home Environment Id | He_id | 32 bit | a |
| user 1 |  |  |  |
| Group Key | GK | 128 bits | a |
| Group Identity | GI | 32 bits | d |
| Sequence for UIC | SEQ_UIC | 32 bits | c |
| Encrypted IMUI | EMUI | 128 bit | c |
| user n |  |  |  |
| Group Key | GK | 128 bits | a |
| Group Identity | GI | 32 bits | d |
| Sequence for UIC | SEQ_UIC | 32 bits | c |
| Encrypted IMUI | EMUI | 128 bit | c |
| Network Domain Security |  |  |  |
| Network’s own Private Key | PVTK | 2048 bits | d,e |
| PKR1 Public Key for network #1 | PUBK1 | 2048 bits | d,e |
| PKRn Public Key for network #n | PUBKn | 2048 bits | d,e |
| Symmetric Send Key #i for sending data from X to Y | KSXY(i) | 128 bits | d,e |
| Symmetric Send Key #j for sending data from Y to X | KSYX (j) | 128 bits | d,e |
| Session key Sequence Number (for sending data from X to Y) | I | 32 – 64 bits | d,e |
| Session key Sequence Number (for sending data from Y to X) | j | 32 – 64 bits | d,e |
| Unpredictable Random Value generated by X | RNDX | 128 bits | c |
| Unpredictable Random Value generated by Y | RNDy | 128 bits | c |

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| Time Variant Parameter | TVP | 64 bits | c |
| Network Operator Identifier A, B | X, Y | 32 bits | a |
| Network-Wide User Traffic Confidentiality |  |  |  |
| Access link cipher key (traffic) | Ka | 128 bits | c |
| Access link cipher key (signaling) | Ka’ | 128 bits | c |
| Received access link cipher key | Kb | 128 bits | c |
| Received access link cipher key (signaling) | Kb’ | 128 bits | c |
| NWUTC Session key | KS | 128 bits | c |

### 5.1.2	Serving Node Visited Location Register SN/VLR

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| Registered User1 Permanent Identity | IMUI |  |  |
| Registered User1 Old Temporary Identity | TMUIo | * |  |
| Registered User1 New Temporary Identity | TMUIn | * |  |
| Optional Enhanced user identity confidentiality for user 1 |  |  |  |
| Encrypted IMUI | EMUI | 128 bit | c |
| Home Environment Id | He_id | 32 bit | a |
| Group Identity | GI | 32 bits | d |
| Registered User n Permanent Identity | IMUI |  |  |
| Registered User n Old Temporary Identity | TMUIo | * |  |
| Registered User n New Temporary Identity | TMUIn | * |  |
| Optional Enhanced user identity confidentiality for user n |  |  |  |
| Encrypted IMUI | EMUI | 128 bit | c |
| Home Environment Id | He_id | 32 bit | a |
| Group Identity | GI | 32 bits | d |

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| Network Domain Security |  |  |  |
| Symmetric Send Key #i for sending data from X to Y | KSXY(i) | 128 bits | d,e |
| Symmetric Send Key #j for sending data from Y to X | KSYX (j) | 128 bits | d,e |
| Network-Wide User Traffic Confidentiality |  |  |  |
| Access link cipher key (traffic) | Ka | 128 bits | c |
| Access link cipher key (signaling) | Ka’ | 128 bits | c |
| Received access link cipher key | Kb | 128 bits | c |
| Received access link cipher key (signaling) | Kb’ | 128 bits | c |

### 5.1.3	Radio Node Controller RNC

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| Cipher Key – Circuit switched | CKCS | 128 bits | c |
| Cipher Key – Packet switched | CKPS | 128 bits | c |
| Integrity key | IK | 128 bits | c |
| Access link cipher key (traffic) | Ka | 128 bits | c |
| Access link cipher key (signaling) | Ka’ | 128 bits | c |
| Received access link cipher key | Kb | 128 bits | c |
| Received access link cipher key (signaling) | Kb’ | 128 bits | c |

### 5.1.3	Mobile Equipment user Identify Module USIM

| Name | Symbol | Parameter Length actual or min-max | Lifetime |
| --- | --- | --- | --- |
| Long Term Secret Key | K | 128 bits | a |
| User Permanent Individual Identity | IMUI | * |  |
| Registered User Old Temporary Identity | TMUIo | * |  |
| Registered User New Temporary Identity | TMUIn | * |  |
| Optional Enhanced user identity confidentiality |  |  |  |
| Home Environment Id | He_id | 32 bits | a |
| Group Key | GK | 128 bits | a |
| Group Identity | GI | 32 bits | d |
| Sequence for UIC | SEQ_UIC | 32 bits | c |
| Encrypted IMUI | EMUI | 128 bits | c |
| Cipher Key – Circuit switched | CKCS | 128 bits | c |
| Cipher Key – Packet switched | CKPS | 128 bits | c |
| Integrity Key | IK | 128 bits | c |
| Key set identifier | KSI | 32 bits | c |
| Cipher Key Lifetime | LIFECKCS | 32 bits | d |
| Integrity Key Lifetime | LIFEIKCS | 32 bits | d |
| Cipher Key Use Counter – Circuit switched | USECKCS | 32 bits | a |
| Integrity Key Use Counter – Circuit switched | USEIKCS | 32 bits | a |
| User-User authentication User Name 1 | UN | 32 bits | f |
| User Response | URUN | 32 bits | f |
| Expected User Response | XURUN | 32 bits | f |
| 1st New User Response | NUR1UN | 32 bits | f |
| 2nd New User Response | NUR2UN | 32 bits | f |
| User Authentication Failure Counter | UUAFAILUN | 32 bits | a |
| User Authentication Failure limit | UUAFLIMUN | 32 bits | d |
| User Authentication Enable Status | UUASTATEUN | 1 bit | f |
| User Authentication Block | UUABLOCKUN | 1 bit | f |

| Name | Symbol |  | Parameter Length (actual or min-max) |  | Lifetime |
| --- | --- | --- | --- | --- | --- |
| Terminal Lock User Name 1 | UN1 |  | 32 bits |  | f |
| Terminal Lock User Name 2 | UN2 |  | 32 bits |  | f |
| Terminal- USIM Lock Password | V1 |  | 32 bits |  | f |
| Lock Disable Password | V2 | 32 bits |  | f |  |
| Lock Enable/Disable Failure Counter | LOCKFAIL | 32 bits |  | a |  |
| Terminal Lock Counter | LOCKCOUNT | 32 bits |  | a |  |
| Lock Enable/Disable Failure Counter Limit | LOCKEDFLIM | 32 bits |  | d |  |
| Terminal Lock Failure Counter Limit | LOCKFLIM | 32 bits |  | d |  |
| Terminal Lock Enable Status | LOCKSTATE | 1 bit |  | f |  |
| Terminal Lock | LOCK | 1 bit |  | f |  |

### 5.1.4	Mobile Equipment ME

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| Terminal- USIM Lock Password | XV1 | 32 bits | f |
| Lock Disable Password | XV2 | 32 bits | f |
| Cipher Key Circuit Switched | CKCS | 128 bits | c |
| Cipher Key Packet Switched | CKPS | 128 bits | c |
| Integrity Key | IK | 128 bits | c |
| Access link cipher key (traffic) | Ka | 128 bits | c |
| Received access link cipher key | Kb | 128 bits | c |
| NWUTC Session key | KS | 128 bits | c |

## 5.2	Sequence Number Mechanism

### 5.2.1	Home Environment Authentication Center  HE/AuC

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| Sequence Number | SQN | 32-64 bits | d |
| Anonymity Key | AK | 32-64 bits | d |
| Message Authentication Code | MAC | 64 bits |  |
| AV1 Random Number | RAND | 128 bits | c |
| Expected Response | XRES | 64 bits | c |
| Cipher Key | CK | 128 bits | c |
| Integrity Key | IK | 128 bits | c |
| Authentication Token | AUTN | 97- 129 bits | c |
| AVn Random Number | RAND | 128 bits | c |
| Expected Response | XRES | 64 bits | c |
| Cipher Key | CK | 128 bits | c |
| Integrity Key | IK | 128 bits | c |
| Authentication Token | AUTN | 97- 129 bits | c |
| Key set identifier | KSI | 32 bits | d |
| Message authentication Code | MAC | 64 bits | c |
| Anonymity Key | AK | 32-64 bits | c |
| Mode | MODE | 1 bit | c |
| Option 1 |  |  |  |
| User 1 Counter- Circuit switched | SQNHE/CSHEhe | 32 bits | d |
| User 1 Counter – Packet switched | SQNHE/PS | 32 bits | d |
| User n Counter- Circuit switched | SQNHE/CSHEhe | 32 bits | d |
| User n Counter – Packet switched | SQNHE/PS | 32 bits | d |
| Option 2 |  |  |  |
| Global Counter | SQNhe | 32 bits | d |

The Authentication center will store 10 Authentication vectors

### 5.2.2	Serving Node Visited Location Register SN/VLR

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| AV1 Random Number | RAND | 128 bits | c |
| Expected Response | XRES | 64 bits | c |
| Cipher Key | CK | 128 bits | c |
| Integrity Key | IK | 128 bits | c |
| Authentication Token | AUTN | 97-129 bits | c |
| AVn Random Number | RAND | 128 bits | c |
| Expected Response | XRES | 64 bits | c |
| Cipher Key | CK | 128 bits | c |
| Integrity Key | IK | 128 bits | c |
| Authentication Token | AUTN | 97 –129 bits | c |

### 5.2.3	Radio Node Controller 	RNC

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| See Common items section 5.1 |  |  |  |

### 5.2.4	Mobile Equipment user identity Module USIM

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| Received Random Number | RAND | 128 bits | c |
| Received Authentication Token | AUTN | 97–129 bits | c |
| Computed Anonymity Key | AK | 32-64 bits | c |
| Computed Message authentication Code | XMAC | 64 bits | c |
| Retrieved Sequence Number | SQN | 32-64 bits | c |
| Computed Response | RES | 32-64 bits | c |
| - and common items – section 5.1 |  |  |  |
| Option 1 |  |  |  |
| Counter- Circuit switched | SQNMS/CSHEhe | 32 bits | d |
| Counter – Packet switched | SQNMS/PS | 32 bits | d |
| Option 2 |  |  |  |
| Sequence Number Window- Circuit switched | SQNMS/CS - WHEhe | 10 | c |
| Sequence Number Window- Packet switched | SQNMS/PS - W | 10 | c |
| Option 3 |  |  |  |
| Sequence Number List – Circuit switched | LISTSQNCS | 10 | c |
| Sequence Number List – Packet switched | LISTSQNPS | 10 | c |

### 5.2.5	Mobile Equipment ME

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| See common items – section 5.1 |  |  |  |

# 6	Location of Security Functions

## 6.1		Sequence Number Mechanism

### 6.1.1	Home Environment Authentication Center HE/AuC

| Name | Symbol | Input Parameters |
| --- | --- | --- |
| Algorithms |  |  |
| Message Authentication Function | F1 | Input: K,SQN,RAND,MODE Output: MAC |
| Message Authentication Function | F2 | Input: K,RAND Output: XRES |
| Cipher Key Generating Function | F3 | Input: K,RAND Output: CK |
| Integrity Key Generating Function | F4 | Input: K, RAND Output: IK |
| Anonymity Key Generating Function | F5 | Input: K, RAND Output: AK |
| Identity Decryption Function | F7 | Input: GK, EMUI Output: SEQ_UIC IMUI |
| Block Encryption Algorithm for Network Operators | BEANO | Input: KSXY, Data Output: Encrypted data |
| Secure Hash Function | HASH | Input: Data Output: hashed data |
| Public Key encryption Function | E_PK(data) | Input: PK, data Output: edata |
| Public Key decryption Function | D_SK(edata) | Input: SK, edata Output: data |

### 6.1.2	Serving Node Visited Location Register SN/VLR

| Name | Symbol | Input Parameters |
| --- | --- | --- |
| Algorithms |  |  |
| Block Encryption Algorithm for Network Operators | BEANO | Input: KSXY, Data Output: Encrypted data |
| Secure Hash Function | HASH | Input: Data Output: hashed data |
| Public Key encryption Function | E_PK(data) | Input: PK, data Output: edata |
| Public Key decryption Function | D_SK(edata) | Input: SK, edata Output: data |

### 6.1.3	Radio Node Controller 	RNC

| Name | Symbol | Input Parameters |
| --- | --- | --- |
| Algorithms |  |  |
| Encryption algorithm | UEA | Input: Count, Direction Bearer, Length, CK Output: Keystream |
| Integrity algorithm | UIA | Input: Message, Count, Fresh, IK Output: MAC |

### 6.1.4	Mobile Equipment user identity Module USIM

| Name | Symbol | Input Parameters |
| --- | --- | --- |
| Algorithms |  |  |
| Message Authentication Function | F1 | Input: K,SQN,RAND, MODE Output: XMAC |
| Message Authentication Function | F2 | Input: K,RAND Output: RES |
| Cipher Key Generating Function | F3 | Input: K,RAND Output: CK |
| Integrity Key Generating Function | F4 | Input: K,RAND Output: IK |
| Anonymity Key Generating Function | F5 | Input: AK,SQNAK Output: SQN |
| Integrity algorithm | UIA |  |

### 6.1.5	Mobile Equipment ME

| Name | Symbol | Input Parameters |
| --- | --- | --- |
| Algorithms |  |  |
| Encryption algorithm | UEA | Input: Count, Direction Bearer, Length, CK Output: Keystream |
| Integrity algorithm | UIA | Input: Message, Count, Fresh, IK Output: MAC |
| Network-wide user traffic confidentiality Algorithm | UNA | Input: IV, KS Output: Keystream |

# Appendix 1 Temporary Key Mechanism

When the mobile first requests service from the SN/VLR, a random seed RSu created by the user (USIM or terminal) is included in the request message.  The message including RSu is forwarded to the HE/AuC, which generates its own random challenge RSn.  An authentication vector is returned to the SN/VLR.  The vector contains {RSn, RES1, XRES2, KT}, where RES1 is the response to the user’s challenge, XRES2 is the response to the network’s challenge which is expected from the user, and KT is the temporary authentication key shared with the SN/VLR.  The network’s challenge RSn and the network authentication response RES1 are sent to the MS.  If the MS verifies RES1, thereby authenticating the identity of the network, it responds with RES2 and generates the new temporary key KT.  The SN/VLR then verifies that RES2 equals XRES2, thereby authenticating the identity of the USIM, and stores the new temporary key KT.  Furthermore, both the USIM and the SN/VLR immediately use KT with the random seeds RSu and RSn to generate the first session keys CK and IK. This process is shown in Figure 4 below.

Figure 5 shows how the SN/VLR can offer secure service to the USIM without reference to the home system HE/AuC by using the temporary key KT.

Figure 4: Temporary Key Generation Protocol

Figure 5. Locally authenticated session key agreement

## A1	Security Information stored

### A1.1	Home Environment Authentication Centre  HE/AuC

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| Dynamic Information |  |  |  |
| Random Seed User | RSU | 128 bits | b |
| AV1Random Seed Network | RSN | 128 bits | b |
| Response to User Challenge RSU | RES1HEhe | 32-64 bits | b |
| Response to Nwk Challenge RSN | XRES2 | 64 bits | b |
| Temporary Key | KTHEhe | 128 bits | b |
| AVn Random Seed Network | RSN | 128 bits | b |
| Response to User Challenge RSU | RES1HEhe | 32-64 bits | b |
| Response to Nwk Challenge RSN | XRES2 | 64 bits | b |
| Temporary Key | KTHEhe | 128 bits | b |
| Fixed Initial Value | PAR1 | TBD | a |
| Fixed Initial Value | PAR2 | TBD | a |
| Fixed Initial Value | PAR3 | TBD | a |
| Fixed Initial Value | PAR4 | TBD | a |
| Fixed Initial Value | PAR5 | TBD | a |
| - and common items – section 5.1 |  |  |  |

### A1.2	Serving Node Visited Location Register SN/VLR

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| Dynamic Information |  |  |  |
| Temporary Key | KT | 128 bits | b |
| Random Seed User | RSU | 128 bits | b |
| Random Seed Network | RSN | 128 bits | b |
| Response to Users Challenge | RES1HEhe | 32-64 bits | b |
| Response to Network Challenge | RES2 | 32-64 bits | b |
| Response to Network Challenge | XRES2 | 64 bits | b |
| Cipher Key | CK* | 128 bits | b |
| Integrity Key | IK* | 128 bits | b |
| Response to SN/VLR challenge (local) | RES | 32-64 bits | b |
| Expected response to challenge | XRES | 64 bits | b |

* May be computed at HE/AuC

### A1.3	Radio Node Controller RNC

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| See common items – section 5.1 |  |  |  |

### A1.4	Mobile Equipment user identity Module USIM

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| Dynamic Information |  |  |  |
| Temporary Key | KT | 128 bits | b |
| Random Seed User | RSU | 128 bits | b |
| Random Seed Network | RSN | 128 bits | b |
| Computed Response ( local authent.) | RES | 32-64 bits | b |
| Response to Users Challenge | RES1HEhe | 32-64 bits | b |
| Response to Network Challenge | RES2 | 32-64 bits | b |
| Expected response from SN/VLR | XRES1 | 64 bits | b |
| - and common items – section 5.1 |  |  |  |

### A1.5	 Mobile Equipment

| Name | Symbol | Parameter Length (actual or min-max) | Lifetime |
| --- | --- | --- | --- |
| See common items – section 5.1 |  |  |  |

## A2	Location of Security Functions

### A2.1	Home Environment Authentication Centre HE/AuC

| Name | Symbol | Input Parameters |
| --- | --- | --- |
| Algorithms |  |  |
| Key Generating Function | F1 | Input: K, RSU, RSN Output: KT |
| Message Authentication Function | F2 | Input: K, RSU, RSN Output: RES1 |
| Message Authentication Function | F3 | Input: K, RSU , RSN Output: XRES1 |
| Identity Decryption Function | F7 | Input: GK, EMUI Output: SEQ_UIC IMUI |
| Block Encryption Algorithm for Network Operators | BEANO | Input: KSXY, Data Output: Encrypted data |
| Secure Hash Function | HASH | Input: Data Output: hashed data |
| Public Key encryption Function | E_PK(data) | Input: PK, data Output: edata |
| Public Key decryption Function | D_SK(edata) | Input: SK, edata Output: data |

### A2.2	Serving Node Visited Location Register SN/VLR

| Name | Symbol | Input Parameters |
| --- | --- | --- |
| Algorithms |  | * May be computed at HE/AuC |
| Message Authentication Function (local authentication only) | F3 | Input: KT, RSU, RSN Output: XRES |
| Cipher Key Generating Function | F4 | Input: KT, RSU, RSN Output: CK* |
| Integrity Key Generating Function | F5 | Input: KT, RSU,RSN Output: IK* |
| Block Encryption Algorithm for Network Operators | BEANO | Input: KSXY, Data Output: Encrypted data |
| Secure Hash Function | HASH | Input: Data Output: hashed data |
| Public Key encryption Function | E_PK(data) | Input: PK, data Output: edata |
| Public Key decryption Function | D_SK(edata) | Input: SK, edata Output: data |

### A2.3	Radio Node Controller 	RNC

| Name | Symbol | Input Parameters |
| --- | --- | --- |
| Algorithms |  |  |
| Encryption algorithm | UEA | Input: Count, Direction Bearer, Length, CK Output: Keystream |
| Integrity algorithm | UIA | Input: Message, Count, Fresh, IK Output: MAC |

### A2.4	Mobile Equipment user identity Module USIM

| Name | Symbol | Input Parameters |
| --- | --- | --- |
| Algorithms |  |  |
| Key generating function | F1 | Input: K, RSU, RSN Output: KT |
| Message Authentication Function | F2 | Input: K, RSU, RSN Output: XRES1 |
| Message Authentication Function | F3 | Input: K, RSU, RSN Output: RES2 |
| Message Authentication Function ( for local authentication) | F3 | Input: KT, RSU, RSN Output: RES |
| Cipher Key Generating Function | F4 | Input: KT, RSU, RSN Output: CK |
| Integrity Key Generating Function | F5 | Input: KT,RSU RSU RSN Output: IK |

### A2.5	Mobile Equipment ME

| Name | Symbol | Input Parameters |
| --- | --- | --- |
| Algorithms |  |  |
| Encryption algorithm | UEA | Input: Count, Direction Bearer, Length, CK Output: Keystream |
| Integrity algorithm | UIA | Input: Message, Count, Fresh, IK Output: MAC |
| Network-wide user traffic confidentiality Algorithm | UNA | Input: IV, KS Output: Keystream |

# Appendix 2	 Document history

| Document history |  |  |
| --- | --- | --- |
| 0.0.0 | 2nd May 1999 | Initial draft: Rapporteur- Colin Blanchard |
| 0.0.1 | 28th May 1999 | After review at TSG SA WG3 #3 Bonn 11- 12th May 1999 |
| 0.0.2 | 11th June 1999 | To incorporate comments from Takeshi Igarashi, Adam Berenzweig, Benno Tietz, Takeshi Chikazawa |