3G TS 33.XXX V0.1.1 (1999-06)

3GPP TSG SA WG3 #4																		TSGS3#4(99)147

London, UK, 16-18 June 1999

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

3G Security;

Cryptographic Algorithm Requirements

(3G TS 33.105 version 0.1.1)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

Reference

DTS/TSGS-0333120U

Keywords

Security, Authentication and key agreement, Ciphering Algorithm,
 Message Authentication Code Algorithm

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

# Contents

Foreword	5

1	Scope	5

2	References	5

3	Definitions and abbreviations	5

3.1	Definitions	5

3.2	Symbols	5

3.3	Abbreviations	6

4	General algorithm requirements	6

4.1	Resilience	6

4.2	World-wide availability and use	6

5	Functional algorithm requirements	7

5.1	Authentication and key agreement	7

5.1.1	Overview	7

5.1.2	 Use	8

5.1.3	Allocation	8

5.1.4	Extent of standardisation	8

5.1.5	Implementation and operational considerations	8

5.1.6	Type of algorithm	9

5.1.6.1	f0	9

5.1.6.2	f1	9

5.1.6.3	f2	9

5.1.6.4	f3	9

5.1.6.5	f4	9

5.1.6.6	f5	9

5.1.7	Interface	9

5.1.7.1	K	9

5.1.7.2	RAND	10

5.1.7.3	SQN	10

5.1.7.4	MODE	10

5.1.7.6	MAC-A (equivalent for XMAC-A)	10

5.1.7.7	RES (or XRES)	10

5.1.7.8	CK	10

5.1.7.9	IK	10

5.1.7.10	AK	11

5.2	User identity confidentiality	11

5.2.1	Overview	11

5.2.2	Use	11

5.2.3	Allocation	11

5.2.4	Extent of standardisation	11

5.2.5	Implementation and operational considerations	11

5.2.6	Type of algorithm	12

5.2.6.1	f6	12

5.2.6.2	f7	12

5.2.7	Interface	12

5.2.7.1	GK	12

5.2.7.2	SEQ_UIC	12

5.1.7.3	IMUI	12

5.2.7.4	EMUI	12

5.3	Data confidentiality	13

5.3.1	Overview	13

5.3.2	Use	13

5.3.3	Allocation	13

5.3.4	Extent of standardisation	13

5.3.5	Implementation and operational considerations	14

5.3.6	Type of algorithm	14

5.3.7	Interfaces to the algorithm	14

5.3.7.1	CK	14

5.3.7.2	COUNT	14

5.3.7.3	BEARER	14

5.3.7.4	DIRECTION	14

5.3.7.5	LENGTH	15

5.3.7.6	KEYSTREAM	15

5.3.7.7	PLAINTEXT	15

5.3.7.8	CIPHERTEXT	15

5.4	Data integrity	15

5.4.1	Overview	15

5.4.2	Use	16

5.4.3	Allocation	16

5.4.4	Extent of standardisation	16

5.4.5	Implementation and operational considerations	16

5.4.6	Type of algorithm	16

5.4.7	Interface	16

5.4.7.1	IK	16

5.4.7.2	COUNT	17

5.4.7.3	FRESH	17

5.4.7.4	MESSAGE	17

5.4.7.5	MAC-I (and equivalently XMAC-I)	17

6	Use of the algorithm specifications	17

6.1	Ownership	18

6.2	Design authority	18

6.3	Users of the specification	18

6.4	Licensing	18

6.4	Management of the specification	18

7	Algorithm specification and test data requirements	18

7.1	Specification of the algorithm	18

7.2	Design conformance test data	18

7.3	Algorithm input/output test data	19

7.4	Format and handling of deliverables	19

8	Quality assurance requirements	19

8.1	Quality assurance for the algorithm	19

8.2	Quality assurance for the specification and test data	19

8.3	Design and evaluation report	19

9	Summary of the design authority deliverables	20

Annex A: Functions for alternative AKA protocol	21

Annex B: History	26

Annex C: Unspecified values	27

# Foreword

- TBA

# 1	Scope

This specification constitutes a requirements specification for the security functions which may be used to provide the network access security features defined in [1].

The specification covers the intended use of the functions, the technical requirements on the functions and the requirements as regards standardization.

For those functions that require standardization, it also covers the intended use of the algorithm specification, the requirements on test data, and quality assurance requirements on both the algorithm and its documentation.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

[1] 	3G TS 33.102: “3rd Generation Partnership Project (3GPP); Technical Specification Group (TSG) SA; 3G Security; Security Architecture”.

[2]	Wassenaar Arrangement.

# 3	Definitions, symbols and abbreviations

## Definitions

For the purposes of the present document, the following definitions apply:

Confidentiality: The property that information is not made available or disclosed to unauthorised individuals, entities or processes.

Data integrity: The property that data has not been altered in an unauthorised manner.

Data origin authentication: The corroboration that the source of data received is as claimed.

Entity authentication: The provision of assurance of the claimed identity of an entity.

Key freshness:  A key is fresh if it can be guaranteed to be new, as opposed to an old key being reused through actions of either an adversary or authorised party.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

||	Concatenation

Å	Exclusive or

f0	random challenge generating function

f1	network authentication function

f2	user authentication function

f3	cipher key derivation function

f4	integrity key derivation function

f5	anonymity key derivation function.

f6	user identity encryption function

f7	user identity decryption function

f8	access link encryption function

f9	access link message authentication function

## 3.3	Abbreviations

For the purposes of the present document, the following abbreviations apply:

3GPP	3rd Generation Partnership Project

AK	Anonymity key

AuC	Authentication Centre

AUTN	Authentication token

CK	Cipher key

EMUI	Encrypted Mobile User Identity

GK	User group key

IK	Integrity key

IMUI	International Mobile User Identity

IPR	Intellectual Property Right

MAC	Medium access control (sublayer of Layer 2 in RAN)

MAC	Message authentication code

MAC-A	MAC used for authentication and key agreement

MAC-I	MAC used for data integrity of signalling messages

PDU	Protocol data unit

RAND	Random challenge

RES	User response

RLC	Radio link control (sublayer of Layer 2 in RAN)

RNC	Radio network controller

SEQ_UIC	Sequence for user identity confidentiality

SDU	Signalling data unit

SQN	Sequence number

UE	User equipment

USIM	User Services Identity Module

XMAC-A	Expected MAC used for authentication and key agreement

XMAC-I	Expected MAC used for data integrity of signalling messages

XRES	Expected user response

# General algorithm requirements

## Resilience

The functions should be designed with a view to its continued use for a period of at least 20 years. Attacks with a work load significantly less than exhaustive key search through the key space should be impossible.

The designers of above functions should design algorithms to a strength that reflects the above qualitative requirements.

## World-wide availability and use

Legal restrictions on the use or export of equipment containing cryptographic functions may prevent the use of such equipment in certain countries.

It is the intention that UE and USIMs which embody such algorithms should be free from restrictions on export or use, in order to allow the free circulation of 3G terminals. Network equipment, including RNC and AuC, may be expected to come under more stringent restrictions. It is the intention is that RNC and AuC which embody such algorithms should be exportable under the conditions of the Wassenaar Arrangement [2].

Existing export restrictions should be taken into account when designing the algorithms.

# Functional algorithm requirements

## Authentication and key agreement

### 5.1.1	Overview

The mechanism for authentication and key agreement described in clause 6.3 of [1] requires the following cryptographic functions:

f0	the random challenge generating function;

f1	the network authentication function;

f2	the user authentication function;

f3	the cipher key derivation function;

f4	the integrity key derivation function;

f5	the anonymity key derivation function.

Figure 1 illustrates the use of the cryptographic functions f0—f5 at the AuC.

Figure 1: Functions for authentication and key agreement in the AuC

The input parameters to the algorithms are the long-term secret Key (K), the SeQuence Number (SQN), a mobility management mode indicator (MODE) and a random challenge (RAND) generated by f0. Based on K and RAND f2—f5 compute an eXpected RESponse (XRES), a Cipher Key (CK), an Integrity Key (IK) and an Anonymity Key (AK) which is bitwise XOR-ed with SQN. Based on K, RAND, SQN and MODE, f1 derives a Message Authentication Code for network to user Authentication (MAC-A).

Figure 2 illustrates the use of the cryptographic functions f1—f5 in the USIM.

Figure 2: Functions for authentication and key agreement in the USIM

This time the Anonymity Key (AK) has to be computed in advance to retrieve SQN from SQN  AK. From there on, f1 is used to derive XMAC-A from K, RAND, SQN and MODE and f2—f4 are used to derive RES, CK and IK in the same way as the AuC derived MAC-A, XRES, CK and IK.

Note:	An alternative mechanism for authentication and key agreement that requires a different set of cryptographic functions is included in Annex D of [1]. The requirements on the cryptographic functions required for that mechanism are described in Annex A of this specification.

### 5.1.2		Use

The functions f0—f5 shall only be used to provide mutual entity authentication between USIM and AuC, derive keys to protect user and signalling data transmitted over the radio access link and conceal the sequence number to protect user identity confidentiality.

### 5.1.3	Allocation

The functions f1—f5 are allocated to the Authentication Centre (AuC) and the USIM. The function f0 is allocated to the AuC.

### 5.1.4	Extent of standardisation

The functions f0—f5 are proprietary to the home environment.

### 5.1.5	Implementation and operational considerations

The functions f1—f5 shall be designed so that they can be implemented on an IC card equipped with a X1-bit microprocessor running at X2 MHz with X3 kbits of memory and produce AK, MAC-A and RES in less than X4 ms, and CK and IK in less than X5 ms.

The functions f0—f5 shall be designed so that they can be implemented either in software in the AuC on a X6-bit microprocessor running at X7 MHz and X8 kbits of memory and produce MAC-A, XRES, CK, IK and AK in less than X9 ms.

### 5.1.6	Type of algorithm

#### 5.1.6.1	f0

f0: the random challenge generating function

f0: 		(internal state)  RAND

f0 should be pseudo random number generating function.

#### 5.1.6.2	f1

f1: the network authentication function

f1: 		(K; RAND, SQN, MODE)  MAC-A (or XMAC-A)

f1 should be a MAC function. In particular, it shall be computationally infeasible to derive K from knowledge of RAND, SQN, MODE and MAC-A (or XMAC-A).

#### 5.1.6.3	f2

f2: the user authentication function

f2: 		(K; RAND)  RES (or XRES)

f2 should be a MAC function. In particular, it shall be computationally infeasible to derive K from knowledge of RAND and RES (or XRES).

#### 5.1.6.4	f3

f3: the cipher key derivation function

f3: 		(K; RAND)  CK

f3 should be a key derivation function. In particular, it shall be computationally infeasible to derive K from knowledge of RAND and CK.

#### 5.1.6.5	f4

f4: the integrity key derivation function

f4: 		(K; RAND)  IK

f4 should be a key derivation function. In particular, it shall be computationally infeasible to derive K from knowledge of RAND and IK.

#### 5.1.6.6	f5

f5: the anonymity key derivation function

f5: 		(K; RAND)  AK

f5 should be a key derivation function. In particular, it shall be computationally infeasible to derive K from knowledge of RAND and AK.

### 5.1.7	Interface

#### 5.1.7.1	K

K: the user authentication key

K[0], K[1], …, K[127]

The length of K is 128 bits. The user authentication key K is a long term secret key stored in the USIM and the AuC.

#### 5.1.7.2	RAND

RAND: the random challenge

RAND[0], RAND[1], …, RAND[127]

The length of RAND is 128 bits.

#### 5.1.7.3	SQN

SQN: the sequence number

SQN[0], SQN[1], …, SQN[X10-1]

The length of SQN is between 32 and 64 bits. The AuC should include a fresh sequence number in each authentication token. The verification of the freshness of the sequence number by the USIM constitutes to entity authentication of the network to the user.

#### 5.1.7.4	MODE

MODE: the mobility management identifier

MODE[0]

The length of MODE is 1 bit. MODE identifies an instance of mobility management, when more than one mobility management is simultaneously active for a single user.

#### 5.1.7.6	MAC-A (equivalent for XMAC-A)

MAC-A: the message authentication code used for authentication of the network to the user

MAC-A[0], MAC-A[1], …, MAC-A[63]

The length of MAC-A is 64 bits. MAC-A authenticates the data integrity and the data origin of RAND, SQN and MODE. The verification of MAC-A by the USIM constitutes to entity authentication of the network to the user.

#### 5.1.7.7	RES (or XRES)

RES: the user response

RES[0], RES[1], …, RES[63]

or XRES: the expected user response

XRES[0], XRES[1], …, XRES[63]

The length of RES and XRES is 64 bits. RES and XRES constitute to entity authentication of the user to the network.

#### 5.1.7.8	CK

CK: the cipher key

CK[0], CK[1], …, CK[127]

The maximum length of CK is 128 bits. In case the effective key length of CK is smaller than 128 bits, the most significant bits of CK carry the key information.

#### 5.1.7.9	IK

IK: the integrity key

IK[0], IK[1], …, IK[127]

The maximum length of IK is 128 bits. In case the effective key length of IK is smaller than 128 bits, the most significant bits of IK carry the key information.

#### 5.1.7.10	AK

AK: the anonymity key

AK[0], AK[1], …, AK[X10-1]

The length of AK is between 32 and 64 bits. It equals the length of SQN.

## 5.2	User identity confidentiality

### 5.2.1	Overview

Figure 3 illustrates the use of the encryption function f6 to encrypt the IMUI and the sequence for user identity confidentialitr (SEQ_UIP) into an EMUI and the use of the decryption function f7 to decrypt the EMUI and retrieve the SEQ_UIP and the IMUI.

Figure 3: Encryption and decryption of the permanent user identity

The mechanism for user identity confidentiality that is described in annex B of [1] requires the following cryptographic functions:

f6	the user identity encryption function;

f7			the user identity decryption function.

### 5.2.2	Use

The functions f6 and f7 shall only be used to protect the confidentiality of the user identity when transmitted from USIM to AuC.

### 5.2.3	Allocation

The function f6 is allocated to the USIM. The function f7 is allocated to the Authentication Centre.

### 5.2.4	Extent of standardisation

The functions f6 and f7 are proprietary to the home environment.

### 5.2.5	Implementation and operational considerations

The function f6 shall be designed so that it can be implemented on an IC card equipped with a X1-bit microprocessor running at X2 MHz and with X3 kbits of memory and produce EMUI in less than X11 ms.

The functions f7 shall be designed so that they can be implemented in software in the AuC on a X6-bit microprocessor running at X7 MHz and X8 kbits of memory and produce SEQ_UIC || IMUI in less than X12 ms.

### 5.2.6	Type of algorithm

#### 5.2.6.1	f6

f6: the user identity encryption function

f6: 		(GK; SEQ_UIC || IMUI)  EMUI

f6 should be a block cipher.

#### 5.2.6.2	f7

f7: the user identity decryption function

f7: 		(GK; EMUI)  SEQ_UIC || IMUI

f7 should be a block cipher and the inverse function of f6, in the sense that

x = f7(y; f6(y; x)), 	for all valid x = SEQ_UIC || IMUI and all valid y = GK.

### 5.2.7	Interface

#### 5.2.7.1	GK

GK: the user group key

GK[0], GK[1], …, GK[X13-1]

The maximum length of the group key GK is X13 bits. The user group key GK is a long term secret key stored in several USIMs and in the AuC.

#### 5.2.7.2	SEQ_UIC

SEQ_UIC: the sequence for user identity confidentiality

SEQ_UIC[0], SEQ_UIC[1], …, SEQ_UIC[X14-1]

The length of SEQ_UIC is X14 bits. The SEQ_UIC is generated by the USIM and should be different each time so as to prevent traceability of a user.

#### 5.1.7.3	IMUI

IMUI: the international mobile user identity

IMUI[0], IMUI[1], …, IMUI[X15-1]

The length of the IMUI is X15bits. The IMUI is the permanent identity of the user, stored in the USIM and in the AuC.

#### 5.2.7.4	EMUI

EMUI: the encrypted mobile user identity

EMUI[0], EMUI[1], …, EMUI[X16-1]

The length of the EMUI is X16 bits.

## 5.3	Data confidentiality

### Overview

The mechanism for data confidentiality of user data and signalling data that is described in 6.4 of [1] requires the following cryptographic function:

f8	the access link encryption function.

Figure 4 illustrates the use of f8 to encrypt plaintext by applying a keystream using a bitwise XOR operation. The plaintext may be recovered by generating the same keystream using the same input parameters and applying it to the ciphertext using a bitwise XOR operation.

Figure 4: Ciphering user and signalling data transmitted over the radio access link

The input parameters to the algorithm are the Cipher Key (CK), a time dependent input (COUNT), the bearer identity (BEARER), the direction of transmission (DIRECTION) and the length of the keystream required (LENGTH). Based on these input parameters the algorithm generates the output keystream block (KEYSTREAM) which is used to encrypt the input plaintext block (PLAINTEXT) to produce the output ciphertext block (CIPHERTEXT).

### 5.3.2	Use

The function f8 shall only be used to protect the confidentiality of user data and signalling data sent over the radio access link between UE and RNC.

### 5.3.3	Allocation

The function f8 is allocated to the UE and the RNC.

Encryption will be applied in the Medium Access Control (MAC) sublayer and in the Radio Link Control (RLC) sublayer of the data link layer (Layer 2). It is assumed that synchronisation of the keystream will be based on the use of a physical layer (Layer 1) frame counter combined with a hyperframe counter introduced to avoid re-use of the keystream. It must be noted that these details are subject to change based on ongoing developments in 3GPP TSG SA3 (Service Aspects – Security Group) and 3GPP TSG RAN2 (Radio Architecture Network - Layer 2/3 Group).

### 5.3.4	Extent of standardisation

The function f8 shall be fully standardized.

### 5.3.5	Implementation and operational considerations

The algorithm should be designed to accommodate a range of implementation options including hardware and software implementations. For hardware implementations, it should be possible to implement one instance of the algorithm using less than X17 gates (working assumption).

A wide range of UE with different bearer capabilities is expected, so the encryption throughput requirements on the algorithm will vary depending on the implementation. However, based on the likely maximum user traffic data rates, it must be possible to implement the algorithm to achieve an encryption speed in the order of 2Mbit/s on the downlink and 1Mbit/s on the uplink.

The exact throughput requirements will depend on the RLC PDU / MAC SDU size and the number of RLC PDUs / MAC SDUs which may be sent in a single physical layer 10ms frame. In addition, within each 10ms frame, the algorithm will need to be reinitialised (or reinstantiated) for each different bearer and for each transmission direction.

The encryption throughput requirements should be met based on clock speeds upwards of 20MHz (typical clock speeds are expected to be much greater than this).

### 5.3.6	Type of algorithm

The function f8 should be a symmetric synchronous stream cipher.

### 5.3.7	Interfaces to the algorithm

#### 5.3.7.1	CK

CK: the cipher key

CK[0], CK[1], …, CK[127]

The algorithm should operate with key lengths up to 128 bits. The cipher key (CK) is structured data where the first n most significant bits are the key. The cipher key is established as part of the authentication and key agreement protocol specified in 6.3 of [1].

#### 5.3.7.2	COUNT

COUNT: a time dependent input.

COUNT[0], COUNT[1], …, COUNT[31]

The length of the COUNT parameter is 32 bits. It is assumed that sychronisation of the keystream will be based on the use of a physical layer (Layer 1) frame counter combined with a hyperframe counter introduced to avoid re-use of the keystream. This allows the keystream to be synchronised every 10ms physical layer frame. The exact structure of the COUNT parameter cannot be specified at present. However, it is assumed to be a 32 bit counter.

#### 5.3.7.3	BEARER

BEARER: the identity of the bearer to be encrypted.

BEARER[0], BEARER[1], …, BEARER[3]

The length o BEARER is  4 bits. The same cipher key may be used for different bearers simultaneously associated with a single user which are multiplexed onto a single 10ms physical layer frame. To avoid using the same keystream to encrypt more than one bearer, the algorithm shall generate the keystream based on the identity of the bearer.

#### 5.3.7.4	DIRECTION

DIRECTION: the direction of transmission of the bearer to be encrypted.

DIRECTION[0]

The length of DIRECTION is 1 bit. The same cipher key may be used for uplink and downlink channels simultaneously associated with a UE, which are multiplexed onto a single 10ms physical layer frame. To avoid using the same keystream to encrypt both uplink and downlink transmissions, the algorithm shall generate the keystream based on the direction of transmission.

An explicit direction value is required in preference to splitting the keystream segment into uplink and downlink portions to allow for asymmetric bearer services.

#### 5.3.7.5	LENGTH

LENGTH: the required length of keystream.

LENGTH[0], LENGTH[1], …, LENGTH[X18-1]

The length of LENGTH is X18 bits. For a given bearer and transmission direction the length of the plaintext block that is transmitted during a single physical layer frame may vary. The algorithm shall generate a keystream block of variable length based on the value of the length parameter.

The format of LENGTH cannot be specified at present since the number and sizes of RLC PDUs / MAC SDUs in each 10ms physical layer frame have not yet been fully specified. However, a maximum RLC PDU / MAC SDU size in the region of 1000 bits has been informally indicated by 3GPP TSG RAN2. The range of values of the length parameter will depend not only on the RLC PDU / MAC SDU size but also the number of RLC PDUs / MAC SDUs which may be sent in a single physical layer 10ms frame for a given bearer and transmission direction.

Not all values between the maximum and minimum values shall be required but it is expected that the ability to produce length values of whole numbers of octets between a minimum and a maximum value will be required.

#### 5.3.7.6	KEYSTREAM

KEYSTREAM: the output keystream.

KS [0], KS [1], …, KS [LENGTH-1]

The length of a keystream block equals the value of the input parameter LENGTH.

#### 5.3.7.7	PLAINTEXT

PLAINTEXT: the plaintext.

PT[0], PT[1], …, PT[LENGTH-1]

The length of a keystream block equals the value of the input parameter LENGTH.

This plaintext block consists of the payload of the particular RLC PDUs / MAC SDUs to be encrypted in a single 10ms physical layer frame for a given bearer and transmission direction. It may consist of user traffic or signalling data. The structure of the plaintext block cannot be specified at present.

#### 5.3.7.8	CIPHERTEXT

CIPHERTEXT: the ciphertext.

CT[0], CT[1], …, CT[LENGTH-1]

The length of a keystream block equals the value of the input parameter LENGTH.

## 5.4	Data integrity

### 5.4.1	Overview

The mechanism for data integrity of signalling data that is described in 6.6 of [1] requires the following cryptographic function:

f9	the access link message authentication function.

Figure 5 illustrates the use of the function f9 to derive a MAC-I from a signalling message.

Figure 5: Derivation of MAC-I (or XMAC-I) on a signalling message

The input parameters to the algorithm are the Integrity Key (IK), a time dependent input (COUNT), a random value generated by the network side (FRESH) and the signalling data (MESSAGE). Based on these input parameters the user computes with the function f9 the message authentication code for data integrity (MAC-I) which is appended to the message when sent over the radio access link. The receiver computes XMAC-I on the messages received in the same way as the sender computed MAC-I on the message sent.

### 5.4.2	Use

The MAC function f9 shall be used to authenticate the data integrity and data origin of signalling data transmitted between UE and RNC.

### 5.4.3	Allocation

The MAC function f9 is allocated to the UE and the RNC.

The exact position of MAC algorithm in the radio network architecture has not yet been fully specified. The current working assumption is that it will be closely integrated with the ciphering algorithm.

### 5.4.4	Extent of standardisation

The function f9 is fully standardized.

### 5.4.5	Implementation and operational considerations

The algorithm should be designed to accommodate a range of implementation options including hardware and software implementations.

### 5.4.6	Type of algorithm

The function f9 shall be a MAC function.

### 5.4.7	Interface

#### 5.4.7.1	IK

IK: the integrity key

- IK[0], IK[1], …, IK[127]

The algorithm should operate with key lengths up to 128 bits. The integrity key (IK) is structured data where the first n most significant bits are the key. The integrity key is established as part of the authentication and key management procedure specified in [1].

#### 5.4.7.2	COUNT

COUNT: a frame dependent input.

- COUNT[0], COUNT[1], …, COUNT[31]

The keystream should be initialised with a time dependent input parameter.

It is assumed that sychronisation of the keystream will be based on the use of a physical layer (Layer 1) frame counter combined with a hyperframe counter introduced to avoid re-use of the keystream. This allows the keystream to be synchronised every 10ms physical layer frame. The exact structure of the COUNT parameter cannot be specified at present. However, it is assumed to be a 32 bit counter.

[Note:	The same integrity key may be used for more than one connection. The user stores the last used hyperframe number and initialises the first hyperframe number of a new connection such that re-use of the same COUNT value is avoided. The user also makes sure that the hyperframe number does not wrap around. This network should compute its MACs on the basis of this COUNT value. This mechanism assure the user that the network is not replaying any old MACs.]

#### 5.4.7.3	FRESH

FRESH: a random number generated by the RNC.

- FRESH[0], FRESH[1], …, FRESH[31]

The same integrity key may be used for several consecutive connections. This FRESH value is an input to the algorithm in order to assure the network side that the user is not replaying old MAC-Is.

[Note:	The same integrity key may be used for more than one consecutive connection. Each time a connection is started, the network sends the user a random value FRESH. This user (and the network) should compute its MAC-Is on the basis of this FRESH value. This mechanism assures the network that the user is not replaying any old MAC-Is. The number of possible values of FRESH should be large enough to avoid collisions (re-use of the same FRESH more than once during the life time of a single integrity key.]

#### 5.4.7.4	MESSAGE

MESSAGE: the signalling data.

- MESSAGE[0], MESSAGE[1], …, MESSAGE[X19-1]

The maximum length of MESSAGE is X19.

#### 5.4.7.5	MAC-I (and equivalently XMAC-I)

MAC-I: the message authentication code for data integrity authentication

MAC-I[0], MAC-I[1], …, MAC-I[X20-1]

The length of MAC-I is X20 bits.

# 6	Use of the algorithm specifications

The purpose of this clause is to address ownership of the algorithm specification, to define which types of organisation are entitled to use the algorithm specification, and to outline how and under what conditions such organisations may obtain the specification.

## 6.1	Ownership

For those functions which require to be fully standardized, all copyright on the algorithm and test data specifications shall be owned jointly by the 3GPP partner organisations.

## 6.2	Design authority

The design authority for the algorithms that require standardisation shall be ETSI SAGE. For the design work on the 3G security algorithms ETSI SAGE may include experts from other 3GPP partner organisations.

## 6.3	Users of the specification

For those functions which require to be fully standardized, the algorithm specification shall be published as a 3GPP specification. It will be used by those who need the algorithm specification to build equipments or components which embody the algorithm.

## 6.4	Licensing

For those functions which require to be fully standardized, the use of the algorithm shall be subject to a license agreement which restricts the use of the algorithm as described in 5.3.2 and 5.4.2.

Users of the algorithm, and users of the algorithm specification, shall be required to sign the licence agreement. Appropriate licence agreements shall be drawn up by the 3GPP partner organisations.

Licences shall be royalty free. In addition, the licence agreement shall require users of the specification not to attempt to patent the algorithm or otherwise register an Intellectual Property Right (IPR) relating to the algorithm or its use.

## 6.4	Management of the specification

For those functions which require to be fully standardized, the algorithm specifications shall be published as a 3GPP specification. The algorithms will thus be open for public evaluation. It is recognised that this will leave the algorithms open to public criticism during the commercial operation of the system. The process of responding to public criticism will need to be handled carefully by an appropriate 3GPP body.

# 7	Algorithm specification and test data requirements

For those functions that require standardization, the design authority should provide four separate deliverables: a specification of the algorithm, a set of design conformance test data, a set of algorithm input/output test data and a design and evaluation report. Requirements on the specification and test data deliverables are given in this clause, those on the design and evaluation report in 9.3.

## 7.1	Specification of the algorithm

An unambiguous specification of the algorithm needs to be provided which is suitable for use by implementers of the algorithm.

The specification shall include an annex which provides simulation code for the algorithm written in ANSI C. The specification may also include an annex containing illustrations of functional elements of the algorithm.

## 7.2	Design conformance test data

Design conformance test data is required to allow implementers of the algorithm to test their implementations.

The design conformance test data needs to be designed to give a high degree of confidence in the correctness of implementations of the algorithm.

The design conformance test data shall be designed so that significant points in the execution of the algorithm may be verified.

## 7.3	Algorithm input/output test data

Algorithm input/output test data is required to allow implementers of the algorithm to test the algorithm as a “black box” function.

The input/output test data shall allow implementers of the algorithm to perform the necessary tests.

The input/output test data shall consist solely of data passed across the interfaces to the algorithm.

## 7.4	Format and handling of deliverables

The specification of the algorithm shall be produced on paper, and published as a 3GPP specification.

The algorithm input/output test data shall be produced on paper and on magnetic disc, and published by 3GPP. The document and disc shall be provided to 3GPP partner organisations.

# 8	Quality assurance requirements

This clause advises the design authority on measures needed to provide users of the algorithm with confidence that it is fit for purpose, and users of the algorithm specification and test data assurance that appropriate quality control has been exercised in their production.

The measures shall be recorded by the design authority in a design and evaluation report which shall be published as a 3GPP specification.

## 8.1	Quality assurance for the algorithm

Prior to its release to 3GPP, the algorithm needs to be approved as meeting the functional requirements specified in clause 7 by all members of the design authority.

## 8.2	Quality assurance for the specification and test data

Prior to delivery of the algorithm specification, two independent simulations of the algorithm needs to be made using the specification, and confirmed against test data designed to allow verification of significant points in the execution of the algorithm.

Design conformance and algorithm input/output test data needs to be generated using a simulation of the algorithm produced from the specification and confirmed as above. The simulation used to produce this test data needs to be identified in the test data deliverables and retained by the design authority.

## 8.3	Design and evaluation report

The design and evaluation report is intended to provide evidence to potential users of the algorithm, specification and test data that appropriate and adequate quality control has been applied to their production. The report shall explain the following:

-	the algorithm and test data design criteria;

-	the algorithm evaluation criteria;

-	the methodology used to design and evaluate the algorithm;

-	the extent of the mathematical analysis and statistical testing applied to the algorithm;

-	the principal conclusions of the algorithm evaluation;

-	the quality control applied to the production of the algorithm specification and test data.

The report shall confirm that all members of the design authority have approved the algorithm, specification and test data.

The report shall contain key conclusions from the commissioned closed evaluation of the algorithm.

# 9	Summary of the design authority deliverables

For those cryptographic functions that require standardisation, the design authority shall deliver:

- Specification of the algorithm;

- Design conformance test data;

- Algorithm input/output test data;

- Design and evaluation report.

All these documents shall be delivered to 3GPP for subsequent publication.

Annex A: Functions for alternative AKA protocol

## A.1	Scope

The mechanism described here achieves mutual authentication and key agreement between the USER (e.g.,USIM) and the AuC in the user’s HE, showing knowledge of a secret key K which is shared between and available only to these two parties.  The temporary authentication key generated during execution of the protocol is shared with the visited SN/VLR, and can be used subsequently with the local authentication and session key agreement protocol described in section D.2  of  Annex D of [1] or with the other local authentication mechanisms described in Annex D [1].  Additionally, the session keys for the first session are created during execution of the protocol.

Architectural detail can be obtained from Annex D of [1].

## A.2	Abbreviations

The following abbreviations are complementing the abbreviations defined above in section 3.3 of this document:

| KT | Temporary Authentication Key |
| --- | --- |
| RES1 | Response to the USER-generated Challenge (HE/AuC  SN/VLR  USER ) |
| RES2 | Response to the Network-generated Challenge (USER SN/VLR  HE/AuC) |
| RSn | Network Generated Challenge (HE/AuC  SN/VLR  USER ) |
| Rsu | USER-generated Challenge (USER  SN/VLR  HE/AuC) |
| SN | Serving Node |
| XRES1 | Expected Response to the Network-generated Challenge (RES1) |
| XRES2 | Expected Response to the User-generated Challenge (RES2) |

## A.3	General algorithm requirements

The requirements listed in the body of the document should also apply to the AKA protocol.  Only the differences will be described in this Annex.

## A.4	Functional algorithm requirements

### A.4.1	Overview

The mechanism for authentication and key agreement described in Annex D of [1] requires the following cryptographic functions:

| f1 | Temporary Authentication Key Generation Function (KT) |
| --- | --- |
| f2 | Function to Calculate Response to User Challenge (RES1) or the Expected Response to Network Challenge (XRES1) |
| f3 | Function to Calculate the Response to the Network Challenge (RES2) or the Expected Response to User Challenge (XRES2) |
| f4 | Function to Derive the Cipher Key (Ck) |
| f5 | Function to Derive the Integrity Key (Ik) |

Figure A.1 below illustrates the use of the cryptographic functions f1 – f5 at the HE/AuC, SN/VLR and USER.

Figure A.1  Functions for generating the Temporary Key, the Ciphering Key, the Integrity Key and the responses to the authentication challenges (X)RES1 and (X)RES2.

f1

f2*

f3*

RES2

XRES2

KT

RSnRSn

RSuRSu

K

The input parameters to the f1, f2, and f3(see note) algorithms are:

- the K (Authentication Key)

- (*note: when authentication is performed locally, between the  SN/VLR  and the User,  the K input for f2,f3   is replaced by the f1 derived Temporary Key – KT for deriving the (X)RES2)

- the RSu (Random Number – User)

- the RSn (Random Number – Network)

- The input parameters for the f4, f5 algorithms are:

- the Temporary Key (KT) generated by f1

- the RSu

- the RSn

### A.4.2	Use

The functions f1 – f5 shall be used only to provide mutual entity authentication between the User and the AuC (or the SN/VLR when the temporary authentication key KT is shared).  The derived keys shall protect the user and the transmission over the radio access link (voice or data).  The confidentiality of the user identity should be protected.

### A.4.3	Extend of standardisation

The f2 – f5 should be standardise by ETSI SAGE.   The f1 is proprietary to the home environment (or optionally standardised by ETSI SAGE).

### A.4.4	Allocation

The functions f1 – f5  are allocated to the Home Entity / Authentication Centre (AuC) and the USER (e.g., UIM). The f2 - f5 can also be allocated to the SN/VLR where the permanent key (K) is replaced, for the currently active registration period, by the Temporary Key (KT) generated at the HE/AuC by f1.

### A.4.5	Implementation and operational considerations

ATK implementation shall conform to the same requirements as described in section 5.1.5 of the document.

### A.4.6	Type of algorithms

#### A.4.6.1	f0

This function is a pseudo random number generator, as described above in this document.  It is used to generate a network random number (RSn) as well as the USER random number (RSu).

f0:  (internal state)  RSn

f0:  (internal state)  RSu

#### A.4.6.2	f1

Authentication function that generates the  temporary authentication key used, by the serving system, to generate the session ciphering, the integrity keys or perform local authentication.  This function is not shared with the SN/VLR and does not have to be standardized.

f1:  (K, RSu, RSn)  KT

It should be computationally unfeasible to derive the K from the knowledge of RSu, RSn. or/and KT.

#### A.4.6.3	f2

Network authentication function deriving the response and the expected response to an authentication challenge from the  AuC.

f2: (K, RSn, RSu)  RES1  (in the AuC)

f2: (K, RSn, RSu)  XRES1 (in the USER terminal)

When a local authentication session key agreement is executed, the KT is used in the computation instead of the K (the SN/VLR is sending the authentication challenge to the USER).

f2: (KT, RSn, RSu)  RES1  (in the SN/VLR)

f2: (KT, RSn, RSu)  XRES1 (in the USER terminal)

It should be computationally unfeasible to derive the K or KT from the knowledge of RSu, RSn. or/and (X)RES1.

#### A.4.6.4	f3

Network authentication function deriving the response and the expected response to an authentication challenge from the  USER terminal.

f3: (K, RSn, RSu)  RES2  (in the USER terminal)

f3: (K, RSn, RSu)  XRES2 (in the AuC)

When a local authentication session key agreement is executed, the KT is used in the computation instead of the K.

f3: (KT, RSn, RSu)  RES2  (in the USER terminal)

f3: (KT, RSn, RSu)  XRES2 (in the SN/VLR)

It should be computationally unfeasible to derive the K or KT from the knowledge of RSu, RSn. or/and (X)RES2.

#### A.4.6.5	f4

Function used to generate a session’s ciphering key used to encode the over the air conversation between the user’s terminal and the serving system.

f4: (KT, RSn, RSu)  Ck  (in the USER terminal)

f4: (KT, RSn, RSu)  Ck  (in the SN/VLR)

It should be computationally unfeasible to derive the K or KT from the knowledge of RSu, RSn. or/and Ck.

#### A.4.6.6	f5

Function used to generate a session’s integrity key used to encode the over the air signaling between the user’s terminal and the serving system.

f5: (KT, RSn, RSu)  Ik  (in the USER terminal)

f5: (KT, RSn, RSu)  Ik  (in the SN/VLR)

It should be computationally unfeasible to derive the K or KT from the knowledge of RSu, RSn. or/and Ik.

### A.4.7	Interface

The following additions are proposed to the interfaces described above in this document.:

#### A.4.7.1	KT

KT: the user temporary authentication key.

The length of the KT should be the same as specified  for K above.

#### A.4.7.2	RSn and RSu

RSn, RSu: random numbers

The length of RSn and RSu should be the same as specified for the RAND above.

#### A.4.7.3	RES1, XRES1,  RES2, XRES2

RES1 or XRES1: authentication challenge from the network or  the expected response from the user.

RES2 or XRES2: authentication challenge from the USER or  the expected network response.

The length of those parameters should be the same as specified for the RES above.

Annex B: History

| Document history |  |
| --- | --- |
| 4 February 1999 | Scope (draft) at 3GPP TSG-SA WG3 #1 |
| 24 February 1999 | Scope and Contents (draft) to 3GPP TSG-SA WG3 mailing list |
| 15 March 1999 | Version 0.0.1 to 3GPP TSG-SA WG3 #2 |
| 26 March 1999 | Version 0.0.2 at 3GPP TSG-SA WG3 #2 |
| 28 April 1999 | Version 0.0.3 to ESTI SAGE (and 3GPP TSG-SA WG3 mailing list) |
| 7 May 1999 | Version 0.0.4 to 3GPP TSG-SA WG3 #3 |
| 19 May 1999 | Version 0.0.5 to 3GPP TSG-SA WG3 mailing list (for editing meeting) |
| 27 May 1999 | Version 0.0.9 to 3GPP TSG-SA WG3 Adhoc meeting attendees |
| 31 May 1999 | Version 0.1.0 to 3GPP TSG-SA WG3 mailing list |
| 14 June 1999 | Version 0.1.1 to 3GPP TSG-SA WG3 #4 |

Annex C: Unspecified values

| Reference | Meaning | Range | Source |
| --- | --- | --- | --- |
| X1 | Bus width of the USIM processor (bit) |  | TSG T WG3 |
| X2 | Clock speed of the USIM processor (MHz) |  | TSG T WG3 |
| X3 | Memory size of the USIM (kbits) |  | TSG T WG3 |
| X4 | Response time for AK, MAC-A and RES (ms) |  | TSG SA WG2 |
| X5 | Response time for CK and IK (ms) |  | TSG SA WG2 |
| X6 | Bus width of the AuC processor (bit) |  | TSG CN |
| X7 | Clock speed of the AuC processor (MHz) |  | TSG CN |
| X8 | Memory size of the AuC (kbits) |  | TSG CN |
| X9 | Response time for authentication vector in AuC (ms) |  | TSG SA WG2 |
| X10 | Length of sequence number (bits) | 32—64 | TSG SA WG3 |
| X11 | Response time for EMUI computation in the USIM (ms) |  | TSG SA WG2 |
| X12 | Response time for SEQ_UIC ¦¦ IMUI in the AuC (ms) |  | TSG SA WG2 |
| X13 | Length of the group key (bits) | 128 | TSG SA WG3 |
| X14 | Length of SEQ_UIC (bits) | 32 | TSG SA WG3 |
| X15 | Length of IMUI (bits) |  | TSG SA |
| X16 | Length of EMUI (bits) | 128 | TSG SA WG3 |
| X17 | Number of gates required for hardware implementation of ciphering algorithm | 10 000 | TSG T WG3 TSG CN |
| X18 | Length of the field LENGTH for ciphering (bits) |  | TSG RAN WG2 |
| X19 | Maximum length of a signalling message (bits) |  | TSG SA WG3 TSG RAN WG2 |
| X20 | Length of MAC-I (bits) | 16—32 | TSG SA WG3 |

RSn and/or RSu can be a random number or a counter