| 3G CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T TS 33.102 |  |  |  |  |  |  | CR |  |  |  | Xxx |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  | 3.0.0 |  |  |  |
| 3G specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by 3G support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to TSG |  |  |  |  |  | SA#4 |  | for approval |  |  |  |  |  | X |  |  | (only one box should |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| list TSG meeting no. here  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  | be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Form: 3G CR cover sheet, version 1.0 The latest version of this form is available from: ftp://ftp.3gpp.org/Information/3GCRF-xx.rtf |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | USIM |  |  |  | ME |  |  |  |  |  |  |  |  | UTRAN |  |  |  |  |  |  | Core Network |  |  |  |  | X |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | Lucent Technologies Inc. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  | 17.06.1999 |  |  |  |  |
| Subject: |  |  | Criteria for Replacing the Authentication “Working Assumption” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3G Work item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A Corresponds to a correction in a 2G specification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change: |  |  | To provide the criteria for replacing the current authentication “working assumption” with the alternate method described in the annex D of TS 33.102 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  | Annex C of TS 33.102 – new paragraph “C.5” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other 2G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<--------- double-click here for help and instructions on how to create a CR.

- Annex C: Management of sequence numbers

This annex is devoted to the management of sequence numbers for the authentication and key agreement protocol.

## C.1	A mechanism using two individual counters on each side

This is the mechanism included in the main body of this specification.

## C.2	A mechanism using a global counter in the HE and two counters in the MS

In this mechanism the HLR/AuC keeps track of time, while the USIM keeps track of a counters for PS mode SQNMS/PS and a counter for CS mode SQNMS/CS.

The HLR/AuC may for instance use as a sequence number the number of seconds t that have elapsed since the start of the year 2000 (GMT). Then, a 32-bit sequence number will suffice for 136 years of operation. When an array of n authentication vectors is generated, the values t, t+1, …, t+n-1 could be used.

At the user end, SQN is treated as in the mechanism described under C.1.

Note 1:	When using a time-value to generate sequence numbers it may not be necessary to conceal the sequence number to avoid user identification.

Note 2:	The re-synchronisation procedure is not required in this case, as time can be recovered from any source.

## C.3	A mechanism using two individual counters in the HE and a window in the USIM

In this mechanism the sequence numbers are generated as in the mechanism described in C.1. However, the USIM verifies the freshness differently. In addition to the highest sequence number SQNMS it has accepted, it keeps track of which values in a window (SQNMS, SQNMS – w] it has already seen, and this for each mode. If a sequence number is received that is lower than SQNMS but has not been seen before, it is nevertheless accepted.

Using this mechanism, it is not required that a previously visited SN/VLR deletes the unused authentication vectors when a user de-registers from the network and may be more efficient as regards long distance signalling when a user abroad switches a lot between two serving networks.

Note:	When a VLR uses fresh authentication vectors obtained during a previous visit of the user, the USIM can reject them although they have not been use before (because w is finite). Rejection of a sequence number can therefore occur in normal operation, i.e., it is not necessarily caused by (malicious) replay or a database failure.

## C.4	A mechanism using a global counter in the HE and a list in the USIM

In this mechanism the sequence numbers are generated as in the mechanism described in C.2. However, the USIM verifies the freshness differently. Instead of keeping track of the highest sequence number SQNMS only, it keeps track of an ordered list of the b highest values is has received, and this for each mode. If a sequence number is received that is lower than the lowest value in that list, it is rejected. If however, a sequence number is received that is larger than the lowest, but lower than the highest sequence number and was not seen before it is accepted and included in the list.

Using this mechanism, it is not required that a previously visited SN/VLR deletes the unused authentication vectors when a user de-registers from the network and may be more efficient as regards long distance signalling when a user abroad switches a lot between two serving networks.

Note:	When a VLR uses fresh authentication vectors obtained during a previous visit of the user, the USIM can reject them although they have not been use before (because b is finite). Rejection of a sequence number can therefore occur in normal operation, i.e., it is not necessarily caused by (malicious) replay or a database failure.

## C.5 Criteria for replacing the authentication “working assumption”

One of the following conditions should be met before considering replacement of the authentication “working assumption”:

- A serious security flaw is discovered with the SQN protocol.  A “serious flaw” is a weakness that allows a demonstrable attack on the authentication system, leading to theft of service (fraud), compromise of privacy, or any degradation below the security level of current systems.  If the flaw can be easily fixed without changing the fundamental nature of the protocol, there are no grounds for replacement.

- Serious operational difficulties are discovered with the SQN protocol.  These are problems implementing the protocol that may be discovered during early development or testing.  A “serious operation difficulty” is one that prevents the successful and reliable completion of the protocol, or places an unreasonable burden on the network.  If the problem can be solved with a simple alteration that does not change the fundamental nature of the protocol, there are no grounds for replacement.